| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 28th Meeting: Mainz, DE, 20–28 October 2022 | Document: JVET-AB0059-v1 |
| --- | --- |

| Title: | AHG9: Comments on Neural-network Post-filter Characteristics SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Some modifications are proposed to the Neural-network post-filter characteristics SEI Message. Following is proposed:

- It is proposed to signal nnpfc_uri_tag[ i ] and nnprf_uri[ i ] syntax elements outside the conditional if statement which is conditioned on nnpfc_purpose_and_formatting_flag. It is asserted that this makes sure URI information is actually signalled in the SEI message in all cases, when nnpfc_mode_idc is specified to be equal to 2.

- It is proposed to add and modify constraints for the case when nnpfc_mode_idc is equal to 2 to allow an update of a previously signalled neural network post-processing filter.

# Introduction

JVET-AA2006 [1] specifies the syntax and semantics for Neural network post-filter characteristics SEI message and for neural-network post-filter activation SEI message. Neural network post-filter characteristics SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

# Proposal

Some modifications are proposed to the Neural-network post-filter characteristics SEI. Message. Following is proposed:

- It is proposed to signal nnpfc_uri_tag[ i ] and nnprf_uri[ i ] syntax elements outside the conditional if statement which is conditioned on nnpfc_purpose_and_formatting_flag. It is asserted that this makes sure URI information is actually signalled in the SEI message in all cases, when nnpfc_mode_idc is specified to be equal to 2.

- It is proposed to add and modify constraints for the case when nnpfc_mode_idc is equal to 2 to allow an update of a previously signalled neural network post-processing filter.

The proposed changes are shown in red color compared to JVET-AA2006.

8.28 Neural-network post-filter characteristics SEI message

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| nnpfc_purpose_and_formatting_flag | u(1) |
| if( nnpfc_purpose_and_formatting_flag ) { |  |
| nnpfc_purpose | ue(v) |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| /* input and output formatting */ |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_flag | u(1) |
| if( nnpfc_inp_format_flag = = 1 ) |  |
| nnpfc_inp_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_flag | u(1) |
| if( nnpfc_out_format_flag = = 1 ) |  |
| nnpfc_out_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ){ |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| nnpfc_complexity_idc | ue(v) |
| if( nnpfc_complexity_idc > 0 ) |  |
| nnpfc_complexity_element( nnpfc_complexity_idc ) |  |
| } |  |
| if( nnpfc_mode_idc = = 2 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit | u(1) |
| nnpfc_uri_tag[ i ] | st(v) |
| nnpfc_uri[ i ] | st(v) |
| } |  |
| } |  |
| /* filter specified or updated by ISO/IEC 15938-17 bitstream */ |  |
| else if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

    - Neural-network post-filter characteristics SEI message semantics

This SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

….

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

nnpfc_id contains an identifying number that may be used to identify a post-processing filter. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive.

Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

nnpfc_mode_idc equal to 0 specifies that the post-processing filter associated with the nnpfc_id value is determined by external means not specified in this Specification.

nnpfc_mode_idc equal to 1 specifies that the post-processing filter associated with the nnpfc_id value is a neural network represented by the ISO/IEC 15938-17 bitstream contained in this SEI message.

nnpfc_mode_idc equal to 2 specifies that the post-processing filter associated with the nnpfc_id value is a neural network identified by a specified tag Uniform Resource Identifier (URI) (nnpfc_uri_tag[ i ]) and neural network information URI (nnpfc_uri[ i ]).

The value of nnpfc_mode_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_mode_idc greater than 2 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_mode_idc.

nnpfc_purpose_and_formatting_flag equal to 0 specifies that no syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present. nnpfc_purpose_and_formatting_flag equal to 1 specifies that syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present.

When nnpfc_mode_idc is equal to 1 or 2 and the current CLVS does not contain a preceding neural-network post-filter characteristics SEI message, in decoding order, that has the value of nnpfc_id equal to the value of nnpfc_id in this SEI message, nnpfc_purpose_and_formatting_flag shall be equal to 1.

When the current CLVS contains a preceding neural-network post-filter characteristics SEI message, in decoding order, that has the same value of nnpfc_id equal to the value of nnpfc_id in this SEI message, at least one of the following conditions shall apply:

–	This SEI message has nnpfc_mode_idc equal to 1 and nnpfc_purpose_and_formatting_flag equal to 0 in order to provide a neural network update.

–	This SEI message has nnpfc_mode_idc equal to 2 and nnpfc_purpose_and_formatting_flag equal to 0 in order to provide a neural network update.

–	This SEI message has the same content as the preceding neural-network post-filter characteristics SEI message.

…

A base post-processing filter for a cropped decoded output picture picA is the filter that is identified by the first neural-network post-filter characteristics SEI message, in decoding order, that has a particular nnpfc_id value within a CLVS.

If there is another neural-network post-filter characteristics SEI message that has the same nnpfc_id value, has nnpfc_mode_idc equal to 1 or 2, has different content than the neural-network post-filter characteristics SEI message that defines the base post-processing filter, and pertains to the picture picA, the base post-processing filter is updated by decoding the ISO/IEC 15938-17 bitstream in that neural-network post-filter characteristics SEI message to obtain a post-processing filter PostProcessingFilter( ). Otherwise, the post-processing processing filter PostProcessingFilter( ) is assigned to be the same as the base post-processing filter.

# Conclusion

This document proposes modifications to neural network post processing characteristics SEI. It is proposed to adopt the proposals in the next version of “Additional SEI messages for VSEI” draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-AA2006-v2, Additional SEI messages for VSEI (Draft 2), Online, July 2022

Acknowledgement
This work was supported by “Strategic Information and Communications R&D Promotion Programme (SCOPE)” of Ministry of Internal Affairs and Communications, Grant no. JPJ000595.