| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0066-v2 |
| --- | --- |

| Title: | ECM-6 intra performance evaluation with non-CTC dataset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | yasugi.ukinobu@sharp.co.jp tomohiro.ikai@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution provides the results of a validation test of ECM-6.0 using DIV2K and Flikr2K datasets to test the performance of ECM on images of various subjects. Experimental results show -6.91%/-17.03%/-17.30% and -6.58%/-14.81%/-15.20% (All-intra, Y/U/V) on DIV2K and Flickr2K, respectively under all-intra condition. Experimental results confirmed that the performance is on per with that on Class-B CTC contents.

Introduction

ECM software version 6.0 was released on 4 September. The authors conducted an experiment comparing the performance of ECM (6.0) and VTM (11.0ecm6) on DIV2K and Flickr2K datasets to test the performance on images of various subjects. This contribution reports the results.

Test configurations

DIV2K dataset is a well-known dataset for super-resolution task challenges consisting of 900 still images of approximately 2K resolution [1][2]. We used 900 (train and validation) of high-resolution images. Flickr2K is another well-known still image dataset[3], containing 2650 2K images. In this test, we converted those images into 10-bit YUV 4:2:0 format with type-2 chroma sample position using HDRTools[4] v0.23. For details, refer to the configuration file included with this document. The encoding settings were the same as for CTC All intra. An additional option “—ConformanceWindowMode=1” was used to avoid encoder errors caused by fractional resolutions in some images.

Note that images “0014”, “0230”, “0769” and “0800” from DIV2K and “000585”, “001045” and “001666” from Flickr2K were excluded from the calculation of BD-rate . Those are monochrome images and cause errors in calculation of chroma BD-rate values in Excel by exceptional PSNR values in chroma channels.

Experimental results

Table 1 shows ECM-6.0 performance comparing to VTM11.0ecm6 anchor, according to JVET-AB0006[5].

Table 1: Performance of ECM-6.0 over VTM11.0ecm6 under CTC

Table 2 shows experimental results on DIV2K and Flickr2K. Since our PC cluster consists of nodes of various performance, the following runtime data is not reliable.

Table 2: Performance of ECM-6.0 over VTM11.0ecm6 on non-CTC datasets

The luma BD-rate values ranged from -2.82% to -31.42% and from -1.93% to -29.66% for DIV2K and Flickr2K, respectively. This result is roughly equivalent to that obtained with Class-B CTC contents, which have a similar resolution to these datasets.

Figures 1 and 2 show the histograms of the BD-rate values on DIV2K and Flickr2K, respectively. The histograms are similar in shape, and both have peaks around 6% for luma and around 12% for chroma. There is a slight tendency for DIV2K to have more images with higher coding efficiency in UV.

Figure 1: Histograms of BD-rate on DIV2K

Figure 2: Histograms of BD-rate on Flickr2K

Figure 3 to 6 show images with the lower (better) and higher YUV BD-rate for DIV2K and Flickr2K. The YUV BD-rate values were calculated as Y+U/10+V/10. The images with better gain consist mostly of geometric shapes and patterns. On the other hand, images with smaller gain consist primarily of natural landscapes and irregular patterns.

| 0828 (-42.00%) | 0373 (-38.29%) | 0138 (-29.90%) | 0719 (-27.46%) |
| --- | --- | --- | --- |
| 0188 (-27.00%) | 0190 (-25.99%) | 0632 (-25.66%) | 0265 (-24.96%) |
| 0562 (-24.62%) | 0493 (-24.59%) |  |  |

Figure 3: Worst 10 YUV BD-rate images (DIV2K)

| 0710 (-5.28%) | 0866 (-5.21%) | 0243 (-5.16%) | 0163 (-5.12%) |
| --- | --- | --- | --- |
| 0216 (-5.08%) | 0807 (-5.01%) | 0312 (-4.99%) | 0426 (-4.85%) |
| 0579 (-4.85%) | 0895 (-4.41%) |  |  |

Figure 4: Best 10 YUV BD-rate images (DIV2K)

| 001682 (-38.33%) | 002439 (-30.20%) | 000108 (-30.09%) | 001956 (-28.74%) |
| --- | --- | --- | --- |
| 000039 (-27.93%) | 002119 (-26.66%) | 000599 (-26.06%) | 001883 (-25.98%) |
| 001876 (-20.81%) | 001324 (-19.93%) |  |  |

Figure 5: Best 10 YUV BD-rate images (Flickr2K)

| 000515 (-4.21%) | 001225 (-4.05%) | 001332 (-4.01%) | 001932 (-3.86%) |
| --- | --- | --- | --- |
| 000959 (-3.78%) | 002564 (-3.68%) | 001976 (-3.49%) | 000590 (-3.39%) |
| 000762 (-3.11%) | 000453 (-2.76%) |  |  |

Figure 6: Worst 10 YUV BD-rate images (Flickr2K)

Conclusion

This contribution provides the results of a validation test of ECM-6.0 using DIV2K and Flikr2K datasets to test the performance of ECM on images of various subjects. Experimental results show -6.91%/-17.03%/-17.30% and -6.58%/-14.81%/-15.20% (All-intra, Y/U/V) on DIV2K and Flickr2K, respectively under all-intra condition. Experimental results confirmed that the performance is on per with that on Class-B CTC contents.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

Patent rights declaration(s)

Sharp Corporation does not have any current or pending patent rights relating to the technology described in this contribution.

References

[1] Agustsson, Eirikur and Timofte, Radu, “NTIRE 2017 Challenge on Single Image Super-Resolution: Dataset and Study”, The IEEE Conference on Computer Vision and Pattern Recognition (CVPR) Workshops, July, 2017

[2] Timofte, Radu and Agustsson, Eirikur and Van Gool, Luc and Yang, Ming-Hsuan and Zhang, Lei and Lim, Bee and others, “NTIRE 2017 Challenge on Single Image Super-Resolution: Methods and Results”, The IEEE Conference on Computer Vision and Pattern Recognition (CVPR) Workshops, July, 2017

[3] https://github.com/LimBee/NTIRE2017

[4] https://gitlab.com/standards/HDRTools

[5] Vadim Seregin, Jie Chen, Fabrice Le Leannec and Kai Zhang, “JVET AHG report: ECM software development (AHG6)”, JVET-AB0006, October 2022