| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0070-v1 |
| --- | --- |

| Title: | AHG9: On inclusion of post-filter hint SEI message into VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung Hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to include post-filter hint SEI message in the VSEI specification. It is asserted that application of filters to decoded pictures is desirable and useful for some use-case scenarios. Furthermore, one of the use-case scenarios in which the SEI message can be used is to signal the filters for resampling decoded pictures, post decoding and outputting process but prior to displaying, when reference picture resampling (RPR) feature is enabled so that the resolution of the pictures to be displayed can be the same.

It is also asserted that currently the post-filter hint SEI message has parsing dependency to the underlying codec (i.e., the need to get the chroma format information). This contribution suggests solution for it by signalling whether filters from chroma component is present or not.

Finally, it is also suggested that for the post-filter hint SEI message, the persistence of the SEI message is modified so that it may persist for more than one picture.

# Introduction

To improve video quality of decoded pictures, in the previous codecs such as H.264/AVC and H.265/HEVC, it is possible to apply filters after the decoding process to the reconstructed pictures. The filters for such process are delivered / signalled in post-filter hint SEI message. Such process is asserted to be a generic process that can also be applicable to VVC but currently not available since the SEI message is not included in the VVC specification nor in the VSEI specification.

Furthermore, as reference picture resampling (RPR) feature is included in VVC, when this feature is enabled, the outputted decoded pictures from the VVC decoder could have different resolutions. Prior to displaying, the outputted pictures may be resampled to make them having the same resolution. It is asserted that the resampling of the outputted pictures can be done outside of the decoding loop and the filters used for such a process do not have to the same as the filters used for the RPR during decoding process. To signal such filters for resampling of the decoded pictures, the existing post-filter hint SEI message could be used as it provides the generic way to signal filter coefficients for such purpose. For convenience, the syntax and semantics of the SEI message are shown below.

| post_filter_hint( payloadSize ) { | Descriptor |
| --- | --- |
| filter_hint_size_y | ue(v) |
| filter_hint_size_x | ue(v) |
| filter_hint_type | u(2) |
| for( cIdx = 0; cIdx < ( chroma_format_idc = = 0 ? 1 : 3 ); cIdx++ ) |  |
| for( cy = 0; cy < filter_hint_size_y; cy++ ) |  |
| for( cx = 0; cx < filter_hint_size_x; cx++ ) |  |
| filter_hint_value[ cIdx ][ cy ][ cx ] | se(v) |
| } |  |

This SEI message provides the coefficients of a post-filter or correlation information for the design of a post-filter for potential use in post-processing of the current picture after it is decoded and output to obtain improved displayed quality.

filter_hint_size_y specifies the vertical size of the filter coefficient or correlation array. The value of filter_hint_size_y shall be in the range of 1 to 15, inclusive.

filter_hint_size_x specifies the horizontal size of the filter coefficient or correlation array. The value of filter_hint_size_x shall be in the range of 1 to 15, inclusive.

filter_hint_type identifies the type of the transmitted filter hints as specified in Table D.6. The value of filter_hint_type shall be in the range of 0 to 2, inclusive. The value of filter_hint_type equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore post-filter hint SEI messages having filter_hint_type equal to 3.

Table D-6 – filter_hint_type values

| Value | Description |
| --- | --- |
| 0 | Coefficients of a 2D-FIR filter |
| 1 | Coefficients of two 1D-FIR filters |
| 2 | Cross-correlation matrix |

filter_hint_value[ cIdx ][ cy ][ cx ] specifies a filter coefficient or an element of a cross-correlation matrix between the original and the decoded signal with 16-bit precision. The value of filter_hint_value[ cIdx ][ cy ][ cx ] shall be in the range of −231 + 1 to 231 − 1, inclusive. cIdx specifies the related colour component, cy represents a counter in vertical direction and cx represents a counter in horizontal direction. Depending on the value of filter_hint_type, the following applies:

- If filter_hint_type is equal to 0, the coefficients of a 2-dimensional finite impulse response (FIR) filter with the size of filter_hint_size_y * filter_hint_size_x are transmitted.

- Otherwise, if filter_hint_type is equal to 1, the filter coefficients of two 1-dimensional FIR filters are transmitted. In this case, filter_hint_size_y shall be equal to 2. The index cy equal to 0 specifies the filter coefficients of the horizontal filter and cy equal to 1 specifies the filter coefficients of the vertical filter. In the filtering process, the horizontal filter is applied first and the result is filtered by the vertical filter.

- Otherwise (filter_hint_type is equal to 2), the transmitted hints specify a cross-correlation matrix between the original signal s and the decoded signal s′.

NOTE 1 – The normalized cross-correlation matrix for a related colour component identified by cIdx with the size of filter_hint_size_y * filter_hint_size_x is defined as follows:

(D-16)

where s denotes array of samples of the colour component cIdx of the original picture, s′ denotes corresponding array of the decoded picture, h denotes the vertical height of the related colour component, w denotes the horizontal width of the related colour component, bitDepth denotes the bit depth of the colour component, OffsetY is equal to ( filter_hint_size_y >> 1 ), OffsetX is equal to ( filter_hint_size_x >> 1 ), 0 <= cy < filter_hint_size_y and 0 <= cx < filter_hint_size_x.

NOTE 2 – A decoder can derive a Wiener post-filter from the cross-correlation matrix of original and decoded signal and the auto-correlation matrix of the decoded signal.

Considering its generic usage and also the possibility for carrying filters for resampling of decoded pictures, it is our assertion that it would be beneficial to include post-filter hint SEI message in VSEI specification.

We also further studied the post-filter hint SEI message and asserted that some modifications may be needed as described in the discussion section below.

# Discussion

## Parsing dependency

For parsing of syntax element filter_hint_value[ cIdx ][ cy ][ cx ], the upper range value of the index cIdx depends on the value of chroma_format_idc. This chroma_format_idc is signalled in parameter set in the bitstream which means that to parse the SEI message, there is dependency to the parameter set and this may not be desirable.

## Persistency of the SEI message

The persistency of post-filter SEI message as defined in HEVC is only for the associated picture that is in the same layer as the SEI message and contained in the access unit. It is asserted that in some cases, the same filter may be applicable to subsequent pictures in the same layer. In such situation, currently the same SEI message needs to be sent in each AU of the pictures that the filter applies to. It is desirable if for such situation we can simply send the SEI at the first AU and it applies to subsequent pictures as well.

# Proposal

We propose the following:

- Include post-filter hint SEI message in VSEI specification.

- Remove the parsing dependency related to the chroma format information by explicitly signal whether or not filter coefficients for chroma components are present.

- Modify the persistent of post-filter hint SEI message to allow its scope to cover one or more pictures by using the cancel flag and persistent flag.

# Text implementation

The text implementation of the above proposed items is shown below. New texts for proposal item 2 are marked by yellow highlight and for proposal item 3 are marked by cyan highlight.

...

### Post-filter hint SEI message

| post_filter_hint( payloadSize ) { | Descriptor |
| --- | --- |
| filter_hint_cancel_flag | u(1) |
| if( !filter_hint_cancel_flag ) { |  |
| filter_hint_persistence_flag | u(1) |
| filter_hint_size_y | ue(v) |
| filter_hint_size_x | ue(v) |
| filter_hint_type | u(2) |
| filter_hint_chroma_coeff_present_flag | u(1) |
| for( cIdx = 0; cIdx < ( chroma_format_idc filter_hint_chroma_coeff_present_flag = = 0 ? 1 : 3 ); cIdx++ ) |  |
| for( cy = 0; cy < filter_hint_size_y; cy++ ) |  |
| for( cx = 0; cx < filter_hint_size_x; cx++ ) |  |
| filter_hint_value[ cIdx ][ cy ][ cx ] | se(v) |
| } |  |
| } |  |

This SEI message provides the coefficients of a post-filter or correlation information for the design of a post-filter for potential use in post-processing of the current picture after it is decoded and output to obtain improved displayed quality.

filter_hint_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous post-filter hint SEI message in output order that applies to the current layer. filter_hint_cancel_flag equal to 0 indicates that post-filter hint information follows.

filter_hint_persistence_flag specifies the persistence of the post-filter hint SEI message for the current layer. filter_hint_persistence_flag equal to 0 specifies that the post-filter hint applies to the current decoded picture only. filter_hint_persistence_flag equal to 1 specifies that the post-filter hint SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

- A new CLVS of the current layer begins.

- The bitstream ends.

- A picture in the current layer in an AU associated with a post-filter hint SEI message is output that follows the current picture in output order.

filter_hint_size_y specifies the vertical size of the filter coefficient or correlation array. The value of filter_hint_size_y shall be in the range of 1 to 15, inclusive.

filter_hint_size_x specifies the horizontal size of the filter coefficient or correlation array. The value of filter_hint_size_x shall be in the range of 1 to 15, inclusive.

filter_hint_type identifies the type of the transmitted filter hints as specified in Table D.6. The value of filter_hint_type shall be in the range of 0 to 2, inclusive. The value of filter_hint_type equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore post-filter hint SEI messages having filter_hint_type equal to 3.

Table D-6 – filter_hint_type values

| Value | Description |
| --- | --- |
| 0 | Coefficients of a 2D-FIR filter |
| 1 | Coefficients of two 1D-FIR filters |
| 2 | Cross-correlation matrix |

filter_hint_chroma_coeff_present_flag equal 1 specifies that filter coefficients for chroma are present. filter_hint_chroma_coeff_present_flag equal 0 specifies that filter coefficients for chroma are not present.

filter_hint_value[ cIdx ][ cy ][ cx ] specifies a filter coefficient or an element of a cross-correlation matrix between the original and the decoded signal with 16-bit precision. The value of filter_hint_value[ cIdx ][ cy ][ cx ] shall be in the range of −231 + 1 to 231 − 1, inclusive. cIdx specifies the related colour component, cy represents a counter in vertical direction and cx represents a counter in horizontal direction. Depending on the value of filter_hint_type, the following applies:

- If filter_hint_type is equal to 0, the coefficients of a 2-dimensional finite impulse response (FIR) filter with the size of filter_hint_size_y * filter_hint_size_x are transmitted.

- Otherwise, if filter_hint_type is equal to 1, the filter coefficients of two 1-dimensional FIR filters are transmitted. In this case, filter_hint_size_y shall be equal to 2. The index cy equal to 0 specifies the filter coefficients of the horizontal filter and cy equal to 1 specifies the filter coefficients of the vertical filter. In the filtering process, the horizontal filter is applied first and the result is filtered by the vertical filter.

- Otherwise (filter_hint_type is equal to 2), the transmitted hints specify a cross-correlation matrix between the original signal s and the decoded signal s′.

NOTE 1 – The normalized cross-correlation matrix for a related colour component identified by cIdx with the size of filter_hint_size_y * filter_hint_size_x is defined as follows:

(D-16)

where s denotes array of samples of the colour component cIdx of the original picture, s′ denotes corresponding array of the decoded picture, h denotes the vertical height of the related colour component, w denotes the horizontal width of the related colour component, bitDepth denotes the bit depth of the colour component, OffsetY is equal to ( filter_hint_size_y >> 1 ), OffsetX is equal to ( filter_hint_size_x >> 1 ), 0 <= cy < filter_hint_size_y and 0 <= cx < filter_hint_size_x.

NOTE 2 – A decoder can derive a Wiener post-filter from the cross-correlation matrix of original and decoded signal and the auto-correlation matrix of the decoded signal.

...

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).