| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0078-v1 |
| --- | --- |

| Title: | EE2-2.1: AmvpMerge for low-delay |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Jaehyun Lim Seung-Hwan Kim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This document reports for EE2-2.1 on AmvpMerge for the low-delay picture. In the EE2-2.1, the coding performance of AmvpMerge for the low-delay picture is investigated with bypassing bilateral matching merge candidate reordering process and also with bypassing template matching MV refinement for AmvpMerge candidate.

- Test A (EE2-2.1a): Enabling AmvpMerge for low-delay picture with bypassing bilateral matching merge candidate reordering process

- Test B (EE2-2.1b): Test A without template matching based MV refinement

Simulation result based on ECM-6.0 with AmvpMerge for low-delay picture are reported as below:

- Test A : (-0.26%, -0.64%, -0.37% / EncT 106%, DecT 100% ) for LDB

- Test B : (-0.16%, -0.25%, -0.33% / EncT 104%, DecT 100% ) for LDB

Both tests show no coding impact (0.00%) on RA configuration.

Introduction

The AmvpMerge is designed to apply bilateral matching based candidate reordering. Therefore, AmvpMerge has not been applied for the low-delay picture due to the reference pictures being all forward pictures. In JVET-AA0069[1], it was proposed that AmvpMerge is enabled with bypassing bilateral matching based merge candidate reordering and with bypassing template matching based MV refinement of AmvpMerge.

Proposed methods

This contribution proposes to allow AmvpMerge with bypassing bilateral matching based and with bypassing template matching based candidate reordering and MV refinement for low-delay picture. Two tests are simulated with enabling AmvpMerge for low-delay picture. AmvpMerge is enabled with bypassing bilateral matching merge candidate reordering process for test A and Test A with bypassing template matching based MV refinement are evaluated for Test B.

Experimental results

The proposed method was implemented on top of ECM-6.0 [2]. Experiments were conducted following the common test conditions [3] for enhanced compression tool.

Simulation result for Test A and Test B are tabulated in Table 1 and Table 2.

Table 1. Simulation result of Test A (EE2-2.1a)

|  | Random Access |  |  |  |  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 CTC |  |  |  |  | Over ECM-6.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.00% | 100% | 100% | -0.20% | -0.78% | -0.54% | 105% | 99% |
| Class C | 0.00% | 0.00% | 0.01% | 99% | 100% | -0.25% | -0.34% | -0.18% | 107% | 100% |
| Class E |  |  |  |  |  | -0.37% | -0.81% | -0.33% | 105% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.26% | -0.64% | -0.37% | 106% | 100% |
| Class D | 0.00% | 0.01% | 0.02% | 98% | 98% | -0.23% | -0.37% | -0.19% | 107% | 100% |
| Class F | 0.00% | 0.01% | 0.00% | 100% | 99% | -0.08% | -0.36% | -0.65% | 103% | 101% |

Table 2. Simulation result of Test B (EE2-2.1b)

|  | Random Access |  |  |  |  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 CTC |  |  |  |  | Over ECM-6.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.01% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | -0.01% | 0.00% | 101% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.01% | 100% | 100% | -0.13% | -0.34% | -0.52% | 103% | 99% |
| Class C | 0.00% | 0.01% | -0.01% | 101% | 100% | -0.24% | -0.27% | -0.14% | 105% | 99% |
| Class E |  |  |  |  |  | -0.12% | 0.01% | -0.29% | 104% | 99% |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 100% | -0.16% | -0.23% | -0.33% | 104% | 99% |
| Class D | 0.00% | 0.01% | 0.01% | 100% | 100% | -0.20% | -0.12% | 0.75% | 105% | 101% |
| Class F | 0.00% | 0.00% | -0.01% | 101% | 100% | -0.15% | -0.56% | -0.79% | 103% | 101% |

Conclusion

The contribution reports the results of EE2-1.1 on AmvpMerge for the low-delay picture. It is asserted that the proposed method provides promising gains for low-delay without additional hardware implementation cost over AmvpMerge in ECM-6.0. It is suggest to adopt the proposed method to the next version of ECM.

References

[1] H. Jang, J.Nam, N.Park, J.Lim, S.Kim (LGE), “Non-EE2: AmvpMerge for low delay”, JVET-AA0069, July, 2022.

[2] ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0.

[3]	M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).