| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0081-v1 |
| --- | --- |

| Title: | AHG10: Increased length of filters used for upscaling reconstructed pictures in VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Yu, L. Litwic | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

RPR enable usage of a reference pictures with different resolution than the current picture for inter prediction. In VTM 8-tap luma and 4-tap chroma MC filters are used for upscaling luma respectively chroma when RPR is used but also for upscaling the reconstructed picture to full resolution (enabled by UpscaledOutput=2).

This contribution proposes to increase the filter length from 8-tap to 12-tap for luma and 4-tap to 6-tap for chroma for the filters used for upscaling the reconstructed picture to full resolution.

Compared to VTM-18.0 for RPR CTC LDB (with switch of resolution every 0.5s):

2x:    Y/U/V:   -0,80%/-2,18%/-1,89%

1.5x: Y/U/V:   -0,97%/-1,51%/-1,32%

A visual inspection revealed similar visual quality.

It is suggested to include the filters to the next release of VTM and use them for upscaling the reconstructed picture to full resolution.

# Introduction

Its important that VVC reference software works as good as possible to ensure a good benchmark for implementors. Since there is freedom in the use of filters for upscaling the reconstructed picture to full resolution it was investigated what could be gained by increasing the filter length to the lengths of the ECM MC filters.

# Proposal

A new 12-tap filter set with enhanced magnitude and phase response compared to the 8-tap luma filter currently used is suggested together with the 6-tap chroma MC filter that was proposed in JVET-Z0068 [3] to be used for upscaling the reconstructed picture to full resolution (enabled by UpscaledOutput=2).

The reason for not using the 12-tap ECM luma MC filter set is that it is relatively aggressive, and it make sense to use a filter set which is a bit more conservative as a last filter before display. The reason for not using the 6-tap ECM chroma filter set is that it performs slightly worse than the proposed 6-tap filters.

The proposed 12-tap luma filter coefficients for upscaling of the reconstructed picture are as follows for phase 0 to 15/16:

{ 0, 0, 0, 0, 0, 256, 0, 0, 0, 0, 0, 0, },

{ 1, -1, 0, 3, -12, 253, 16, -6, 2, 0, 0, 0, },

{ 0, 0, -3, 9, -24, 250, 32, -11, 4, -1, 0, 0, },

{ 0, 0, -4, 12, -32, 241, 52, -18, 8, -4, 2, -1, },

{ 0, 1, -6, 15, -38, 228, 75, -28, 14, -7, 3, -1, },

{ 0, 1, -7, 18, -43, 214, 96, -33, 16, -8, 3, -1, },

{ 1, 0, -6, 17, -44, 196, 119, -40, 20, -10, 4, -1, },

{ 0, 2, -9, 21, -47, 180, 139, -43, 20, -10, 4, -1, },

{ -1, 3, -9, 21, -46, 160, 160, -46, 21, -9, 3, -1, },

{ -1, 4, -10, 20, -43, 139, 180, -47, 21, -9, 2, 0, },

{ -1, 4, -10, 20, -40, 119, 196, -44, 17, -6, 0, 1, },

{ -1, 3, -8, 16, -33, 96, 214, -43, 18, -7, 1, 0, },

{ -1, 3, -7, 14, -28, 75, 228, -38, 15, -6, 1, 0, },

{ -1, 2, -4, 8, -18, 52, 241, -32, 12, -4, 0, 0, },

{ 0, 0, -1, 4, -11, 32, 250, -24, 9, -3, 0, 0, },

{ 0, 0, 0, 2, -6, 16, 253, -12, 3, 0, -1, 1, },

The proposed 6-tap chroma filter coefficients for upscaling of the reconstructed picture are as follows for phase 0 to 15/16:

{0, 0, 256, 0, 0, 0, },

{ 1, -6, 256, 6, -1, 0, },

{ 2, -11, 254, 14, -4, 1, },

{ 4, -18, 252, 23, -6, 1, },

{ 6, -24, 249, 32, -9, 2, },

{ 6, -26, 244, 41, -12, 3, },

{ 7, -30, 239, 53, -18, 5, },

{ 8, -34, 235, 61, -19, 5, },

{ 10, -38, 228, 72, -22, 6, },

{ 10, -39, 220, 84, -26, 7, },

{ 10, -40, 213, 94, -29, 8, },

{ 11, -42, 205, 105, -32, 9, },

{ 11, -42, 196, 116, -35, 10, },

{ 11, -42, 186, 128, -37, 10, },

{ 11, -42, 177, 138, -38, 10, },

{ 11, -41, 167, 148, -40, 11, },

{ 11, -41, 158, 158, -41, 11, },

{ 11, -40, 148, 167, -41, 11, },

{ 10, -38, 138, 177, -42, 11, },

{ 10, -37, 128, 186, -42, 11, },

{ 10, -35, 116, 196, -42, 11, },

{ 9, -32, 105, 205, -42, 11, },

{ 8, -29, 94, 213, -40, 10, },

{ 7, -26, 84, 220, -39, 10, },

{ 6, -22, 72, 228, -38, 10, },

{ 5, -19, 61, 235, -34, 8, },

{ 5, -18, 53, 239, -30, 7, },

{ 3, -12, 41, 244, -26, 6, },

{ 2, -9, 32, 249, -24, 6, },

{ 1, -6, 23, 252, -18, 4, },

{ 1, -4, 14, 254, -11, 2, },

{ 0, -1, 6, 256, -6, 1, }

In Figure 1 and Figure 3 the proposed 12-tap luma and 6-tap chroma upscaling filters are visualized with respect to magnitude response and phase delay and in Figure 2 and Figure 4 the current 8-tap luma and 4-tap chroma upscaling filters are visualized. It can be noted that the proposed filters have closer to ideal phase delay and ideal magnitude response than the current filters.

Figure 1: Phase delay responses (a) and magnitude responses (b) of proposed 12-tap luma upscaling filters

Figure 2: Phase delay responses (a) and magnitude responses (b) of current 8-tap luma upscaling filters

Figure 3: Phase delay responses (a) and magnitude responses (b) of proposed 6-tap chroma upscaling filters

Figure 4: Phase delay responses (a) and magnitude responses (b) of current 4-tap chroma upscaling filters

# Results

The proposal is compared against the VTM-18.0 [1] for RPR common test conditions with switch of resolution every half second [2]. In section 3.1 and 3.2 a substantial BDR improvement for PSNR (upscaled to full resolution) can be noted for both 2x and 1.5x when increasing the filter lengths to 12 for luma and 6 for chroma. The encoding run times are asserted to be inaccurate since the encoding of the anchor and the tests were run on machines with different capabilities. However, the decoding run times are asserted to be enough accurate since the decoding of the anchor and the test were run on same type of machines.

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-18.0 RPR 2x |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,79% | -1,81% | -1,86% | 100% | 97% |
| Class C | -0,44% | -2,19% | -2,13% | 100% | 100% |
| Class E | -1,30% | -2,78% | -1,61% | 100% | 105% |
| Overall | -0,80% | -2,18% | -1,89% | 100% | 100% |
| Class D | -0,36% | -1,61% | -2,14% | 100% | 102% |
| Class F | -0,45% | -3,31% | -3,75% | 100% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-18.0 RPR 1.5x |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,58% | -1,35% | -1,29% | 100% | 101% |
| Class C | -0,68% | -1,87% | -1,86% | 100% | 102% |
| Class E | -2,02% | -1,30% | -0,64% | 100% | 99% |
| Overall | -0,97% | -1,51% | -1,32% | 100% | 101% |
| Class D | -0,58% | -1,44% | -1,91% | 100% | 98% |
| Class F | -0,88% | -4,20% | -4,51% | 100% | 102% |

# Conclusions

It is asserted that an increase of the filter lengths to 12 respectively 6 taps for upscaling of reconstructed pictures in RPR scenario can give a nice BDR improvement. A visual inspection revealed similar visual quality. It is suggested to upgrade the upscaling filters in next version of VTM.

# References

[1] VTM-18.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-18.0

[2] J. Luo, V. Seregin, “JVET functionality confirmation test conditions for reference picture resampling”, JVET-Q2015, Jan. 2020.

[3] K. Andersson, R. Yu, “EE2-2.6 related: Longer chroma interpolation filters”, JVET-Z0068, April. 2022

# Patent rights declaration(s)

Ericsson AB may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).