| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0082-v1 |
| --- | --- |

| Title: | AHG12: Fixes for RPR in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Yu | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

RPR can be used to make use of a reference pictures with different resolution than the current picture. It was discovered that RPR CTC could not be run without encoder and decoder crash. It was also noticed that the 12-tap ECM luma filter was not used for upscaling luma before outputting pictures when upscaling to full resolution (UpscaledOutput=2) and not either for upscaling luma pictures for ME on encoder side.

The encoder/decoder issue was fixed by increasing the code size from 9 to 10 bit for encoding lambda in ARMC.

The fix of the luma filter length from 8-tap to 12-tap gives compared to ECM-6.0 including the first fix for RPR CTC LDB (with switch of resolution every 0.5s):

2x:    Y/U/V:   -1,70%/0,04%/-0,12%

1.5x: Y/U/V:   -2,03%/-0,16%/-0,05%

It is suggested to consider the two fixes for next version of ECM.

# Introduction

When ECM-6.0 was tested for RPR CTC it was discovered that it did not work (encoder crash and decoder crash in several cases). When doing some checks with higher QP it was noticed that the encoding of lambda for ARMC was tried for a value greater than supported by the length of the codeword. It was also found that the 8-tap luma filter was used for upscaling rather than the 12-tap luma MC filter in ECM. For chroma upscaling the 6-tap chroma MC filter in ECM is used which is correct.

# Proposal

The first issue went away by encoding of lambda for ARMC with an increased code length from 9 to 10.

The second issue is solved by simply referring to the 12-tap luma MC filter instead of the 8-tap filter.

# Results

Comparison against the ECM-6.0 [1] with first fix for RPR common test conditions with switch of resolution every half second [2]. A substantial BDR improvement for PSNR2 (upscaled to full resolution) can be noted for both 2x and 1.5x when fixing the upscaling filter length to 12-tap. The encoding run times are asserted to be inaccurate since the encoding of the anchor and the tests were run on machines with different capabilities. However, the decoding run times are asserted to be enough accurate since the decoding of the anchor and the test were run on same type of machines

## Fix to use the 12-tap luma MC filter instead of 8-tap for upscaling

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 RPR2x with first fix |  |  |  |  |
| G | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,62% | 0,26% | -0,06% | 109% | 99% |
| Class C | -0,96% | -0,13% | -0,26% | 105% | 102% |
| Class E | -2,81% | -0,12% | -0,01% | 109% | 99% |
| Overall | -1,70% | 0,04% | -0,12% | 108% | 100% |
| Class D | -0,68% | -0,79% | -0,16% | 106% | 100% |
| Class F | -1,10% | -0,38% | -0,40% | 107% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM6 RPR 1.5x with first fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,36% | -0,10% | 0,08% | 109% | 98% |
| Class C | -1,56% | -0,17% | -0,09% | 114% | 100% |
| Class E | -3,78% | -0,24% | -0,20% | 116% | 99% |
| Overall | -2,03% | -0,16% | -0,05% | 112% | 99% |
| Class D | -1,37% | -0,48% | 0,15% | 112% | 100% |
| Class F | -1,70% | 0,24% | -0,13% | 115% | 100% |

# Conclusions

It is asserted that an alignment of the filter length to 12-tap also for upscaling of reconstructed pictures and for motion estimation in RPR scenario also gives a nice BDR improvement. It is therefore suggested to fix that in the next version of ECM.

The solution to the problem with ARMC is also suggested for next version of ECM to enable RPR functionality.

# References

[1] ECM-6.0 reference software,  https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

[2] J. Luo, V. Seregin, “JVET functionality confirmation test conditions for reference picture resampling”, JVET-Q2015, Jan. 2020.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).