| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0084-v3 |
| --- | --- |

| Title: | EE1-2.3：A CNN-based Super Resolution Method with GOP Level Adaptive Resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang, Liqiang Wang, Xiaozhong Xu, Shan Liu (Tencent) Junghak Nam, Sunmi Yoo, Jaehyun Lim, Seung Hwan Kim (LGE) | Tel: Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com junghak.nam@lge.com |
| Source: | Tencent, LG Electronics Inc. |  |  |

_____________________________

- Abstract

This proposal reports the EE1-2.3 test results, which is a combination of JVET-AA0084 and JVET-Y0068 for more balanced compression performance. The proposed method can adaptively select a scale factor from ×1.0 and ×2.0 at the GOP level and designed CNN-based super-resolution will be used for the latter case. Compared with VTM-11.0-NNVC-v2, the experimental results show {-3.63%, -2.40%, -2.07%} and {-3.47%, 0.61%, -0.15%} BD-rate savings for {Y, Cb, Cr} under RA and AI configurations, respectively.

Introduction

The JVET-AA0084 [1] obtains significant coding gains by proposing a convolutional neural network (NN)-based super resolution method combined with existing RPR functionality. In addition, the JVET-Y0068 [2] has demonstrated that adaptive RPR selection at GOP level may provide a better coding performance. This contribution tests the combination of JVET-AA0084 and JVET-Y0068.

Proposed method

GOP level encoding resolution decision

In this proposal, the resampling factors ×2.0 (half size) and ×1.0 (original size) are selected at GOP level and the scale factor is applied on all pictures in the GOP. To determine whether to resample the picture by ×2.0, the first picture is downscaled to quarter resolution and then resampled to the original resolution. The PSNR is calculated between the original picture and the down-up scaled picture. The scale factor is selected according to a threshold which is adaptively varied based on the initial QP. The pre-defined threshold is the same as that in JVET-Y0068.

CNN-based super-resolution

The designed network structure for luma is shown in Fig. 1. Along with the reconstruction Ylrrec, the prediction Ylrpred, slice QP and base QP are also fed into the proposed network. Several ordinary convolutions and N residual blocks are used to obtain effective features from the input. Then, a pixel shuffle layer is utilized to output the high-resolution residuals. Finally, the rpr reconstruction Yhrrpr upsampled by RPR mechanism is adopted to generate the high-resolution reconstruction Yhrrec. Besides, please note that the base QP is removed for the I slices.

Fig. 1: The super resolution network architecture for luma

The designed network structure for chroma shown in Fig. 2 is similar to the network structure for luma, and the main difference is the input part. Since the cross-component correlation is beneficial to the super resolution of chroma components, the luma reconstruction is fed into the network. Besides, the slice type is also taken as one input for better generalization.

Fig. 2: The super resolution network architecture for chroma

Network information in training stage

The network information in the training stage is shown in Table 1.

Table 1 Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla V100 32GB |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1000 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time: | ~240h |
|  | Training data information: | TVD_4K, BVI-DVC_4K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
|  | Number of iterations | 1000 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Other information: |  |

Network information in inference stage

The network information in the inference stage is shown in Table 2.

Table 2 Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | luma model for I slice: 1.49 M luma model for B slice: 1.49M chroma model: 1.50M |
|  | Total Number of Parameters (All Models) | 4.48M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 3 models in total: 17.14MB |
|  | Multiplay Accumulate (MAC)/pixel | 469K |
| Optional |  |  |
|  | Total Conv. Layers | 76 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | whole frame |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Results

In the experiments, VTM-11.0 + new MCTF [3] [4] is used as the anchor and the QP values are set as {22, 27, 32, 37, 42}, according to the common test conditions [5]. The proposed NN-based super resolution with libtorch are evaluated. The test results for RA and AI configurations are shown as follows.

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
| sequences | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.20% | -5.69% | -4.83% | 131% | 3104% |
| Class A2 | -1.06% | 0.89% | 0.68% | 127% | 303% |
| Class B | -0.40% | 0.00% | -0.30% | 128% | 279% |
| Class C | 0.00% | 0.00% | 0.00% | 115% | 108% |
| Class E |  |  |  |  |  |
| Average on A1 and A2 | -3.63% | -2.40% | -2.07% | 129% | 1704% |
| Overall | -1.58% | -0.96% | -0.93% | 125% | 357% |
| Class D | 0.00% | 0.00% | 0.00% | 114% | 109% |
| Class F | 0.00% | 0.00% | 0.00% | 144% | 113% |

Table 4. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
| sequences | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -4.23% | -1.52% | -1.73% | 189% | 997% |
| Class A2 | -2.71% | 2.74% | 1.43% | 136% | 384% |
| Class B | -0.75% | 0.03% | -0.30% | 129% | 292% |
| Class C | 0.00% | 0.00% | 0.00% | 112% | 105% |
| Class E | 0.00% | 0.00% | 0.00% | 129% | 103% |
| Average on A1 and A2 | -3.47% | 0.61% | -0.15% | 163% | 691% |
| Overall | -1.37% | 0.21% | -0.13% | 134% | 251% |
| Class D | 0.00% | 0.00% | 0.00% | 108% | 112% |
| Class F | 0.00% | 0.00% | 0.00% | 110% | 104% |

Conclusions

In this contribution, a combination of CNN-based super-resolution and adaptive GOP level encoding resolution decision is tested. The adaptive encoding resolution decision enables the encoder to adaptively select the encoding resolution between an original size and a half size. This proposed method shows a promising coding gains.

Reference

- R. Chang, L. Wang, X. Xu, S. Liu, “AHG11: Neural Network based Super Resolution for Video Coding Using Multiple Side Information”, JVET-AA0084, Jul. 2022.

- J. Nam, S. Yoo, J. Lim, S.  Kim, E1-2.1-related: RPR encoder with multiple scale factors, JVET-Y0068, Jan. 2022.

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- P. Wennersten, C. Hollmann, J. Ström, “GOP-based temporal filter improvements”, JVET-V0056, Apr. 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-AA2016, Jul. 2022.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).