| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0086-v1 |
| --- | --- |

| Title: | Optimized VVC encoder with multilayer coding capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports an optimized VVC encoder with multilayer coding capability, which is developed based on VVenC-1.4.0 released from HHI in GitHub. The VVenC-multilayer software is developed to accelerate the investigation of the optimum rate/quality settings for base and enhancement layers in spatial scalability scenario. To balance quality and encoder runtime, 5 presets (faster, fast, medium, slow, and slower) are defined for encoder configurations as is the case with VVenC-1.4.0.

The simulation is conducted using 4K test sequences distributed by ITE (Institute of Image Information and Television Engineers). In the simulation, 2K sequence for the input of base layer is generated by SHVC down sampling filter. The BD-rate of {Y, Cb, Cr} components calculated from PSNR values of the enhancement layer and total bitrate, and the encoder speedup of single thread coding over VTM-16.0 are reported as follows:

Faster		BD-rate: {40.3%, 32.8%, 43.1%}, Encoder speedup: 226.7x

Fast		BD-rate: {29.0%, 25.4%, 29.5%}, Encoder speedup: 114.2x

Medium	BD-rate: {10.5%, 10.4%, 11.4%}, Encoder speedup: 35.1x

Slow		BD-rate: {4.9%, 4.0%, 3.9%}, Encoder speedup: 11.3x

Slower		BD-rate: {0.1%, 0.3%, -0.1%}, Encoder speedup: 1.2x

The output multilayer bitstream conforms VVC specifications and it is confirmed that the bitstream can be decoded by VTM-16.0. The public release of the developed software and the pull request to the original repository of VVenC are currently being considered.

Introduction

Multilayer coding capability is supported in VVC version 1, which enables for broadcast and streaming application to enable multi-resolution services for different devices. In JVET activity, verification tests for SDR, HDR, and 360° video have conducted, and the test for multilayer coding is currently under investigation.

In the JVET-Y meeting, JVET-Y0048 reported layer bitrate allocation analysis for multilayer coding scenario. During the discussion, it was identified that the investigation of optimum rate/quality settings for base and enhancement layers was important. However, the investigation may require considerable computational resources since VTM multilayer encoding for HD and 4K takes at most 400~ hours, which might be a serious burden for the investigation.

Alternative implementation of VVC encoder other than VTM is Fraunhofer Versatile Video Encoder (VVenC) developed and published by Fraunhofer HHI, which is an optimized VVC software encoder providing speedups between 18x to 1500x over VTM. This however does only support single layer coding and does not support multilayer coding capability for the time being.

In this contribution, development of the optimized VVC encoder with multilayer coding capability and its performance are reported. The software named VVenC-multilayer was developed based on VVenC-1.4.0 and implementation relating to multilayer coding support was ported from VTM-16.0. As is the case with VVenC, five presets (slower, slow, medium, fast, and faster) are defined for the encoder configurations to balance quality and encoding runtime. The bitstream output from the developed software conforms Multilayer Main 10 profile of VVC specifications and it is confirmed that the bitstream can be decoded in VTM-16.0.

Test setting

Test sequences

4K test sequences distributed by ITE (Institute of Image Information and Television Engineers, Japanese academic society) are used for the test. The detailed information on the test sequence can be obtained from https://www.ite.or.jp/content/test-materials/uhdtv_a/. 2K sequences generated by 4K sequences using SHVC down-sampling filter are coded as base layer and original 4K sequences are coded as enhancement layer.

The version of TAppDownConvert is SHM-12.4 and the command line arguments for the down conversion are as follows:

TAppDownConvertStatic 3840 2160 input.yuv 1920 1080 output.yuv 10 10 0 0 0 300 -phase 0 0 0 0

where the number of frames to be encoded is set equal to 300 in the experiments. The thumbnails of the test sequences used for the test is listed in Table 1.

Table 1. Thumbnails of the test sequences

| a03_TrainC | a04_Expressway |
| --- | --- |
| a06_Festival | a08_JapaneseMaple |
| a09_Maiko | a10_Umbrella |
| a11_LayeredKimono | a12_Railcar |

Anchor

To evaluate the coding efficiency and encoding runtime of VVenC-multilayer, the VTM anchor is generated using VTM-16.0. For QP settings for base and enhancement layers, {(22, 22), (27, 27), (32, 32), (37, 37)} and {(22, 24), (27, 29), (32, 34), (37, 39)} are used.

An example of command line arguments are as follows:

EncoderAppStatic -c encoder_random_access_vtm.cfg -c multi_layer/two_layers.cfg -l0 -c 2K_sequence.cfg -l1 -c 4K_sequence.cfg -l0 -q 22 -l1 -q 24

Tested configuration

A total of 5 presets (slower, slow, medium, fast, and faster) are defined as is the case with VVenC-1.4.0. Coding tool selection and partitioning constraints for those presents are not changed from the original VVenC definition.

Simulation results

In the simulation, the 2K sequence generated by SHVC down sampling filter and the original 4K sequence are encoded using both VTM-16.0 and VVenC-multilayer. The BD-rate values of {Y, Cb, Cr} components of VVenC-multilayer with different presets over VTM-16.0 are calculated from PSNR values of the enhancement layer and total bitrate. The encoder runtime ratios against VTM-16.0 are calculated using a single thread coding. Table 2 and 3 show the BD-rate results of the case where QP for enhancement layer is equal to QP for base layer, and the case where QP for enhancement layer is greater that QP for base layer by 2, respectively.

Table 2. BD-rate of VVenC-multilayer over VTM-16.0 multilayer coding
calculated from PSNR of EL and total bitrate (QPE=QPB)

| Presets | BD-rate Y | BD-rate U | BD-rate V | EncTime | Speedup |
| --- | --- | --- | --- | --- | --- |
| slower | 0.12% | 0.26% | -0.13% | 83.19% | 1.2 |
| slow | 4.91% | 4.03% | 3.89% | 8.84% | 11.3 |
| medium | 10.45% | 10.42% | 11.36% | 2.85% | 35.1 |
| fast | 29.02% | 25.36% | 29.51% | 0.88% | 114.2 |
| faster | 40.32% | 32.75% | 43.08% | 0.44% | 226.7 |

Table 3. BD-rate of VVenC-multilayer over VTM-16.0 multilayer coding
calculated from PSNR of EL and total bitrate (QPE=QPB+2)

| Presets | BD-rate Y | BD-rate U | BD-rate V | EncTime | Speedup |
| --- | --- | --- | --- | --- | --- |
| slower | 0.26% | 0.21% | 0.15% | 84.92% | 1.2 |
| slow | 5.00% | 4.28% | 4.11% | 8.78% | 11.4 |
| medium | 10.67% | 11.36% | 11.57% | 2.95% | 33.9 |
| fast | 28.32% | 25.14% | 28.31% | 0.94% | 105.8 |
| faster | 38.50% | 35.51% | 44.16% | 0.47% | 211.5 |

It is confirmed that encoder speedups between 1.2x to 226x can be achieved. The runtime and BD-rate performance for different presets are illustrated in Figure 1. To compare the speedup tendency against the VVenC single layer coding, the base layer coding results over VTM base layer encode for different presets are also illustrated in the figure. Note that the encoding runtime for the base layer was extracted from the output log file of multilayer coding and precision of encoding runtime for each POC is logged as integer, so that the encoding runtimes of the faster preset could not be precisely extracted since encoding runtimes for some POCs are often less than 1 second and logged as 0.

From the graph, it is confirmed that the encoding speedup tendency is mostly preserved between the original VVenC-1.4.0 and the developed VVenC-multilayer.

Figure 1. Run time vs BD-rate for different presets (QPE=QPB)

Conclusion

This contribution reports an optimized VVC encoder with multilayer coding capability, which is developed based on VVenC-1.4.0. It is observed that the BD-rate performance and encoder speedup of VVenC-multilayer are mostly preserved from the original VVenC-1.4.0. The public release of the developed software is being considered and the pull request to the original repository of VVenC is also under preparation. The developed software might be useful for further investigation of the optimum rate/quality settings for base and enhancement layers in verification test planning of multilayer coding.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).