| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0093-v1 |
| --- | --- |

| Title: | EE1-2.2 related: Lightweight CNN Filter for Super-Resolution with RPR functionality in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shimin Huang Cheolkon Jung Yang Liu Ming Li | Email: | shimin_huang2022@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

This contribution presents a lightweight super-resolution filter that combines convolutional neural networks (CNNs) with existing RPR functionality in VVC. In this contribution, we modify the basic unit of MMSDAB proposed by JVET-AA0065 and add spatial attention into MMSDAB to make MMSDANet lightweight, called LMSDANet. Compared with VTM-11.0_NNVC-2.0, the proposed CNN filter achieves average {-9.16%, 17.03%, -8.28%} and {-4.13%, 6.31%, -2.26%} BD-rate gains (average on A1 and A2) in AI and RA configurations, respectively.

Introduction

A CNN filter for RPR-based super-resolution was proposed in JVET-AA0065 (AA0065 filter) to upsample the downsampled video frames, called MMSDANet. However, the network complexity of MMSDANet is too high, and the kMAC even exceeds 2000k. To address this issue, this contribution adjusts the basic unit MMSDAB for MMSDANet based on multi-scale spatial attention, called a multi-scale spatial attention block (MSSAB). The adjusted MMSDAB and MMSDANet are called as LMSDAB and LMSDANet, respectively. LMSDAB has fewer parameters and simpler network structure than MMSDAB. Thus, the number of parameters in LMSDANet is reduced from 2.1M of MMSDANet to 1.18M, and its kMAC is reduced from 2044k/pixel to 964k/pixel.

Proposed method

Network architecture

In LMSDAB, the branch used to extract multi-scale information is simplified from MMSDAB. As shown in Fig. 3, LMSDAB is as the basic unit of LMSDANet. LMSDAB aims to extract multi-scale and depth features from a large receptive field and stacking convolutional layers and emphasize important spatial and channel information by MSSAB and CAB from the extracted features. LMSDAB consists of three parts: the first is feature extraction part, the second is feature fusion part, and the third is attention enhancement part. The feature extraction part contains one 1x1 convolution layer and three 3x3 convolution layers. The feature fusion part concatenates features in the channel dimension and uses a 1x1 convolution layer for fusion and dimension reduction. The attention enhancement part uses MSSAB and CAB to enhance the fused features in both spatial and channel dimensions. The structures of LMSDANet and MMSDANet (AA0065 filter) are consistent. This contribution only modifies MMSDAB and presents LMSDAB.

Fig. 1 Network architecture of the proposed LMSDANet for Y channel.

Fig. 2 Network architecture of the proposed LMSDANet for UV channel.

Fig. 3 Network architecture of the modified MMSDAB (LMSDAB).

As shown in Figure 4, the multi-scale spatial attention is mainly composed of three spatial attention blocks, a concatenation layer and a convolution layer. The input of three SABs is respectively from the output of three branches of the feature extraction part in LMSDAB, and each proposed feature is subject to a spatial attention operation. Then, three spatial attention maps are concatenated together and go through a 3x3 convolution layer for fusion to obtain the final multi-scale spatial attention map.

Fig. 4 Network architecture of the MSSAB.

Training and inference

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed CNN filter in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel Core i9 12900k @3.9GHz |
|  | Framework: | LibTorch v1.9 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | luma up-sampling model: 1.13M/model chroma up-sampling model: 1.18M/model |
|  | Total Parameter Number | 17.5M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 15 models in total: 74 MB |
|  | Multiply Accumulate (MAC) | 964kMAC/pixel |
| Optional | Total Conv. Layers | 91 for up-sampling the luma, 92 for up-sampling the chroma |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |

DIV2K [2] and BVI_DVC [3] datasets are used for training. The official NNVC anchor (VTM-11.0 [4] + new MCTF [5]) is utilized to compress these images with QPs {22, 27, 32, 37, 42}. In the context of RPR scheme, the training data will be down-sampled by a factor of 2 before encoding. Then, we use the decoded low-resolution images and the corresponding original images for training. We train a separate model for each QP for the three channels Y, U, V, thus there are 15 models in total. The models are fixed.

Table 2 Network information for the proposed CNN filter in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA 3090 24GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | luma:50, chroma:50 |
|  | Batch size: | 64 |
|  | Training time: | 26h/model |
|  | Training data information: | DIV2K, BVI_DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0_NNVC-2.0 + new MCTF, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experiment results

The proposed CNN filter for RPR-based super-resolution is tested on the official NNVC anchor (VTM-11.0_NNVC-2.0 + new MCTF) according to the common test conditions (CTC) [6]. The QP values are {22, 27, 32, 37,42} for tests. This contribution performs the evaluations in the AI and RA configurations.

Visual Comparison

Fig. 6 shows visual quality comparison in Campfire and ParkRuning3 sequence of Class A1 and A2. (c) and (e) lose much detail and are over-smoothened due to compression, while (c) has more details than (e) because of not using RPR functionality, i.e. (c) does not use downsampling and upsampling of RPR functionality. (d) and (e) are very similar in visual effects, indicating that LMSDANet maintains the performance of the AA0065 filter while reducing complexity.

Fig. 5 Visual quality comparison at QP=42 in Campfire (Top) and ParkRuning3 (Bottom) sequence of Class A1 and A2. (a) Input low-resolution images after RPR downsampling. (b) Uncompressed high-resolution images. (c) Compressed images at QP=42 without RPR functionality. (d) AA0065 filter (MMSDANet) with RPR functionality. (e) Proposed filter (LMSDANet) with RPR functionality.

Quantitative Measurements

BD-rates over VTM-11.0_NNVC-2.0 in AI and RA configurations are shown in Table 3 and Table 4.

- Under AI configuration, the proposed CNN filter achieves {-9.16%, 17.03%, -8.28%} BD-rate gains on average for {Y, U, V} components, respectively.

- Under RA configuration, the proposed CNN filter achieves {-4.13%, 6.31%, -2.26%} BD-rate gains on average for {Y, U, V} components, respectively.

Table 3 BD rate over VTM-11.0_NNVC-2.0 in AI configuration.

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -10.32% | -19.16% | -14.14% | #DIV/0! | #DIV/0! |
|  | FoodMarket4 | -5.14% | -4.76% | -4.72% | #DIV/0! | #DIV/0! |
|  | Campfire | -14.00% | 139.78% | -18.34% | #DIV/0! | #DIV/0! |
| Class A2 4K | CatRobot1 | -8.18% | -11.20% | -14.61% | #DIV/0! | #DIV/0! |
|  | DaylightRoad2 | -3.33% | -23.53% | -11.26% | #DIV/0! | #DIV/0! |
|  | ParkRunning3 | -14.00% | 21.05% | 13.42% | #DIV/0! | #DIV/0! |
| Average on A1 |  | -9.82% | 38.62% | -12.40% | #DIV/0! | #DIV/0! |
| Average on A2 |  | -8.50% | -4.56% | -4.15% | #DIV/0! | #DIV/0! |
| Overall |  | -9.16% | 17.03% | -8.28% | #DIV/0! | #DIV/0! |

Table 4 BD rate over VTM-11.0_NNVC-2.0 in RA configuration.

|  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -7.32% | -26.63% | -13.70% | #DIV/0! | #DIV/0! |
|  | FoodMarket4 | -6.33% | -7.39% | -7.17% | #DIV/0! | #DIV/0! |
|  | Campfire | -19.76% | 38.26% | -21.17% | #DIV/0! | #DIV/0! |
| Class A2 4K | CatRobot1 | 3.43% | -8.92% | -0.67% | #DIV/0! | #DIV/0! |
|  | DaylightRoad2 | 13.01% | -18.85% | -3.89% | #DIV/0! | #DIV/0! |
|  | ParkRunning3 | -10.56% | 60.13% | 30.60% | #DIV/0! | #DIV/0! |
| Average on A1 |  | -10.22% | 1.83% | -13.20% | #DIV/0! | #DIV/0! |
| Average on A2 |  | 1.96% | 10.79% | 8.68% | #DIV/0! | #DIV/0! |
| Overall |  | -4.13% | 6.31% | -2.26% | #DIV/0! | #DIV/0! |

Complexity Comparison

Table 4 shows the complexity comparison between the proposed filter (LMSDANet) and AA0065 filter (MMSDNet).

Table 4 Complexity comparison between the proposed filter (LMSDANet) and AA0065 filter (MMSDNet).

| Filter | Number of Parameters (Each Model: MB) | Total Parameter Number(MB) | Memory Parameter (MB) | Multiply Accumulate (MAC) |
| --- | --- | --- | --- | --- |
| Proposed filter (LMSDANet) | Y:1.13M U/V:1.18 | 17.5 | 15 models:74 | 964 kMAC/pixel |
| AA0065 filter (MMSDNet) | Y:2.0M U/V:2.1M | 31 | 15 models:118 | 2044 kMAC/pixel |

Conclusion

In this contribution, we optimized the network basic unit MMSDAB in JVET-AA0065, simplified the branch structure of MMSDAB, and increased MSSABs to reduce the network parameters while maintaining the original performance. Experimental results demonstrate that LMSDANet performs slightly better than MMSDANet in JVET-AA0065 while remarkably reducing computational complexity.

We recommend further study on the proposed CNN filter by including it in EE1.

References

- S. Huang, C. Jung, Y. Liu, and M. Li, AHG11: CNN Filter for Super-Resolution with RPR functionality in VVC, JVET-AA0065, 2022.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/, R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- D. Ma, F. Zhang, and D. R. Bull, BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn).

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- P. Wennersten, C. Hollmann, and J. Ström, GOP-based temporal filter improvements, JVET-V0056, 2021.

- S. Liu, A. Segall, E. Alshina, and R. -L. Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, 2021.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).