| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0099-v1 |
| --- | --- |

| Title: | Non-EE2: CCCM with Multi-shape filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng Fang, Shuang Peng, Dong Jiang，Jucai Lin, Xue Zhang | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution proposes an improved CCCM method with multi-shape filters. In this method, five additional filters with different shapes are provided. The proposed method was implemented on top of ECM-6.0, it reportedly provides {Y, U, V} BD-rate reduction with encoder and decoder runtimes as follows:

AI: -0.01%, -0.85%, -0.83%, 110% (EncT), 98% (DecT)

Introduction

In ECM-6.0[1], 7-tap convolutional cross-component model (CCCM) prediction method with plus sign shape is adopted. In addition to plus sign spatial component, the input of convolutional filter also includes a nonlinear term and bias term.

Spatial component consists of a center (C) luma sample which is collocated with the chroma sample to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated below.

Spatial component of the convolutional filter

The nonlinear term P(X) is represented as power of two of the luma sample X and scaled to the sample value range of the content:

P(X) = ( X*X + midVal ) >> bitDepth

where midVal = 1 << (bitDepth - 1).

The bias term B represents a scalar offset between the input and output, and is set to middle chroma value (512 for 10-bit content).

The output of the filter is calculated as follows:

predChromaVal = c0C + c1N + c2S + c3E + c4W + c5P(C) + c6B

where ci are the filter coefficients.

Proposed method

In this contribution, five additional filters are all 7-tap filters, the corresponding shapes are designed as illustrated below.

| (a) Horizontal shape | (b) Vertical shape | (c) Diagonal shape | (d) Anti-diagonal shape | (e) X shape |
| --- | --- | --- | --- | --- |

Spatial component of five additional filters

- For the first shape (horizontal shape), the output of the filter is calculated as follows:

predChromaVal = c0C + c1W + c2E + c3P(C) + c4P(W) + c5P(E) + c6B

- For the second shape (vertical shape), the output of the filter is calculated as follows:

predChromaVal = c0C + c1N + c2S + c3P(C) + c4P(N) + c5P(S) + c6B

- For the third shape (diagonal shape), the output of the filter is calculated as follows:

predChromaVal = c0C + c1WN + c2ES + c3P(C) + c4P(WN) + c5P(ES) + c6B

- For the fourth shape (anti-diagonal shape), the output of the filter is calculated as follows:

predChromaVal = c0C + c1WS + c2EN + c3P(C) + c4P(ES) + c5P(EN) + c6B

- For the fifth shape (x shape), the output of the filter is calculated as follows:

predChromaVal = c0C + c1WN + c2ES + c3EN + c4WS + c5P(C) + c6B

The encoder selects the best shape from six candidate shapes by comparing rate distortion costs.

For signaling, when the CCCM flag is enabled to the current PU, one syntax element is signaled to select one of six shapes.

Experimental results

The proposed method is tested on top of ECM-6.0 under the common test conditions[2].

Results of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -1.26% | -1.52% | 109% | 100% |
| Class A2 | 0.03% | -0.84% | -0.96% | 114% | 100% |
| Class B | -0.01% | -0.92% | -0.77% | 108% | 95% |
| Class C | -0.02% | -0.51% | -0.55% | 111% | 101% |
| Class E | 0.00% | -0.78% | -0.47% | 110% | 96% |
| Overall | -0.01% | -0.85% | -0.83% | 110% | 98% |
| Class D | 0.00% | -0.48% | -0.55% | 111% | 99% |
| Class F | -0.10% | -0.79% | -0.93% | 105% | 102% |

Conclusions

This contribution suggests using multi-shape filters to improve the prediction performance of CCCM. The results show that the proposed method can bring more coding gains.

References

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).