| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0101-v1 |
| --- | --- |

| Title: | AHG11: A CNN Filter for RPR-based SR with Wavelet Decomposition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui Lan Cheolkon Jung Yang Liu Ming Li | Email: | lanhui_xidian@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

This contribution proposes a convolutional neural network (CNN) filter for reference picture resampling (RPR) based super-resolution (SR) with wavelet decomposition. The proposed CNN filter takes the LR reconstructed frame (RecLR), LR prediction frame (PreLR) and RPR upsampled frame (RPRoutput) as the input for RPR-based SR. We adopt wavelet decomposition to make RPRoutput the same size as RecLR and PreLR as well as obtain the relationship between high frequency and low frequency components. Thus, the proposed CNN filter not only learns a mapping function between LR and HR images, but also effectively removes blocking artifacts in the reconstructed frame. Experimental results show that the proposed CNN filter in Y channel achieves -8.98% and -4.05% BD-rate reductions in AI and RA configurations over VTM-11.0_NNVC-2.0 anchor, respectively.

Introduction

Although videos with higher resolution and frame rates bring a better visual experience, they make the amount of video data increase dramatically, which brings great challenges to the efficient transmission and storage. For real-time video communications, experts combine image SR with video coding to save bandwidth. In the Alliance for Open Media Video 1 (AV1) [1], video frames are encoded at low resolution (LR), and upsampled to the original resolution through bilinear or bicubic interpolation at the decoder end. In the following work, the Versatile Video Coding (VVC) [2] proposes a new strategy based on reference picture resampling (RPR) for adaptive resolution video transmission according to the network conditions. When the network bandwidth is limited, the encoder can reduce the transmitted data by encoding LR video frames. When the network condition is good or the bandwidth is sufficient, the original resolution of the video frame is transmitted to ensure the quality of the video. Fig. 1 illustrates RPR based video coding. However, in RPR, traditional interpolation is still used for upsampling, which has a limit of providing high quality reconstructed frames.

In this contribution, we propose a CNN filter for RPR-based SR in VVC based on wavelet decomposition. As shown in Fig. 1, the proposed CNN filter takes the low resolution prediction frame (PreLR), the low resolution reconstruction frame (RecLR) and the RPR upsampled frame (RPRoutput) as the input. Although PreLR and RecLR are of low resolution with blocking artifacts, they contain the original edge structure with block information that is helpful for both SR and de-blocking. RPRoutput is smooth, but provides a preliminary SR result and helps the proposed network to learn image details. Thus, the proposed CNN filter taking advantage of three inputs for RPR-based SR. Moreover, since the resolution of RPRoutput is twice that of RecLR and PreLR, we adopt wavelet decomposition that decomposes an image into four subbands (LL, HL, LH, HH) without information loss.

Fig. 1 Overview of the reference picture resampling (RPR) based video coding. PreLR is the prediction frame which is obtained after inter prediction, which contains serious blocking artifacts. RecLR is the reconstruction frame obtained by in-loop filter, which is of low resolution with the original edge structure. RPRoutput is the result after RPR up-sampling which can regarded as an initial SR result. We take PreLR, RecLR and RPRoutput as the input of our CNN filter.

Moreover, the wavelet decomposition generates the relationship between high frequency (HF) and low frequency (LF) components for RPR-based SR. To maximize feature reuse under the limited parameters, we design a residual spatial and channel attention block (RSCB). RSCB combines residual blocks with spatial attention and channel attention to learn the weighted local information and global information in different receptive fields. RSCB makes the proposed network pay more attention to the region of interest and the high frequency (HF) details within the limited complexity. Moreover, the residual structure accelerates the convergence speed of the network, prevents the gradient vanishing, and trains deeper networks. The proposed CNN filter uses only three RSCBs with both residual and dense connections to prevent information loss. Experimental results show that the proposed CNN filter in Y channel achieves -8.98% and -4.05% BD-rate reductions in AI and RA configurations over VTM-11.0 NNVC-2.0 anchor, respectively.

Fig. 2 Illustration of the proposed CNN filter for RPR-based SR. The proposed CNN filter takes RecLR, PreLR and RPRoutput as the input.

- Proposed method

As shown in Fig. 2, the proposed CNN filter does not change the RPR based video coding framework in Fig. 1. The proposed CNN filter takes three inputs of RecLR, PreLR and RPRoutput for RPR-based SR. Thus, this filter can learn the mapping relationship between LR and HR frames as well as recover textures while reducing blocking artifacts.

Fig. 3 Illustration of the proposed network architecture for RPR based super-resolution for video compression with wavelet decomposition. DWT: Discrete wavelet transform.

Network Architecture

The proposed network is composed of three parts: Feature extraction, multilevel mapping, and reconstruction. Fig. 3 illustrates the network architecture of the proposed CNN filter. The proposed CNN filter is a residual network that estimates a residual map between RPRoutput and its ground truth as follows:

- The feature extraction part consists of one convolutional layer, which is used to integrate and extract the preliminary features of all the inputs. The inputs are the LR reconstructed frame, LR predicted frame and the result of RPR upsampling. Since the size of RPRoutput is twice that of other input images, these three inputs cannot be directly concatenated together. Thus, we use discrete wavelet transform (DWT) to transform RPRoutput into the frequency domain and get four subbands (LL, LH, HL, HH). The dimensions of the four subbands obtained after DWT transformation are the same as those of RecLR and PreLR. We concatenate the four frequency components with RecLR and PreLR as input to the proposed network. The benefits of using DWT are as follows: (1) DWT can convert RPRoutput into four frequency component which have the same size of RecLR and PreLR. (2) DWT is reversible and does not lose information in transform. (3) DWT reduces the size of the input video frame and the network complexity. After feature extraction, we get the initial features where the kernel size of this convolutional layer is 3×3.

- The multilevel mapping part consists of three residual spatial and channel attention blocks (RSCBs), each block has five residual spatial block (RSB). As shown in Fig. 4(a), RSB includes two convolutional layers with different kernel sizes, one spatial attention block, and one local skip connection. We combine different deep features by concatenating the output of five RSBs and use a 1×1 convolutional layer to reduce the number of channels. Finally, we use a channel attention model to obtain the weight of each channel. Based on the combination of spatial and channel attentions, the proposed network focuses more on the recovery of texture details. To reuse features in different layers and prevent information loss, we use dense connections between three RSCBs as shown in Fig. 4(b).

Fig. 4 (a) Residual spatial block (RSB). (b) Dense connections.

- The reconstruction part reconstructs the final SR video frame, which consists of two convolutional layers and one upsampling layer. The first convolutional layer reduces the number of channels where the kernel size is 1×1. We use the sub-pixel layer for upsampling and add the output of final convolutional layer with the RPRoutput, we get the HR reconstruction frame. Before upsampling, we use a global skip connection to construct a residual network[3] and strengthen the learning ability of the multilevel mapping sub-network.

Fig. 5 (a) Spatial attention block. (b) Channel attention block

- Loss function:

We use L1 and L2 loss to train the proposed network. The loss function f(x) is expressed as follows:

Loss= α*1N(xyx-yx)+1N(xyx-yx2)

where α is the coefficient to balance the L1 and L2 loss, and we set α to 0.1. We use only L1 loss to speed up the convergence until epoch is 250, then we combine L1 loss with L2 loss into the loss function to generate better results, i.e. 250<epoch<=300.

Training and Inference

In the training phase, we used DIV2K [4] and BVI-DVC [5] as training datasets and used VTM-11.0_NNVC-2.0 to compress them. Other details of the training and inference stage are described in Tables 1 and 2, respectively.

Table 1. Network information of the proposed CNN filter testing in the training stage.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA GTX 3090 |
|  | Framework: | PyTorch v1.1.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 300 |
|  | Batch size: | 64 |
|  | Training time: | ~36h/model |
|  | Training data information: | BVI-DVC, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, NNVC-2.0 QP {22, 27, 32, 37, 42} |
|  | Loss function: | Weighted L1 and L2 |
| Optional |  |  |
|  | Number of iterations | 1000/epoch |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | random flipped |
|  | Other information: |  |

Table 2: Network information for the proposed CNN filter testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 2.38 M |
|  | Total Number of Parameters (All Models) | 14 M total |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 9.6 M total |
|  | Multiply Accumulate (MAC)/pixel | 2193.9 K/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 57 Common convolution |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

- Experimental Results

Experimental Setup

- HW: PC with CPU intel i9 and GPU NVIDIA GTX 3090 (24G).

- SW: Python 3.9 with PyTorch 1.10 framework.

- Datasets for training: DIV2K (800 images), BVI-DVC (200 images)

- Training details: Batch_size = 64, Epoch = 300, Lr = 1e-4, Optimizer = Adam.

- Evaluation metrics: Visual comparison, RD curves, and BD-rates.

- Compared methods: VTM-11.0 NNVC-2.0 anchor (NNVC anchor).

- Loss function: Loss = α*1N(xyx-yx)+1N(xyx-yx2)

- Test sequences according to the common test condition (CTC): Class A1, Class A2.

Visual Quality Comparison

Fig. 6 Visual quality comparison in Tango2 sequence of A1 Class. Ground truth: Original image. PreLR, RecLR and RPRoutput: Input of the proposed CNN filter. Proposed: SR results by the proposed CNN filter.

Fig. 7 Visual quality comparison in CatRobot sequence of A2 Class. Ground truth: Original image. PreLR, RecLR and RPRoutput: Input of the proposed CNN filter. Proposed: SR results by the proposed CNN filter.

Figs. 6 and 7 provide visual quality comparison at QP 42 in Tango and CatRobot, respectively. It can be observed that PreLR is the result before in-loop filtering and contains obvious blocking artifacts. RecLR has better quality than PreLR with low resolution. RPRoutput has the same resolution as the original frame, but loses high frequency information with RPR downsampling/upsampling. Compared with RPRoutput, the proposed CNN filter can recover better high frequency details, thus reconstructing better SR results.

Objective Performance Comparison

The test results under all intra (AI) and random access (RA) configurations are shown in Tables 1 and 2, respectively. Among them, green columns represent positive gain, while pink columns represent negative gain. The tests are all conducted according to CTC[7]. We use VTM-11.0[6] with new MCTF as the baseline for tests. Tables 1 and 2 show BD-rates of the proposed CNN filter in comparison with VTM-11.0_NNVC-2.0 anchor (NNVC anchor). The proposed CNN filter has been implemented only for Y channel, thus we do not provide the test results for U and V channels in the tables. The proposed CNN filter achieves -8.98% and -4.05% BD-rate reductions on Y channel in AI and RA configurations, respectively.

Table 1 Results of the proposed CNN filter in AI configuration compared with NNVC anchor.

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -10.16% | - | - |  |  |
|  | FoodMarket4 | -3.86% | - | - |  |  |
|  | Campfire | -12.33% | - | - |  |  |
| Class A2 4K | CatRobot1 | -9.62% | - | - |  |  |
|  | DaylightRoad2 | -4.07% | - | - |  |  |
|  | ParkRunning3 | -13.86% | - | - |  |  |
| Average on A1 |  | -8.78% | - | - |  |  |
| Average on A2 |  | -9.18% | - | - |  |  |
| Overall |  | -8.98% | - | - |  |  |

Table 2 Results of the proposed CNN filter in RA configuration compared with NNVC anchor.

|  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -7.48% | - | - |  |  |
|  | FoodMarket4 | -5.07% | - | - |  |  |
|  | Campfire | -14.44% | - | - |  |  |
| Class A2 4K | CatRobot1 | 1.68% | - | - |  |  |
|  | DaylightRoad2 | 11.53% | - | - |  |  |
|  | ParkRunning3 | -10.52% | - | - |  |  |
| Average on A1 |  | -9.00% | - | - |  |  |
| Average on A2 |  | 0.90% | - | - |  |  |
| Overall |  | -4.05% | - | - |  |  |

The RD curves on the A1 and A2 sequences in AI configuration are presented in Fig. 8 and Fig. 9, while those in RA configuration are presented in Fig. 10 and Fig. 11. As shown in the figures, the proposed method achieves remarkable gains all of the A1 and A2 sequences. When the bit-rate is large (the QP is small), the NNVC anchor value is slightly better than the proposed CNN filter, but as the bit-rate decreases, the proposed CNN filter gradually exceeds NNVC anchor. The results verify that the proposed CNN filter not only saves bits significantly, but also reconstructs video frames with similar quality to NNVC anchor.

Fig. 8 RD curves for video sequences of A1 class by VTM-11.0_NNVC-2.0 anchor and the proposed CNN filter in AI configuration.

Fig. 9 RD curves for video sequences of A2 class by VTM-11.0_NNVC-2.0 anchor and the proposed CNN filter in AI configuration.

Fig. 10 RD curves for video sequences of A1 class by VTM-11.0_NNVC-2.0 anchor and the proposed CNN filter in RA configuration.

Fig. 11 RD curves for video sequences of A2 class by VTM-11.0_NNVC-2.0 anchor and the proposed CNN filter in RA configuration.

Conclusions

This contribution proposed a CNN filter for RPR-based super-resolution (SR) with wavelet decomposition. The proposed CNN filter adopts wavelet decomposition to generate the relationship between HF and LF components for RPR-based SR. Moreover, the proposed CNN filter uses PreLR, RecLR, and RPRoutput as the input to recover textures in the reconstructed frame while reducing blocking artifacts. The wavelet decomposition makes RPRoutput the same size as RecLR and PreLR, and the spatial and channel attention mechanism is combined with residual blocks in the proposed CNN filter. The proposed CNN filter achieves -8.98% and -4.05% BD-rate reductions on Y channel under AI and RA configurations over VTM-11.0_NNVC-2.0 anchor, respectively.

We recommend further study on the proposed CNN filter by including it in EE1.

- References

[1] Y. Chen, D. Mukherjee, J. Han, A. Grange, Y. Xu, S. Parker, C. Chen, H. Su, U. Joshi, C.-H. Chiang et al., “An overview of coding tools in AV1: the first video codec from the alliance for open media,” APSIPA Transactions on Signal and Information Processing, vol. 9, 2020.

[2] B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile video coding (draft 7),” JVET-P2001, 2019.

[3] K. He, X. Zhang, S. Ren, and J. Sun, “Deep Residual Learning for Image Recognition,” Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), pp. 770-778, 2016.

[4] https://data.vision.ee.ethz.ch/cvl/DIV2K/, R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

[5] https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn , Di Ma, Fan Zhang, and David R. Bull.

[6] VTM-11.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

[7] JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

- Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).