| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0107-v2 |
| --- | --- |

| Title: | Non-EE1: CNN-based super resolution with luma-only rescaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Yue Li2, Junru Li3, Kai Zhang2, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA 3Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | linchaoyi.cy@bytedance.com yue.li@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc |  |  |

_____________________________

# Abstract

This contribution combines CNN-based super resolution and CNN-based loop filter (CNNLF) in the neural common software (NCS), to support luma-only rescaling. When CNNLF (filter set #1)  in NCS-1.0 is turned on, the simulation results reportedly show {-8.63%, -7.82%, -8.34%} BD-rate savings for {Y, Cb, Cr} under AI configurations, respectively. When CNNLF is turned off, the simulation results reportedly show {-4.91%, -7.06%, -7.84%} BD-rate savings for {Y, Cb, Cr} under AI configurations, respectively, compared with NNVC-2.0.

# Introduction

Recently, CNN-based in-loop filter (CNNLF) and CNN-based super resolution show their effectiveness, respectively. However, the combination of these two tools is seldom explored. In this contribution, CNN-based super resolution [1] is integrated with the CNN-based in-loop filter [2] based on the released neural common software (NCS).

# Proposed method

In the proposed method, the rescaling operation is applied on the luma component only, to avoid the loss on chroma components, which is usually observed in super resolution methods. The down-sampled luma component and no-changed chroma components are coded with the 4:4:4 color format.

The up-sampling model for the luma component is illustrated in Fig. 1. The input to the model consists of three parts, i.e., the low-resolution luma reconstruction samples, the low-resolution luma prediction samples, and the QP map filled with the QP value. Those three parts are concatenated together and then fed into the first convolutional layer. The output from the first layer further goes through several residual blocks and one additional convolutional layer. Then, a shuffle layer generates the high-resolution reconstruction from the output of the last convolutional layer. To keep the complexity to be acceptable, in this contribution, N is set equal to 16, M is set equal to 96 and 64 for processing intra and inter slices, respectively.

We use the CNN-based in-loop filters from JVET-AA0111 [2]. To achieve better tradeoff between luma and chroma coding performance, we set chromaQpOffset=-7.

Fig. 1 The proposed model for luma up-sampling.

## Inference

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory |  | HW environment: | Intel® Xeon ® Platinum 8260 CPU @ 2.40GHz |
|  |  | Framework: | LibTorch v1.10.1 |
|  |  | Number of GPUs per Task | 0 |
|  |  | Number of Parameters (Each Model) | 1.37 M |
|  |  | Total Parameter Number | 1.37 M |
|  |  | Parameter Precision (Bits) | 32 (F) |
|  |  | Memory Parameter (MB) | 5.22 |
|  |  | Multiply Accumulate (MAC) | 342 kMAC/pixel |
| Optional |  | Total Conv. Layers | 35 for up-sampling the luma, 37 for up-sampling the chroma |
|  |  | Total FC Layers | 0 |
|  |  | Batch size: | 1 |
|  |  | Patch size | Whole frame |

## Training details

DIV2K [3] and BVI_DVC [4] datasets are used for training. The VTM-11.0 [5] + new MCTF [6] is utilized to compress these images with QPs {22, 27, 32, 37, 42}. The training data will be down-sampled by 2x before encoding. Then, we use the decoded low-resolution images and the corresponding original images for training.

Table 2 Network information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory |  | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  |  | Framework: | PyTorch v1.10 |
|  |  | Number of GPUs per Task | 1 |
|  |  | Epoch: | 120 |
|  |  | Batch size: | 16 |
|  |  | Training time: | 64 h/model |
|  |  | Training data information: | DIV2K |
|  |  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {22, 27, 32, 37, 42} |
|  |  | Loss function: | L1 |
| Optional |  | Number of iterations |  |
|  |  | Patch size | 128*128 |
|  |  | Learning rate: | 1e-4 |
|  |  | Optimizer: | ADAM |
|  |  | Preprocessing: |  |
|  |  | Other information: |  |

# Experimental results

The proposed CNN-based up-sampling filter is tested compared with NNVC-2.0, according to the common test conditions [7]. The QP values are {22, 27, 32, 37, 42}. The test results for AI configurations are shown in Table 3, Table 4, and Table 5, respectively.

Under AI configurations, when CNNLF (filter set #1) in NCS-1.0 is turned on, the proposed method can bring {-8.63%, -7.82%, -8.34%} BD-rate changes on average for {Y, U, V} components, respectively.

Under AI configurations, when CNNLF is turned off, the proposed method can bring {-4.91%, -7.06%, -7.84%} BD-rate changes on average for {Y, U, V} components, respectively.

Table 3 Results of the proposed method under AI configurations (CNNLF is turned off)

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over NNVC-2.0 (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 4K | Tango2 | -9.82% | -4.14% | -0.14% | 90% | 47% |
|  | FoodMarket4 | -4.25% | -3.85% | -1.44% |  |  |
|  | Campfire | -4.96% | -12.65% | -14.05% |  |  |
| Class A2 4K | CatRobot1 | -7.04% | -2.13% | -1.94% | 65% | 46% |
|  | DaylightRoad2 | -2.26% | -12.23% | -17.11% |  |  |
|  | ParkRunning3 | -1.13% | -7.34% | -12.33% |  |  |
| Average on A1 |  | -6.34% | -6.88% | -5.21% | 90% | 47% |
| Average on A2 |  | -3.47% | -7.23% | -10.46% | 65% | 46% |
| overall |  | -4.91% | -7.06% | -7.84% | 78% | 46% |

Table 4 Results of the proposed method under AI configurations (CNNLF is turned on)

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over NNVC-2.0 (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 4K | Tango2 | -14.22% | -5.46% | -0.90% | 118% | 8972% |
|  | FoodMarket4 | -7.77% | -5.04% | -2.97% |  |  |
|  | Campfire | -7.72% | -12.90% | -14.41% |  |  |
| Class A2 4K | CatRobot1 | -11.86% | -3.10% | -2.83% | 85% | 7170% |
|  | DaylightRoad2 | -6.92% | -12.94% | -17.60% |  |  |
|  | ParkRunning3 | -3.29% | -7.49% | -11.32% |  |  |
| Average on A1 |  | -9.90% | -7.80% | -6.10% | 118% | 8972% |
| Average on A2 |  | -7.35% | -7.84% | -10.59% | 85% | 7170% |
| overall |  | -8.63% | -7.82% | -8.34% | 102% | 8071% |

Table 5 Results of NCS-1.0 under AI configurations

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over NNVC-2.0 (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 4K | Tango2 | -7.29% | -29.95% | -28.53% | 212% | 41577% |
|  | FoodMarket4 | -7.59% | -17.16% | -18.13% |  |  |
|  | Campfire | -4.49% | -6.78% | -12.96% |  |  |
| Class A2 4K | CatRobot1 | -7.53% | -21.25% | -20.74% | 164% | 33736% |
|  | DaylightRoad2 | -5.75% | -28.67% | -21.10% |  |  |
|  | ParkRunning3 | -6.21% | -11.45% | -11.63% |  |  |
| Average on A1 |  | -6.45% | -17.96% | -19.88% | 212% | 41577% |
| Average on A2 |  | -6.50% | -20.46% | -17.82% | 164% | 33736% |
| overall |  | -6.48% | -19.21% | -18.85% | 188% | 37657% |

# Conclusion

This contribution combines CNN-based super resolution and CNN-based loop filter in the NCS with luma-only rescaling. The proposed method shows a promising coding performance. It is recommended that the proposed method to be adopted into NCS.

# References

- C. Lin, Y. Li, K. Zhang, L. Zhang, “EE1-2.4: CNN-based Super Resolution for Video Coding Using Separate Networks for Chroma Components”, JVET-Y0070, Jan. 2022.

- Y. Li, K. Zhang, J. Li, L. Zhang, H. Wang, M. Coban, A.M. Kotra, M. Karczewicz, F. Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg. “EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation”, JVET-AA0111, July. 2022

- Timofte, R., Gu, S., Wu, J., Van Gool. “Ntire 2018 challenge on single image super-resolution: Methods and results”. In Proceedings of the IEEE conference on computer vision and pattern recognition workshops, 2018.

- Di Ma, Fan Zhang, David R. Bull, “BVI-DVC: A Training Database for Deep Video Compression” (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ).

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- P. Wennersten, C. Hollmann, J. Ström, “GOP-based temporal filter improvements”, JVET-V0056, Apr. 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-AA2016, July. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).