| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0108-v1 |
| --- | --- |

| Title: | AHG11: ALF-SPLIT for NCS-1.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenjie Zou, Yi Zhou, Chengming Gu (Xidian University) Cheng Huang, Yaxian Bai, Yuanjian Zhang(ZTE Corporation) | Email: | wjzou@xidian.edu.cn yi.zhou@stu.xidian.edu.cn cmgu@stu.xidian.edu.cn huang.cheng5@zte.com.cn bai.yaxian@zte.com.cn zhang.yuanjian@zte.com.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

- Abstract

An ALF split scheme was proposed in JVET-Y0046 which proposed to signal ALF enabled/disable flag separately for luma and chroma components. It is reported that, with NCS-1.0 as anchor and test platform, the overall performance are as follows:

NCS-1.0 (filter set #1)

- AI: 0.02%/-0.71%/-0.87%

NCS-1.0 (filter set #0)

- AI: 0.01%/-0.25%/-0.14%

Introduction

In VVC [1], as depicted in Figure 1, there is an ALF Flag to indicate whether ALF is enable or disable. When this flag is on, luma ALF is always used, and then the chroma ALF and CCALF, are further be controlled by their own flags. If the ALF flag is off, the chroma ALF and CCALF will be forcibly closed.

Figure 1: Syntax structure in VVC

A method called ALF-SPLIT was proposed in JVET-Y0046[2] as shown in Figure 2. When a neural network-based filter is used as in-loop filter, two separate ALF flags are introduced, which indicate ALF enabled/disable for luma and chroma components, respectively.

Figure 2: Modified Syntax structure

Experimental results

NCS-1.0 has already been published [3]. To verify the performance of ALF-SPLIT, extensive tests were conducted based on NCS-1.0 (filter set #1) and NCS-1.0 (filter set #0).

First, the distribution of the ALF flag in the NCS-1.0 (filter set #1) [4], and NCS-1.0 (filter set #1)+AFL-SPLIT for AI configuration is shown in Table 1. The percentage of the ALF ON is calculated as:

R=NumALFon/NumSlice

where NumALFon represents the number of slices where the ALF flag is set to be ON, NumSlice is the number of slices.

With the proposed ALF-SPLIT method, it can be seen that the percentage of chromaALF and CCALF are increased by an average 14% and 8%, compared with NCS-1.0 (filter set #1). This phenomenon illustrates that the proposed method can let more ALF usage of the chroma components.

Table 1. The ratio of ALF ON(NCS-1.0 filter set #1 AI)

|  | NCS-1.0 (filter set #1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ALF |  | Cb ALF | Cr ALF | Cb CCALF | Cr CCALF |
| Class A1 | 77.70% |  | 77.70% | 77.70% | 41.06% | 43.72% |
| Class A2 | 99.65% |  | 99.65% | 99.65% | 59.65% | 59.65% |
| Class B | 90.31% |  | 77.38% | 77.38% | 53.11% | 50.77% |
| Class C | 56.82% |  | 51.05% | 51.05% | 29.37% | 27.03% |
| Class E | 100.00% |  | 100.00% | 100.00% | 60.00% | 39.29% |
| Overall | 79.24% |  | 73.28% | 73.28% | 41.85% | 37.95% |
| Class D | 30.21% |  | 7.53% | 7.53% | 12.89% | 10.13% |
| Class F | 100.00% |  | 99.67% | 99.67% | 36.90% | 35.06% |
|  | NCS-1.0 (filter set #1)+ALF-SPLIT |  |  |  |  |  |
|  | ALF | Chroma ALF | Cb ALF | Cr ALF | Cb CCALF | Cr CCALF |
| Class A1 | 77.70% | 100.00% | 100.00% | 100.00% | 48.67% | 53.10% |
| Class A2 | 99.65% | 100.00% | 100.00% | 100.00% | 60.00% | 60.00% |
| Class B | 90.31% | 85.19% | 85.07% | 85.07% | 60.00% | 57.61% |
| Class C | 56.82% | 86.44% | 78.49% | 78.49% | 55.90% | 52.55% |
| Class E | 100.00% | 100.00% | 100.00% | 100.00% | 60.00% | 39.29% |
| Overall | 79.24% | 90.13% | 87.01% | 87.01% | 50.56% | 45.24% |
| Class D | 30.21% | 59.58% | 45.86% | 45.86% | 32.47% | 19.08% |
| Class F | 100.00% | 99.67% | 99.67% | 99.67% | 36.90% | 35.06% |

In the case of NCS-1.0 (filter set #0), the ratio of ALF flag ON for chroma and CCALF increases with the proposed ALF-SPLIT method. And each class presents consistency.

Table 2. The ratio of ALF ON(NCS-1.0 filter set #0 AI)

|  | NCS-1.0 (filter set #0) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ALF |  | Cb ALF | Cr ALF | Cb CCALF | Cr CCALF |
| Class A1 | 99.12% |  | 97.70% | 97.70% | 51.68% | 55.40% |
| Class A2 | 100.00% |  | 96.49% | 96.49% | 60.00% | 60.00% |
| Class B | 92.76% |  | 82.85% | 82.85% | 58.75% | 56.58% |
| Class C | 69.04% |  | 60.84% | 60.84% | 39.92% | 38.24% |
| Class E | 100.00% |  | 100.00% | 100.00% | 60.00% | 43.29% |
| Overall | 88.01% |  | 79.89% | 79.89% | 48.91% | 44.39% |
| Class D | 55.15% |  | 22.18% | 22.18% | 24.02% | 15.48% |
| Class F | 100.00% |  | 99.16% | 99.16% | 48.03% | 41.76% |
|  | NCS-1.0 (filter set #0)+ALF-SPLIT |  |  |  |  |  |
|  | ALF | Chroma ALF | Cb ALF | Cr ALF | Cb CCALF | Cr CCALF |
| Class A1 | 99.12% | 98.41% | 98.41% | 98.41% | 51.68% | 55.40% |
| Class A2 | 100.00% | 96.49% | 96.49% | 96.49% | 60.00% | 60.00% |
| Class B | 92.76% | 85.53% | 84.96% | 84.96% | 60.00% | 57.83% |
| Class C | 69.04% | 83.77% | 79.33% | 79.33% | 57.57% | 53.14% |
| Class E | 100.00% | 100.00% | 100.00% | 100.00% | 60.00% | 43.29% |
| Overall | 88.01% | 89.04% | 86.39% | 86.39% | 54.20% | 47.67% |
| Class D | 55.15% | 59.92% | 46.36% | 46.36% | 42.09% | 22.26% |
| Class F | 100.00% | 99.16% | 99.16% | 99.16% | 48.03% | 41.76% |

Based on the platform in NCS-1.0(filter set #1), coding performance experiment was conducted following the common test conditions defined in [4]. Experimental results are shown in Table 3 and Table 4. The proposed method obtains 0.02%, -0.71%, and -0.87% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 3. Performance of the proposed method on top of NCS-1.0 (filter set #1)(AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.99% | -2.70% |  |  |
| Class A2 | 0.00% | -0.04% | -0.02% |  |  |
| Class B | 0.01% | -0.57% | -0.57% |  |  |
| Class C | 0.05% | -1.73% | -1.16% |  |  |
| Class E | 0.00% | 0.00% | 0.00% |  |  |
| Overall | 0.02% | -0.71% | -0.87% |  |  |
| Class D | 0.11% | -1.64% | -1.51% |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |

Table 4. Performance of the proposed method on top of NCS-1.0 (filter set #1)(RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.28% | -0.06% |  |  |
| Class C | 0.02% | -0.15% | -0.15% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.02% | -0.48% | -0.28% |  |  |
| Class F | 0.01% | -0.06% | -0.07% |  |  |

Based on the platform in NCS-1.0 (filter set #0), coding performance experiment was conducted following the common test conditions defined in [4]. Experimental results are shown in Table 5 and Table 6. The proposed method obtains 0.01%, -0.25%, and -0.14%( BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 5. Performance of the proposed method on top of NCS-1.0 (filter set #0)(AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% |  |  |
| Class B | 0.00% | -0.11% | -0.17% |  |  |
| Class C | 0.03% | -0.98% | -0.61% |  |  |
| Class E | 0.01% | 0.00% | 0.00% |  |  |
| Overall | 0.01% | -0.25% | -0.14% |  |  |
| Class D | 0.08% | -1.37% | -0.65% |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |

Table 6. Performance of the proposed method on top of NCS-1.0 (filter set #0)(RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.37% | -0.10% |  |  |
| Class C | 0.02% | -0.15% | -0.19% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.02% | -0.58% | -0.59% |  |  |
| Class F | 0.00% | -0.02% | -0.05% |  |  |

- 4 Conclusions

This contribution provides more flexibility for luma ALF and chroma ALF. The results from this contribution demonstrated that the proposed ALF-SPLIT shows coding gains on BD-Rate compare with NCS-1.0. It is suggested to investigate in EE based on the next version of NCS.

- 5 References

- ITU-T Study Group, Series H: Audiovisual and multimedia systems: Infrastructure of audiovisual services–coding of moving video-versatile video coding, in General Secretariat and Telecom Radiocommunication (ITU-R) Standardization (ITU-T), sec. H 266, 2020.

- Wenjie Zou, Yi Zhou, AHG11: ALF improvement for NNVC, Document JVET-Y0046, online, January 2022.

- Elena Alshina, Ru-Ling Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology , Document JVET-AA2016, online, July 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).