| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0110-v1 |
| --- | --- |

| Title: | EE2-1.15-related: Improvements on planar horizontal and planar vertical mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyungyong (Kyle) Kim Dongcheol Kim Ju-Hyung (John) Son Jin Sam Kwak | Email: | kyungyong.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a derivation method of a transform kernel for planar horizontal and planar vertical mode. If an intra prediction mode of a current block is the planar horizontal/vertical mode, the vertical/horizontal intra prediction mode is used to derive a transform kernel in MTS set and LFNST set. On top of ECM-6.0, simulation results of the proposed method are reported as below:

AI: {-0.14%, -0.03%, -0.04%, XXX%, XXX%}

RA: {}

Introduction

In JVET-AA0104 [1], two new additional planar modes are proposed where only the horizontal interpolation or only the vertical interpolation are used to obtain the predicted samples.

For planar horizontal mode, only the horizontal linear interpolation is performed based on the left reference sample and the top-right reference sample to predict the current sample as:

predx,y=W-1-x*rec-1,y+x+1*recW,-1+(W≫1)≫log2W

For planar vertical mode, only the vertical linear interpolation is performed based on the above reference sample and the bottom-left reference sample to predict the current sample as:

predx,y=H-1-y*recx,-1+y+1*rec-1,H+(H≫1)≫log2H

Proposal

In JVET-AA0104 [1], if an intra mode of a current block is one of the two additional planar modes, the planar mode is used to derive a transform kernel in MTS set and LFNST set.

However, the residual characteristics of the two additional planar modes may be similar to the residual characteristics of the horizontal and vertical direction mode. Therefore, for the planar horizontal/vertical mode, this contribution proposes to use vertical/horizontal mode to derive a transform kernel in MTS set and LFNST set as shown in Fig. 1.

If an intra prediction mode of a current block is the planar vertical mode, the horizontal intra prediction mode is used to derive a transform kernel in MTS set and LFNST set. Also, if an intra prediction mode of a current block is the planar horizontal mode, the vertical intra prediction mode is used to derive a transform kernel in MTS set and LFNST set.

Figure 1. The proposed setting method of the intra prediction mode used to derive a transform kernel in MTS set and LFNST set

Test Result

The proposed methods were implemented on top of the ECM-6.0 [2]. Simulations were performed by following the common test conditions [3]. More detailed results could be found in the attached excel sheets. The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.13% | -0.01% |  |  |
| Class A2 | -0.15% | -0.01% | -0.08% |  |  |
| Class B | -0.13% | 0.04% | 0.04% |  |  |
| Class C | -0.11% | 0.00% | -0.05% |  |  |
| Class E | -0.21% | -0.08% | -0.14% |  |  |
| Overall | -0.14% | -0.03% | -0.04% |  |  |
| Class D | -0.08% | 0.02% | 0.00% |  |  |
| Class F | -0.11% | -0.13% | 0.11% |  |  |
| Class TGM | 0.02% | 0.03% | 0.05% |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

This contribution proposes a derivation method of a transform kernel for planar horizontal & vertical mode. Compared with ECM-6.0, the proposed methods achieve -0.14% and -0.XX% gain in AI and RA configurations, respectively.

Reference

[1] X. Li, R.L. Liao, J. Chen and Y. Ye (Alibaba), “Non-EE2: On planar horizontal mode and planar vertical mode”, JVET-AA0104, July 2022.

[2] M. Coban, and et al., “Algorithm description of Enhanced Compression Model 6 (ECM 6),” JVET-AA2025, July 2022.

[3] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, January 2022.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).