| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0118-v1 |
| --- | --- |

| Title: | EE2-2.5a: Enhanced temporal motion information derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This proposal reports the performance of EE2-2.5a, which was initially proposed in JVET-AA0141. In EE2-2.5a, two aspects are studied to improve the temporal motion information derivation for sub-block-based TMVP (SbTMVP) and AMVP modes. Firstly, two collocated frames are utilized to provide temporal motion information. Secondly, the motion shift to locate TMVP or SbTMVP is adaptively determined from multiple locations according to template costs. It is reported that on top of ECM-6.0, simulation results of EE2-2.5a are as below:

|  | RA: | {-0.12%, -0.15%, -0.18%; 101%, 101%}; |
| --- | --- | --- |
|  | LDB: | {-0.14%, 0.03%,0.20%; 103%, 103%}. |

- Introduction

In ECM, the temporal MVP for AMVP mode is derived by fetching the motion information from the center or the bottom-right of the collocated block. And a similar strategy is also applied to SbTMVP mode, where the motion information from the left neighboring position is used as a motion shift, which is then employed to obtain TMVPs at sub-CU level. It is asserted that such a design may not ensure the trajectory consistency between the pre-defined positions and the current CU.

- Proposed methods

In EE2-2.5a, two aspects are proposed to further improve TMVP in SbTMVP and AMVP modes. Firstly, two collocated frames are utilized to provide temporal motion information. Secondly, the motion shift to locate TMVP is adaptively determined from multiple locations according to template costs.

Two reference frames with the least POC distance relative to the to-be-coded frame are selected to be the collocated frames. Two motion shift candidate lists are constructed respectively for the two collocated frames. When constructing the motion shift list Li for the i-th collocated frame Ci, the candidates in the motion candidate list are checked. If either MV of a motion candidate points to Ci, the corresponding MV is included into Li serving as a motion shift candidate. The motion shifts with the minimum template matching cost are used to derive SbTMVP or TMVP candidate.

In EE2-2.5a, at most 4 SbTMVP candidates are included in the sub-block-based merge list. ARMC [1] is thereby modified since more SbTMVP candidates are included. In EE2-2.5a, the SbTMVP candidate with the least template matching cost derived from the first collocated frame is placed in the first entry without reordering, while other SbTMVP candidates are sorted together with AFFINE candidates.

- Experimental results

The proposed method has been implemented on top of the ECM-6.0 [2]. Results under RA/LDB configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method (tool-off test).

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.23% | -0.05% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.11% | -0.09% | -0.19% | 101% | 101% |  |  |  |  |  |
| Class B | -0.07% | -0.11% | -0.23% | 102% | 101% | -0.08% | -0.16% | 0.31% | 103% | 102% |
| Class C | -0.20% | -0.18% | -0.21% | 102% | 102% | -0.27% | -0.28% | -0.20% | 103% | 104% |
| Class E |  |  |  |  |  | -0.04% | 0.76% | 0.54% | 103% | 102% |
| Overall | -0.12% | -0.15% | -0.18% | 101% | 101% | -0.14% | 0.03% | 0.20% | 103% | 103% |
| Class D | -0.12% | -0.11% | -0.05% | 103% | 104% | -0.16% | -0.06% | 0.82% | 103% | 103% |
| Class F | -0.03% | -0.13% | -0.05% | 102% | 102% | -0.12% | -0.46% | -0.41% | 103% | 102% |

- Conclusions

This proposal reports the EE test results for enhanced temporal motion information derivation methods. Simulation results indicate 0.12%/0.14% coding gain in RA/LDB configurations with 101%/101% and 103%/103% computational complexity. It is recommended to adopt the proposed method in ECM.

- References

[1] N. Zhang, K. Zhang, L. Zhang, H. Liu, Z. Deng, Y. Wang, “AHG12: Adaptive Reordering of Merge

Candidates with Template Matching,” JVET-V0099, Apr. 2021.

[2] ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).