| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0120-v1 |
| --- | --- |

| Title: | On HRD delivery schedule interpolation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this document a proposal is described that suggests to include a formula for an interpolated value for the initial CPB removal delay offset in section C.5, in a similar way, as done for the interpolated value of initial CPB removal delay. It is asserted that this parameter is needed for conformance and is currently missing.

# Introduction

The current text in C.5 of the VVC text relies only on the parameters bitrate, CPB and initial CPB removal delay for conformance when bitstreams are delivered from the HSS operating using an interpolated delivery schedule, i.e. an intermediate transmission bitrate between signalled operation points.

However, one important parameter is missing when it comes to the delivery of bitstreams by the HHS, namely the initial CPB removal delay offset which is asserted to be an oversight. When incorrect values of initial CPB removal delay offset are selected for intermediate operation points, buffer under- or overflows occur.

Therefore, the proposal in section 2 provides changes to derive an interpolated value for the initial CPB removal delay offset in a similar way as for the initial CPB removal delay.

The following figures show CPB fullness when the HSS operates according to each of the two delivery schedules provided by the encoder:  R0 and R1 being equal to 13.5 and 16 Mbps, respectively. The CPB sizes CPB0 and CPB1 are respectively 1400 and 1000 Megabytes with CPB initial removal delay offset values InitCpbRemovalDelayOffset0 and InitCpbRemovalDelayOffset1 equal to 30 and 0 millisecods, respectively.

The following figures show the CPB fullness for a value or a Rsel equal to 14.75 Mbps (i.e. exactly in between 13.5 and 16 Mbps) and corresponding to a CPB size of 1200 Mbytes with different values for the selected InitialCPBRemovalDelayOffset.

Selecting an InitialCPBRemovalDelayOffset equal to InitCpbRemovalDelayOffset0 (30 ms), i.e. using the value of delivery schedule with index 0 instead of interpolating, leads to a buffer overflow.

Selecting InitialCPBRemovalDelayOffset equal to InitCpbRemovalDelayOffset1 (0 ms), i.e. using the value of delivery schedule with index 1 instead of interpolating, leads to a buffer underflow.

Selecting InitialCPBRemovalDelayOffset equal to  the value resulting from a linear interpolation 
α * InitCpbRemovalDelayOffset0 + ( 1 − α ) * InitCpbRemovalDelayOffset1 with α being 
( Rsel - R0 ) /  ( R1 - R0 ), i.e. similar interpolation then is carried out for CPB size, also leads to a buffer overflow.

Selecting InitialCPBRemovalDelayOffset as computed in the proposed solution, i.e. in a similar way as the interpolation of InitialCPBRemovalDelay does not cause buffer over- or underflows.

# Proposal

The proposed modification to the specification text is shown in the following in red.

C.5	Decoder conformance

C.5.1	General

A decoder conforming to this Specification shall fulfil all requirements specified in this clause.

A decoder claiming conformance to a specific profile, tier and level shall be able to successfully decode all bitstreams that conform to the bitstream conformance requirements specified in clause C.4, in the manner specified in Annex A, provided that all DCI NAL units, when available, all VPSs, SPSs, PPSs and APSs referred to in the VCL NAL units, and appropriate BP, PT, and DUI SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified in this Specification.

When a bitstream contains syntax elements that have values that are specified as reserved and it is specified that decoders shall ignore values of the syntax elements or NAL units containing the syntax elements having the reserved values, and the bitstream is otherwise conforming to this Specification, a conforming decoder shall decode the bitstream in the same manner as it would decode a conforming bitstream and shall ignore the syntax elements or the NAL units containing the syntax elements having the reserved values as specified.

There are two types of conformance that can be claimed by a decoder: output timing conformance and output order conformance.

To check conformance of a decoder, test bitstreams conforming to the claimed profile, tier and level, as specified in clause C.4 are delivered by a hypothetical stream scheduler (HSS) both to the HRD and to the decoder under test (DUT). All cropped decoded pictures output by the HRD shall also be output by the DUT, each cropped decoded picture output by the DUT shall be a picture with PictureOutputFlag equal to 1, and, for each such cropped decoded picture output by the DUT, the values of all samples that are output shall be equal to the values of the samples produced by the specified decoding process.

For output timing decoder conformance, the HSS operates as described in clause C.2, with delivery schedules selected only from the subset of values of ScIdx for which the bit rate and CPB size are restricted as specified in Annex A for the specified profile, tier and level or with "interpolated" delivery schedules as specified by Equations 1597, 1598, and 1599 and XXXX for which the bit rate and CPB size are restricted as specified in Annex A. The same delivery schedule is used for both the HRD and the DUT.

When the HRD parameters and the BP SEI messages are present with hrd_cpb_cnt_minus1 and bp_cpb_cnt_minus1, respectively, greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r ) ÷ r ) and initial CPB removal delay offset ( o( r ) ÷ r ) as follows:

α = ( r − BitRate[ Htid ][ ScIdx − 1 ] ) ÷ ( BitRate[ Htid ][ ScIdx ] −
		BitRate[ Htid ][ ScIdx − 1 ] )		(1597)

c( r ) = α * CpbSize[ Htid ][ ScIdx ] + ( 1 − α ) * CpbSize[ Htid ][ ScIdx − 1 ]	(1598)

f( r ) = α * InitCpbRemovalDelay[ ScIdx ] * BitRate[ Htid ][ ScIdx ] +
		( 1 − α ) * InitCpbRemovalDelay[ ScIdx − 1 ] * BitRate[ Htid ][ ScIdx − 1 ]	(1599)

o( r ) = α * InitCpbRemovalDelayOffset [ ScIdx ] * BitRate[ Htid ][ ScIdx ] +
		( 1 − α ) * InitCpbRemovalDelayOffset [ ScIdx − 1 ] * BitRate[ Htid ][ ScIdx − 1 ]	(XXXX)

for any ScIdx > 0 and r such that BitRate[ Htid ][ ScIdx − 1 ]  <=  r  <=  BitRate[ Htid ][ ScIdx ] such that r and c( r ) are within the limits as specified in Annex A for the maximum bit rate and buffer size for the specified profile, tier and level.

NOTE 1 – InitCpbRemovalDelay[ ScIdx ] could be different from one BP to another and have to be re-calculated.

For output timing decoder conformance, an HRD as specified in this annex is used and the timing (relative to the delivery time of the first bit) of picture output is the same for both the HRD and the DUT up to a fixed delay.

For output order decoder conformance, the following applies:

–	The HSS delivers the bitstream BitstreamToDecode to the DUT "by demand" from the DUT, meaning that the HSS delivers bits (in decoding order) only when the DUT requires more bits to proceed with its processing.

NOTE 2 – This means that for this test, the CPB of the DUT could be as small as the size of the largest DU.

–	A modified HRD as described in the next paragraph below is used, and the HSS delivers the bitstream to the HRD by one of the schedules specified in the bitstream BitstreamToDecode such that the bit rate and CPB size are restricted as specified in Annex A. The order of pictures output shall be the same for both the HRD and the DUT.

–	The HRD CPB size is given by CpbSize[ Htid ][ ScIdx ] as specified in clause 7.4.6.3, where ScIdx and the HRD parameters are selected as specified in clause C.1. The DPB size is given by dpb_max_dec_pic_buffering_minus1[ Htid ] + 1. Removal time from the CPB for the HRD is the final bit arrival time and decoding is immediate. The operation of the DPB of this HRD is as described in clauses C.5.2 through C.5.2.3.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).