| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0126-v1 |
| --- | --- |

| Title: | AHG11: SADL update |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Franck Galpin Thierry Dumas Philippe Bordes Edouard François | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution presents updates in the Small AdHoc Deep Learning (SADL) library already described in JVET-W0181, JVET-Y0110, JVET-Z0161 and JVET-AA0086.

# Overview

The table below summarizes the framework characteristics.

| Language | Pure C++, header only. |
| --- | --- |
| Footprint | ~5000 LOC, library ~200kB, no dependency |
| Optimization | Some SIMD at hot spots, e.g. convolution (conv2D) and matrix-vector multiplication (matMul) (best effort) |
| Compatibility | Onnx to sadl converter |
| Layer Supports | constants, add, maxPool, matMul, reshape, ReLU, conv2D, mul, concat, max, leakyReLU, shape, expand, PReLU, flatten, transpose, Cond2DTranspose |
| Type support | float, int32, int16, int8 |
| Quantization | Support adaptive quantizer per layer |
| License | BSD 3-Clause |

# Changelog

Changes are also available in the repository at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl

- Documentation update

- Added Conv2Dtranspose layer for auto-encoder support.

- Add conv2D 5x5 for auto-encoder support.

- Increase maximum tensor size support

- Improve model information output

- Clean python code

# Conclusion

It is proposed to update the SADL version in NCS to the latest version.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.