| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0131 |
| --- | --- |

| Title: | EE2-3.1a: IntraTMP for chroma component |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser A. Robert T. Dumas T. Poirier F. Galpin | Email: | karam.naser@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents EE2-3.1 test results of IntraTMP for chroma components. On top of ECM-6.0, simulation results are reported as follows:

- Class F:

  - AI: -0.03%	-0.02%	0.10%	with 100% EncT and 102% DecT.

  - RA: 0.06%	 0.06%	0.10%	with 101% EncT and 105% DecT.

- TGM:

  - AI:  -0.18%	 -0.09%	 -0.21%	with 98% EncT and 96% DecT.

  - RA: -0.09%	 -0.18%	 -0.14%	with 98% EncT and 104% DecT.

# Proposed method

IntraTMP was adopted in ECM-1.0, and is only activated for class F and TGM. It is currently applied to luma component only. This test further allows IntraTMP for chroma components, where a single flag is used to indicate that both Cb and Cr components are using IntraTMP. However, template matching search is performed independently for each component.

# Experimental results

The proposed method is tested on top of ECM-6.0 software under the common test conditions [1] in AI and RA configurations (class F and TGM). The following results were obtained:

Table 1: performance of the proposed method over ECM-6.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6,0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.03% | -0.02% | 0.10% | 100% | 102% |
| Class TGM | -0.18% | -0.09% | -0.21% | 98% | 96% |
| Average | -0.10% | -0.05% | -0.06% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6,0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.06% | 0.06% | 0.10% | 101% | 105% |
| Class TGM | -0.09% | -0.18% | -0.14% | 98% | 104% |
| Average | -0.02% | -0.06% | -0.02% | 100% | 105% |

# Conclusion

It is proposed to adopt the extension of IntraTMP to the chroma components in the reference software.

# References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).