| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0139-v0 |
| --- | --- |

| Title: | EE2-related: On Chroma Fusion improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv Jinrong Zhang (vivo) | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, a new chroma fusion method is proposed where the weights are derived by the adjacent template of both the reconstructed luma samples and the predicted chroma samples obtained by applying the non-LM mode. The derivation is based on the LDL decomposition method used in CCCM. On top of ECM-6.0, the experimental results are summarized as follows:

AI: - % (Y), - % (U), - % (V),  % (EncT),  % (DecT)

RA: - % (Y), - % (U), - % (V),  % (EncT),  % (DecT)

Introduction

In ECM6.0 [1], there is one chroma fusion method that the DM mode and the four default modes can be fused with the MMLM_LT mode as follows:

pred=w0*pred0+w1*pred1+(1≪(shift-1))≫shift

where pred0 is the predictor obtained by applying the non-LM mode, pred1 is the predictor obtained by applying the MMLM_LT mode and pred is the final predictor of the current chroma block. The preset weight combination, {w0, w1} is equal to {1, 3}, {3, 1} or {2, 2}.

Proposed method

A new chroma fusion method is proposed as follows:

predCi,j= α0∙recL'i,j+α1∙predC'i,j+α2∙midValue

where predCi,j represents the final predicted chroma sample, recL'i,j represents the reconstructed luma samples. predC'i,j represents the predicted chroma samples obtained by applying the non-LM mode. For 10-bit content, the midValue is set equal to 512. The model parameters α0, α1 and α2 are derived from the same adjacent samples (two line) based on the LDL decomposition method used in CCCM.

There are three chroma fusion methods as shown in Table 1.

Table 1. The chroma fusion modes in the proposed method

| Value | Name |
| --- | --- |
| 0 | Disable |
| 1 | Preset weights |
| 2 | Single Linear model |
| 3 | Multiple Linear model |

Experimental results

The proposed method is tested on top of ECM6.0 under the common test conditions [2].

Table 2. Simulation results of the proposed method

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.03% | -0.41% | -0.34% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.04% | -0.30% | -0.37% | 103% | 99% |
| Class F |  |  |  |  |  |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.04% | -0.59% | -0.45% | 101% | 101% |
| Class F |  |  |  |  |  |

Conclusion

This contribution proposes an extension on chroma fusion prediction mode. The results show that the gain of chroma fusion mode can be further improved. It is suggested to further study this method in EE2.

References

- M. Coban, F. Le Léannec, M. Sarwer, and J. Ström, “Algorithm description of Enhanced Compression Model 5 (ECM 5),” JVET-Z2025, Jul. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).