| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0151-v1 |
| --- | --- |

| Title: | EE2-2.1 related: ARMC merge candidate list reordering for AMVP-merge mode for low-delay pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Cui, Cagan Selim Coban, Zhi Zhang, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) Hyeongmun Jang (LGE) | Tel: Email: | kaicui@qti.qualcomm.com |
| Source: | Qualcomm Incorporated, LG Electronics Inc |  |  |

_____________________________

# Abstract

This contribution proposes to use ARMC for merge candidate list reordering of AMVP-merge mode for low-delay pictures. In this mode, a merge candidate with the smallest cost after reordering is paired with the first AMVP candidate to derive the prediction. The proposed method was implemented on top of ECM-6.0. It reports -0.25% (Y), -0.19% (U), -0.10% (V) BD-rate reduction for low-delay-B configuration with 104% encoding and 100% decoding run time compared to ECM-6.0.

# Introduction

In ECM-6.0, AMVP-merge mode [2] is used for bi-directional predicted pictures. The bi-directional predictor is composed of an AMVP predictor in one direction and a merge predictor in the other direction. For AMVP direction LX, X can be 0 or 1, the merge part in the other direction (1 – LX) is implicitly derived by minimizing the bilateral matching cost between the AMVP predictor and a merge predictor. The mode can be enabled to a coding block when the selected merge predictor and the AMVP predictor satisfy DMVR condition. The bilateral matching MV refinement is applied for the merge MV candidate and AMVP MVP as a starting point. Otherwise, if template matching functionality is enabled, template matching MV refinement is applied to the merge predictor or the AMVP predictor which has a higher template matching cost. In JVET-AA0069, AMVP-merge mode is extended to low-delay pictures by bypassing the bilateral matching based merge candidates reordering and MV refinement.

# Proposed method

This contribution proposes to apply ARMC reordering process to uni-directional merge candidates of AMVP-merge mode for low-delay pictures. A merge candidate with the smallest TM cost is selected after ARMC reordering to pair with the first AMVP candidate. Merge and MVP indices are not signalled.

# Simulation results

The proposed method was implemented on top of ECM-6.0. The performance was evaluated using the common test condition defined in [3]. The test results are reported in Table 1 using ECM-6.0 as the anchor.

Table 1 Simulation results of the proposed method over ECM-6.0

|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.03% | -0.05% | XX% | XX% | -0.15% | -0.51% | -0.34% | XX% | XX% |
| Class C | 0.01% | -0.04% | 0.02% | XX% | XX% | -0.29% | -0.20% | 0.07% | XX% | XX% |
| Class E |  |  |  |  |  | -0.38% | 0.35% | 0.05% | 104% | XX% |
| Overall |  |  |  |  |  | -0.25% | -0.19% | -0.10% | 104% | 98% |
| Class D | -0.01% | 0.02% | 0.00% | XX% | XX% | -0.14% | -0.35% | 0.53% | XX% | XX% |
| Class F |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, ARMC merge candidate reordering for AMVP-merge mode for low-delay pictures is proposed. The simulation results show that a coding efficiency of {-0.25%, -0.19%, -0.10%} for YUV under low-delay B configuration has been achieved. It is suggested to adopt the proposed method into ECM.

# Reference

[1] ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] Z. Zhang, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz, “EE2: Bilateral and template matching AMVP-merge mode (test 3.3)”, JVET-X0083, Oct. 2021.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

[4] H. Jang, J. Nam, N. Park, J. Lim, S. Kim, “Non-EE2: amvpMerge for low delay”, JVET-AA0069, July 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).