| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0158-v1 |
| --- | --- |

| Title: | [AHG11] On chroma order adjustment in NNLF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenyu Dai Yue Yu Haoping Yu Dong Wang | Tel: Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes a chroma order adjustment method based on JVET-AA0088[1] to further improve the coding performance. In JVET-AA0088, the chroma information including U and V components are part of the inputs/outputs to the neural network. It is proposed to allow the input/output order switch between the U and V components at the frame level. The switch is signaled at frame level. Compared with JVET-AA0088, the proposed method demonstrates {-0.01%, -1.29%, -0.59%} and {-0.10%, -2.51%, -1.20%} BD-rate savings with the RA and LDB configurations, respectively.

# Introduction

In JVET-AA0088[1], a neural network-based in-loop filter with constrained memory size and low complexity was presented and was adopted as one of the two NNLF sets in the AHG11 reference software.

The network structure of JVET-AA0088 is shown in Fig. 1. Along with the reconstructed image (rec_yuv), additional side information is also fed into the network, such as the prediction image (pred_yuv), slice QP, base QP and slice type.

Fig. 1: Network architecture of JVET-AA0088

# Proposed method

In the current neural network-based in-loop filter design, the input tensors are constructed by concatenating the video colour components and corresponding information in a fixed order defined in the training stage. Similarly, the output video colour components are extracted from the output tensor in a fixed order. In JVET-AA0088, the input order is {rec_y, rec_u, rec_v, pred_y, pred_u, pred_v, slice QP, base QP, slice type}, while the output order is {output_y, output_u, output_v}. The same fixed orders for input and output are followed in application of the NNLF (i.e. in inference).

This contribution proposes that, instead of following a fixed order of input/output information, a frame level chroma order adjustment can be made to switch the input/output order of U and V components during the NNLF inference stage. When chroma order adjustment is enabled, the input order is modified to {rec_y, rec_v, rec_u, pred_y, pred_v, pred_u, slice QP, base QP, slice type}, and the output order is modified to {output_y, output_v, output_u}.

A frame-level flag is signaled to indicate whether the chroma order adjustment method is used.

# Experiment results

The proposed method has been implemented on top of the AHG11 reference software with the macro NnlfOption=1, which indicates the activation of the NNLF set from JVET-AA0088. The simulations were performed by following the NNVC common test conditions (CTC)[2]. Since the chroma order adjustment method is only used for B-Slice, only Random Access and Low Delay B configurations were tested.

Table 1. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over AHG11 reference software (NnlfOption=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -2.42% | -0.70% | 107% | 100% |
| Class A2 | -0.01% | -1.18% | -0.31% | 108% | 100% |
| Class B | -0.03% | -1.21% | -0.58% | 108% | 101% |
| Class C | -0.02% | -0.63% | -0.73% | 106% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -1.29% | -0.59% | 107% | 101% |
| Class D | -0.03% | -0.19% | -1.52% | 107% | 101% |
| Class F | -0.03% | -0.31% | -0.69% | 110% | 108% |

Table 2. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over AHG11 reference software (NnlfOption=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -2.67% | -0.72% | 107% | 103% |
| Class C | -0.11% | -2.31% | -1.81% | 105% | 102% |
| Class E | -0.05% | -1.10% | -1.83% | 111% | 105% |
| Overall | -0.10% | -2.51% | -1.20% | 107% | 103% |
| Class D | -0.10% | -1.95% | -4.61% | 107% | 102% |
| Class F | -0.11% | -1.52% | -2.46% | 110% | 106% |

# Conclusions

This contribution proposes a chroma order adjustment method for the NNLF set based on JVET-AA0088 in AHG11 reference software to further improve the coding performance. This method can provide a good chroma coding improvement on top of JVET-AA0088 without increasing decoder-side complexity. It is suggested to adopt this method into the NNLF set from JVET-AA0088 in the AHG11 reference software.

# References

- EE1-1.5: neural network based in-loop filter with a single model , JVET-AA0088, L. Wang, S. Lin, X. Xu, S. Liu (Tencent), F. Galpin (InterDigital)

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-AA2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).