| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0171-r1 |
| --- | --- |

| Title: | AHG 7: Asymmetric Deblocking at Virtual Boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Hong Limin Wang Krit Panusopone | Email: | seungwook.hong@nokia.com limin.2.wang@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

In the current design of VVC and ECM, deblocking is disabled at virtual boundaries of GDR/recovering pictures. Virtual boundaries are therefore likely visible. This contribution proposes asymmetric deblocking at virtual boundaries, where deblocking is still disabled for pixels on the refreshed area sides of virtual boundaries, but will be performed for pixels on the non-refreshed area sides of virtual boundaries. In addition, refreshed-area pixels of virtual boundaries are used to compensate the output of deblocking of corresponding non-refreshed area pixels.

Simulations were conducted under LDB configuration on ECM 6.0. The results demonstrate that

- Objectively, BD rates of asymmetric deblocking at virtual boundaries over no deblocking at virtual boundaries are x% for Y, y% for U and z% for V, respectively, and

- Subjectively, asymmetric deblocking improves the quality around virtual boundaries.

# Introduction

In VVC and ECM, virtual boundaries are used to divide refreshed areas and non-refreshed areas of GDR/recovering pictures. Deblocking is disabled for the pixels around virtual boundaries, which may likely make virtual boundaries visible. To reduce blocking artifacts around virtual boundaries, deblocking should be allowed across virtual boundaries. However, for GDR applications, deblocking of refreshed-area pixels around a virtual boundary of a GDR/recovering picture may require use of the coding information of non-refreshed area, which may result in leaks.

# Proposed Solution

To improve subjective quality around virtual boundaries of GDR/recovering pictures without causing leaks, it is proposed to enable asymmetric deblocking at virtual boundaries for GDR. Specifically, deblocking is still disabled for refreshed-area pixels of a virtual boundary if their deblocking requires use of the coding information of non-refreshed area, but deblocking will be performed for non-refreshed-area pixels of the virtual boundary even if their deblocking requires use of the coding information of refreshed area. In addition, refreshed-area pixels of a virtual boundary are used to compensate the output of deblocking of corresponding non-refreshed-area pixels.

Let pi and qi be the corresponding pixels on the refreshed area side and the non-refreshed area side of a virtual boundary, as shown in Fig. 1.

Fig. 1. Deblocking is performed for pixels in non-refreshed area, but not for pixels in refreshed area.

Deblocking is performed for pixel qi in the non-refreshed area with use of the coding information of both the refreshed area and the non-refreshed area as in the current VVC/ECM design, but deblocking is not performed for pixel pi in the refreshed area because deblocking of pi cannot use the coding information of the non-refreshed area.

Let pi' be the output of ideal deblocking of pi assuming use of non-refreshed area were allowed. The difference (pi'-pi) may be used to further compensate the output of the corresponding pixel qi in non-refreshed area as

qi''=qi'-12ipi'-pi

where

- qi'' is the final output of deblocking of qi, and

- qi' is the output of deblocking of qi.

# Experimental Results

The proposed asymmetric deblocking was implemented on ECM 6.0 (with fixes as indicated in [4-7]). Simulations with no deblocking at virtual boundaries and with the proposed asymmetric deblocking at virtual boundaries were conducted over JVET test sequences using low-delay B configuration with the following setting,

- The first picture is coded as an IDR picture,

- A GDR picture is inserted every second,

- GOP interval (distance between GDR picture and associated recovery point picture) is set to frame rate,

- Each inter picture refers up to 4 immediately preceding pictures in decoding order,

- The same QP is used for all pictures.

The comparison results are shown in Tables 1 in terms of BD rate. The overall BD rates of asymmetric deblocking over no deblocking at virtual boundaries are x% for Y, y% for U and z% for V, respectively.

Table 1. Comparison of asymmetric deblocking over no deblocking at virtual boundaries

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Anchor (ECM6.0 with fixes) on Linux |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.01% | 0.08% | 0.04% | 101% | #DIV/0! |
| Class E | 0.04% | 0.14% | -0.44% | 99% | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.04% | -0.25% | -0.33% | 99% | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

As examples, a few subjective quality comparisons of asymmetric deblocking against no deblocking at virtual boundaries are presented as follows.

Fig. 2. POC 80 of PartyScene of 832x480@50p.

In Fig. 2, the top image is the original picture of POC 80, and the bottom-left and the bottom-right images are a portion of the reconstructed picture (right after the deblocking process with Qp = 37) without deblocking at virtual boundary and with asymmetric deblocking at virtual boundary, respectively. It can be observed that the quality around the virtual boundary with asymmetric deblocking has been improved as compared to without deblocking at the virtual boundary.

Fig. 3. POC 115 of Basketball Pass of 416x240@50p.

In Fig. 3, the top image is the original picture of POC 115, and the bottom-left and the bottom-right images are a portion of reconstructed picture (right after the deblocking process with Qp = 37) without deblocking at virtual boundary and with asymmetric deblocking at virtual boundary, respectively. It can be seen from Fig. 3 that the virtual boundary with asymmetric deblocking is less visible than without deblocking at virtual boundary.

Fig. 4. POC 67 of Blowing Bubbles 416x240@50p.

In Fig. 4, the top image is the original picture of POC 67, and the bottom-left and the bottom-right images are a portion of reconstructed picture (right after the deblocking process with Qp = 37) without deblocking at virtual boundary and with asymmetric deblocking at virtual boundary, respectively. It can be seen from Fig. 4 that the area around the virtual boundary with asymmetric deblocking is smoother than without deblocking at virtual boundary.

Fig. 5. POC 87 of Johnny 1280x720@60p.

In Fig. 5, the top image is the original picture of POC 87, and the bottom-left and the bottom-right images are a portion of reconstructed picture (right after the deblocking process Qp = 37) without deblocking at virtual boundary and with asymmetric deblocking at virtual boundary, respectively. It can be seen from Fig. 5 that the virtual boundary with asymmetric deblocking is less visible than without deblocking at virtual boundary.

# Conclusions

In VVC and ECM, deblocking is disabled at virtual boundaries, which likely makes virtual boundaries visible. This contribution proposes asymmetric deblocking at virtual boundaries. For GDR/recovering pictures, deblocking is disabled for pixels on the refreshed-area side of a virtual boundary, but will be performed for pixels on the non-refreshed-area side of the virtual boundary. Deblocking of non-refreshed area may be further compensated by the information of refreshed area. Simulation results demonstrates that subjective quality improvement with asymmetric deblocking is indeed noticeable. It is proposed to adopt asymmetric deblocking at virtual boundaries in ECM.

# References

- B. Bross, J. Chen, S. Liu, Y-K Wang, “Versatile Video Coding”, JVET-O2001-vE, June 2020.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0.

- G. Martin-Cocher, S. Puri, T. Poirier, K. Naser, J. Xu, D. Nicholson, M. Sychev, L. Wang, S. Liu, W. Yang, J.M. Tiesse, and M. Karczewicz, “AHG-7, LLCC Scenarios and baseline configurations”, JVTE -Y0043, Jan. 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/249

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/250

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/251

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/252

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).