| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0173-v2 |
| --- | --- |

| Title: | AHG12: BVP candidates clustering and BVD sign derivation for Reconstruction-Reordered IBC mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Vikas Warudkar, Jung-Kyung Lee | Tel: Email: | duizcoll@ofinno.com, vwarudkar@ofinno.com, jlee@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution proposes modifying the IBC AMVP list construction based on clustering the block vector predictors according to the L2 distance between the candidates and selecting one candidate per cluster based on the TM cost. In addition, if the Reconstruction-Reordered IBC (RRIBC) mode is selected for IBC AMVP, two new BVP candidates are determined, and the sign of the non-null BVD component is inferred at the decoder side. On top of ECM-6.0, the test results for the proposed method for class F and class TGM are reported as follows:

AI: -0.26%, -0.22%, and -0.21%, with 99% EncT, 95% DecT.

RA: -0.22%, -0.26%, and -0.18%, with 100% EncT, 100% DecT.

# Proposed methods

The IBC list construction in the current ECM-6.0 [1] includes additional spatial candidates, a pairwise-average candidate, the number of HMVP candidates is increased to 25, and new padding candidates are added to the IBC list compared to the regular IBC list.

In addition, the IBC-TM merge mode applies a candidate's pruning method, followed by the merge list reordering (ARMC-TM) and the candidate's refinement using the Template Matching (TM) cost. In the IBC-TM AMVP mode, the three merge candidates are TM refined and sorted based on the TM cost, and the two first candidates are selected for the motion estimation process as usual.

This contribution proposes a modification to the AMVP list construction. If the number of valid BVP candidates exceeds two, up to six block vector predictor candidates are clustered based on the L2 Euclidean distance between them. The radius (R) determines a group of vectors as a logarithmic function of the width (cbWidth) and height (cbHeight) of the current block as follows:

R=Log2 ((cbWidth∙cbHeight)≫MIN_PU_SIZE) 		(1)

The clustering method is applied in the candidate list order, and the candidates assigned to a group are removed from the list for the subsequent clusters. In each group, the block vector predictor with a lower TM cost is selected as the representative candidate of that group. The representative candidates of the two first groups are chosen for the motion estimation process as in the regular IBC AMVP list.

In addition, if the Reconstruction-Reordered IBC (RRIBC) [2] mode is selected for IBC AMVP, two new BVP candidates are determined, which are adjusted to the boundaries of the valid IBC search region according to the horizontal or vertical direction indicted by the RRIBC mode. The optimal IBC AMVP index is signaled, which allows deriving the sign of the non-null BVD component at the decoder side. Consequently, the absolute value of the non-null component of the BVD is signaled to the decoder, improving the coding efficiency.

# Experimental Results

The proposed method has been implemented on top of the ECM-6.0 [3]. The simulations were performed following the common test conditions [4]. More detailed results can be found in the attached spreadsheets. Test results of the proposed method, under AI and RA configurations, are summarised in Tables I and II, respectively.

Table I. Experimental result of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.22% | -0.12% | -0.09% | 99% | 96% |
| Class TGM | -0.29% | -0.33% | -0.32% | 99% | 94% |
| Overall | -0.26% | -0.22% | -0.21% | 99% | 95% |

Table II. Experimental result of the proposed method under RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.20% | -0.27% | -0.13% | 100% | 100% |
| Class TGM | -0.23% | -0.25% | -0.23% | 100% | 100% |
| Overall | -0.22% | -0.26% | -0.18% | 100% | 100% |

- Conclusions

This contribution proposes a modification in the IBC AMVP list construction. The merge candidates are clustered according to their L2 distance, and one candidate in each cluster is selected based on the TM cost. In the case of blocks encoded using the IBC RRIBC mode, the sign of the non-null component is inferred at the decoder side. On top of ECM-6.0, the test results report some coding gain with a reduction in run-time. It is recommended to include the proposed method in the next round of EE2.

# References

- Vadim Seregin, Jie Chen, Guichun Li, Karam Naser, Jacob Ström, Martin Winken, Xiaoyu Xiu, Kai Zhang "Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)." JVET-AA2024, July 2022.

- Zhipin Deng, Kai Zhang, Li Zhang, “Non-EE2: Reconstruction-Reordered IBC for screen content coding”, JVET-Z0159, Apr. 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-6.0

- M. Karczewicz, Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," JVET-Y2017, January 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).