| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0186-v1 |
| --- | --- |

| Title: | EE2-related: Modification of extended offline-filter taps for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ikram Jumakulyyev, Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | ijumakul@qti.qualcomm.com nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-6.0, an online-trained adaptive loop filter contains 2 offline-filtered taps. In EE2-5.1, the number of offline-filtered taps is extended to 8. In this contribution, this number is further extended to 12.

On top of ECM-6.0, simulation results are reported as below:

AI: -0.11%, 0.02%, 0.02%, %, %,

RA: %, %, %, %, %,

LB: %, %, %, %, %.

On top of EE-2.5.1, simulation results are reported as below:

AI: -0.02%, 0.01%, 0.01%, %, %,

RA: %, %, %, %, %,

LB: %, %, %, %, %.

# Introduction

In ECM-6.0, as shown in Figure 1, an online-trained adaptive loop filter (ALF) contains 3 types of taps: taps applied to neighboring reconstructed samples (i.e., taps #0~#19), taps applied to offline-filtered results (i.e., taps #20 and #21) and taps applied to samples before deblocking filter (i.e., taps #22~#24).

Figure 1. Filter shape of ALF in ECM-6.0.

In EE2-5.1, the offline-filtered taps are extended (i.e., tap #20~tap #27) to improve the coding efficiency of ALF as shown in Figure 2.

Figure 2. Filter shape of ALF in EE2-5.1.

# Proposal

On top of EE2-5.1, in this contribution, the offline-filtered taps are further extended (i.e., tap #20~tap #31) as shown in Figure 3.

Figure 3. Further extended filter shape of ALF on top of EE2-5.1.

# Simulation results

The proposed method was implemented on top EE2-5.1 and tested with common test condition described in JVET-Y2017.

## Results on top of ECM-6.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.01% | 0.01% |  |  |
| Class A2 | -0.14% | 0.02% | 0.02% |  |  |
| Class B | -0.07% | 0.03% | 0.03% |  |  |
| Class C | -0.09% | 0.03% | 0.03% |  |  |
| Class E | -0.14% | 0.02% | 0.02% |  |  |
| Overall | -0.11% | 0.02% | 0.02% |  |  |
| Class D | -0.04% | 0.07% | 0.07% |  |  |
| Class F | -0.10% | 0.05% | 0.05% |  |  |
|  | Random access Main10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V |  |  |
| Class A1 | -0.10% | -0.05% | 0.03% |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.19% | 0.08% | -0.07% |  |  |
| Class C | -0.25% | -0.06% | -0.01% |  |  |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.47% | -0.23% | -0.07% |  |  |
| Class F | -0.09% | 0.00% | 0.01% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V |  |  |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class E | #VALUE! | #VALUE! | #VALUE! |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.54% | -0.12% | 0.73% |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |

## Results on top of EE2-5.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-5.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | 0.01% |  |  |
| Class A2 | -0.03% | 0.01% | 0.01% |  |  |
| Class B | -0.01% | 0.01% | 0.01% |  |  |
| Class C | -0.03% | 0.01% | 0.01% |  |  |
| Class E | 0.00% | -0.02% | -0.02% |  |  |
| Overall | -0.02% | 0.01% | 0.01% |  |  |
| Class D | -0.01% | 0.02% | 0.02% |  |  |
| Class F | -0.03% | 0.02% | 0.02% |  |  |
|  | Random access Main10 |  |  |  |  |
|  | Over EE2-5.1 |  |  |  |  |
|  | Y | U | V |  |  |
| Class A1 | -0.03% | 0.21% | -0.05% |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.02% | -0.07% | 0.02% |  |  |
| Class C | -0.10% | -0.03% | 0.00% |  |  |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.26% | -0.08% | -0.15% |  |  |
| Class F | -0.05% | 0.00% | 0.09% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over EE2-5.1 |  |  |  |  |
|  | Y | U | V |  |  |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class E | #VALUE! | #VALUE! | #VALUE! |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.24% | 0.14% | 0.35% |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).