| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0190-v1 |
| --- | --- |

| Title: | Non-EE2: Combination of JVET-AB0094 and JVET-AB0095 for screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Xue Hao, Yanzhuo Ma, Fuzheng Yang, Jing Ren, Ming Li | Email: | jyhuo@mail.xidian.edu.cn xhao@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn renjing1@oppo.com myron.li@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

Proposed is a combination of the methods proposed in JVET-AB0094 and JVET-AB0095.

The experimental results are as below:

For AI configuration:

F: -x.xx%, -x.xx %, and -x.xx %, with xxx % EncT, xxx % DecT.

TGM: -x.xx%, -x.xx %, and -x.xx %, with xxx % EncT, xxx % DecT.

For RA configuration:

F: -x.xx%, -x.xx %, and -x.xx %, with xxx % EncT, xxx% DecT.

TGM: -x.xx%, -x.xx %, and -x.xx %, with xxx% EncT, xxx% DecT.

Introduction

In ECM6.0[1], it includes a method of chroma prediction using block vector in MODE_IBC. In single tree partition, the prediction process of IBC is applied for both luma and chroma components, and the chroma block vector is derived from the corresponding luma block vector depending on the chroma format sampling structure. In dual tree partition, the prediction process of IBC is only applied for luma component.

And the BVD signs are directly coded in EP mode.

In JVET-AB0094[2], it is proposed that direct block vector (DBV) mode improves the coding efficiency of chroma intra prediction for screen content. For chroma components, when chroma dual tree is activated in intra slice, if one of the luma blocks (the following five locations in Figure 1) is coded with MODE_IBC, its block vector bvL is used and scaled to derive chroma block vector bvC. The scaling factor depends on the chroma format sampling structure.

Then, by using the position of the current chroma block (xCb, yCb) and its bvC, the corresponding offset position (xCb+ bvC[0] , yCb + bvC[1]) is determined, and a block copying prediction is performed.

Figure 1 the five locations in reconstructed luma samples

Figure 2 the prediction process of DBV mode

In JVET-AB0095[3], it is proposed that possible BVD sign combinations are sorted according to template matching cost and index corresponding to the true BVD sign is derived and coded with context model. At decoder side, the BVD signs are derived as following major steps:

- Step1: 4 candidate BVs are derived by BVP and combinations between possible signs and absolute BVD

( BVP[0] + absBVD[0], BVP[1] + absBVD[1] )

( BVP[0] + absBVD[0], BVP[1] - absBVD[1] )

( BVP[0] - absBVD[0], BVP[1] + absBVD[1] )

( BVP[0] - absBVD[0], BVP[1] - absBVD[1] )

- Step2: Derive prediction costs of 4 candidate BVs based on template matching and sort them         according to the costs

- Step3: Use BVDSP index to pick the true BVD sign

- Step4: Add the true BVD to the BVP to get the final BV

Bilinear filter is used in reference template generation. The template matching cost is measured by the SAD between the neighbouring samples of the current CU and their corresponding reference samples, as illustrated in Figure 3.

Figure 3. Template matching cost derivation in BVDSP

In general case 4 BVD candidates are evaluated (+/– bvd_abs.x, +/– bvd_abs.y). If one of BVD components (x or y) is zero, only 2 BVD candidates are possible. Case when all BVD component are zero is trivial and no sign derivation is needed.

Proposed Methods

In this contribution, a combination of JVET-AB0094 and JVET- AB0095 is proposed, that is,

- A new DBV mode is proposed to improve the coding efficiency of chroma intra prediction for screen content.

- Instead of direct signaling of BVD signs in EP mode, BVDSP mode is proposed where possible BVD signs combinations are sorted according to template matching cost and index of true BVD signs is coded with context model.

Experimental results

The proposed method was implemented based on ECM-6.0 under common test conditions [4]. Table 1 summarizes the results with all intra and random access configurations.

Table 1. Simulation results of the proposed method

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -x.xx% | -x.xx% | -x.xx% | #NUM! | #NUM! |
| Class TGM | -x.xx% | -x.xx% | -x.xx% | #NUM! | #NUM! |

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -x.xx% | -x.xx% | -x.xx% | #NUM! | #NUM! |
| Class TGM | -x.xx% | - x.xx % | - x.xx % | #NUM! | #NUM! |

Conclusions

This contribution presents a combination of JVET-AB0094 and JVET-AB0095 for screen content. Compared with ECM-6.0, the proposed method shows average BD-rates of -x.xx% and -x.xx% for class F and TGM under AI case and average BD-rates of -x.xx% and -x.xx% for class F and TGM under RA case. It is suggested to investigate in EE.

- 5 References

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- Junyan Huo, Xue Hao, et. al., “Non-EE2: Direct block vector (DBV) mode for chroma prediction”, JVET-AB0094, October 2022.

- Junyan Huo, Xue Hao, et. al., “Non-EE2: Block Vector Difference Sign Prediction (BVDSP) for IBC blocks”, JVET-AB0095, October 2022.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).