| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0206-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0160 ([AHG11] Combination of chroma order adjustment and residual adjustment for NNLF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liqiang Wang | Email: | liqiangwang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results for JVET-AB0160, which proposes a combination method of chroma order adjustment and residual adjustment based on NCS-1.0 (filter set #0).

The proposed method in JVET-AB0160 is only used for B slices, so only tests in LD and RA configurations are performed in this contribution.

It can exactly match in LD configuration while mentioning some cases in RA have minor differences which are probably due to the rounding difference.

Introduction

JVET-AB0160 proposes a combination method of chroma order adjustment [] and residual adjustment [] based on NCS-1.0 (filter set #0) [].

Chroma order adjustment: The input/output order can be adjusted between the U and V components at the frame level. In addition, the adjustment flag is signaled in bitstream.

Residual adjustment: An offset can be applied to the residual generated by NNLF. There are two candidates {1, 2} for this offset. Two frame-level flags are signaled to indicate whether this residual adjustment method is enabled for luma and chroma channels. If enabled, another offset index is signaled to indicate which offset candidate should be used for the corresponding channel.

SADL is used as the inference library, and the model precision is int16.

Results

All the experiments are performed based on the common test conditions in []. The anchor is set as NCS-1.0 (filter set #0). The test results are shown in following tables.

Table 1. Performance of JVET-AB0160

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NCS-1.0 (filter set #0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.01% | -3.36% | -1.18% | 108% | 100% |
| Class A2 | -0.10% | -1.68% | -0.55% | 107% | 99% |
| Class B | -0.11% | -1.91% | -0.80% | 108% | 103% |
| Class C | -0.09% | -1.20% | -1.07% | 106% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -1.97% | -0.90% | 107% | 102% |
| Class D | -0.04% | -0.70% | -1.97% | 107% | 103% |
| Class F | -0.04% | -0.44% | -0.70% | 111% | 113% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NCS-1.0 (filter set #0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class A2 | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.70% | -3.52% | -2.71% | 106% | 107% |
| Class E | -0.81% | -0.50% | -1.14% | 113% | 133% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.40% | -3.34% | -5.51% | 106% | 103% |
| Class F | -0.18% | -3.65% | -3.86% | 111% | 112% |

Reference

[] Z. Dai, Y. Yu, H. Yu, D. Wang. [AHG11] On chroma order adjustment in NNLF. JVET-AB0158, 28th Meeting, Mainz, DE, 20–28 October 2022.

[] Z. Dai, Y. Yu, H. Yu, D. Wang. [AHG11] On adjustment of residual for NNLF. JVET-AB0159, 28th Meeting, Mainz, DE, 20–28 October 2022.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NCS-1.0.

[] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AA2016, 27th Meeting, by teleconference, 13–22 July 2022.