| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0256-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0078 (EE2-2.1: AmvpMerge for low delay) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Woong Lim Sung-Chang Lim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-1836 woong.lim@etri.re.kr sclim@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution reports a crosscheck result for JVET-AB0078 contribution which proposes to allow AmvpMerge with bypassing bilateral matching based and with bypassing template matching based candidate reordering for MV refinement to improve coding efficiency of low-delay pictures. The crosschecking is conducted on implementation on top of ECM-6.0. It is confirmed that the partial crosscheck results are matched with those provided by the proponent.

Introduction

In JVET-AB0078[1], it was proposed that AmvpMerge is enabled with bypassing bilateral matching based merge candidate reordering and with bypassing template matching based MV refinement of AmvpMerge and included in EE2 as Test 2.1a and Test 2.1b[2].

Test A (EE2-2.1a): AmvpMerge enabled case with bypassing bilateral matching merge candidate reordering process

Test B (EE2-2.1b): Test A with bypassing template matching based MV refinement.

Experimental results

The experiments were performed based on ECM-6.0 software [3] under JVET ECM common test conditions [4]. And, the experiments were conducted on the platform of Intel Xeon Gold 6256 (3.6GHz) and 256GB RAM with GCC 9.4.0 compiler under Ubuntu 20.04.4 LTS. The partial experimental results of Test A is summarised in Table 1. And, the detailed results can be found in accompanied Excel file. (Test B is not yet completed.)

Table 1. Simulation result of Test A (EE2-2.1a)

|  | Random Access |  |  |  |  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 CTC |  |  |  |  | Over ECM-6.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.01% | 100% | 99% | -0.25% | -0.34% | -0.18% | 103% | 99% |
| Class E |  |  |  |  |  | -0.37% | -0.81% | -0.33% | 105% | 99% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.01% | 0.02% | 100% | 99% | -0.23% | -0.37% | -0.19% | 105% | 98% |
| Class F |  |  |  |  |  |  |  |  |  |  |

References

- H. Jang, J. Nam, N. Park, J. Lim and S. Kim, "EE2-2.1: AmvpMerge for low-delay," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AB0078, 28th meeting, Mainz, DE, 20-28 October 2022.

- V. Seregin, J. Chen, G. Li, K. Naser, J. Strom, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-AA2024, by teleconference, July 2022

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-6.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

Patent rights declaration(s)

ETRI does not have any current or pending patent rights relating to the technology described in this contribution.