| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0259-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0171 (AHG7: Asymmetric Deblocking at Virtual Boundaries) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Tangi Poirier | Tel: Email: | tangi.poirier@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This document reports the crosscheck contribution of JVET-AB0171[1] on Asymmetric deblocking at Virtual Boundaries. The partial results perectly match those provided in JVET-AB0171

# Introduction

In VVC and ECM, virtual boundaries are used to divide refreshed areas and non-refreshed areas of GDR/recovering pictures. Deblocking is disabled for the pixels around virtual boundaries, which may likely make virtual boundaries visible. To reduce blocking artifacts around virtual boundaries, deblocking may be allowed across virtual boundaries. However, for GDR applications, deblocking of refreshed-area pixels around a virtual boundary of a GDR/recovering picture may require use of the coding information of non-refreshed area, which may result in leaks. It is proposed to enable the filtering only on one side of the Virtual Boundary.

# Analysis

To improve subjective quality around virtual boundaries of GDR/recovering pictures without causing leaks, it is proposed to enable asymmetric deblocking at virtual boundaries for GDR. Specifically, deblocking is still disabled for refreshed-area pixels of a virtual boundary if their deblocking requires use of the coding information of non-refreshed area, but deblocking will be performed for non-refreshed-area pixels of the virtual boundary even if their deblocking requires use of the coding information of refreshed area. In addition, refreshed-area pixels of a virtual boundary are used to compensate the output of deblocking of corresponding non-refreshed-area pixels.

# Experimental results

The proposed asymmetric deblocking was implemented on ECM 6.0 (with fixes as indicated in [2-5]). Simulations with no deblocking at virtual boundaries and with the proposed asymmetric deblocking at virtual boundaries were conducted over JVET test sequences using low-delay B configuration with the following setting,

- The first picture is coded as an IDR picture,

- A GDR picture is inserted every second,

- GOP interval (distance between GDR picture and associated recovery point picture) is set to frame rate,

- Each inter picture refers up to 4 immediately preceding pictures in decoding order,

- The same QP is used for all pictures.

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.01% | 0.09% | 0.03% | 95% | 94% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.04% | -0.25% | -0.33% | 100% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

A mismatch was observed on anchor simulations for the sequence C_RaceHorses at QP 22 compared to the results from the proponents. The cause of the mismatch is not yet known.

# Conclusion

Results of JVET-AB0171 have been confirmed, subjective quality is improved at virtual boundaries.

# References

- S. Hong, L. Wang, K. Panusopone, “AHG7: Asymmetric deblocking at Virtual Boundaries”, JVET-AB0171, Oct. 2022

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/249

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/250

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/251

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/252

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).