| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0275-v1 |
| --- | --- |

| Title: | Information about Ericsson’s response to the CfP for Video Coding for Machines |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Jacob Ström, Per Wennersten, Lukasz Litwic, Rickard Sjöberg, Mitra Damghanian | Tel: Email: | {christopher.hollmann, jacob.strom, per.wennersten, lukasz.litwic, rickard.sjoberg, mitra.damghanian} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents Ericsson’s response to the Call for Proposals for Video Coding for Machines, which was issued by SC 29/WG 2 in April 2022. The proposed method only changes the VTM encoder, i.e., the bitstream is VVC compliant and the decoder unchanged. The proposed method is reported to work by setting QP offsets on a CTU basis without any additionally signaled metadata and no postprocessing was used to obtain the results. A BD-rate gain of around −23% is reported for the two video datasets that were used in the evaluation of the CfP. It is also reported that the method can be implemented on other standards such as HEVC as well.

Going forward, it is proposed to establish a new ad-hoc group to study non-normative encoder changes that benefit video compression for machine vision use cases.

# Introduction

At a joint meeting between SC 29/WG 2, SC 29/WG 4, SC 29/WG 5 and Q6/SG16 the progress of the VCM AhG in regards to the Call for Proposals issued by SC 29/WG 2 was discussed. During the discussion it was mentioned that JVET was interested in learning more about the encoder-only methods of various proposals. This contribution presents a summary of Ericsson’s response to the CfP, which is a pure encoder optimization for VTM. A more detailed description can be found in the contribution to WG 2 [1], which is also attached to this contribution.

# Proposed algorithm

The algorithm that is described in Ericsson’s response to the CfP [1] is built on top of VTM-12, which is the same version as the anchor used for the evaluation of responses to the VCM CfP. The purpose is to show the results an optimized VVC encoder can achieve without adding any benefits from changing VTM versions. However, the modifications introduced can be implemented on top of any VTM version that is VVC compliant. The bitstream that is created by the encoder is compliant to the VVC standard, there is no additional metadata, and the output of the decoder can directly be used by the task network.

As the necessary syntax elements are present in the High Efficiency Video Coding (HEVC) standard as well, it is also possible to implement the proposed method on top of HEVC, but this has not been tested. Given that QP adaption has been used for a long time, it is not unreasonable to assume that even future codecs might benefit from the proposed method.

An overview of the general architecture of the proposal is shown in Figure 1. The input video is fed into an analyzer and the encoder. The analyzer produces a list of objects that can be found in each frame. The encoder takes the result of the analysis as an additional input and uses it to assign QP offsets to different CTUs, depending on their overlap with different objects. The QP offset is signaled using the delta QP mechanism, creating a VVC-compliant bitstream. The bitstream is decoded using a VVC decoder and then the decoded video is analyzed using the networks defined in the CTC for VCM [2].

Video frames

Analyzer

Encoder

List of objects

Bitstream

Figure 1. Overview of Ericsson's response to the CfP on Video Coding for Machines

# Evaluation of the proposed method

More details about the datasets, evaluation methodology and procedure can be found in the CTC defined by the VCM AhG. The CTC also contains a set of scripts that can be used for evaluating the performance of the reconstructed videos and images. Note that these scripts can be found in the version that was published at the April 2022 meeting [2].

The CfP uses four different datasets to evaluate seven combinations of dataset and machine vision task. As only SFU-HW and TVD provide videos, the results included here are focused on these datasets. Further results can be found in the MPEG contribution [1].

Table 2. Overview of results

| MV Task | Dataset | BD-rate | BD-mAP/MOTA | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Object Detection | SFU-HW | −23.92% | 2.71 | 0.77 | 0.84 |
| Object Tracking | TVD | −22.92% | 1.78 | 0.66 | 0.95 |

The reduced encoding times primarily stem from using a higher QP for parts of the sequences and images. Note that the encoding times here include running the analyzer on a GPU system. While it is asserted that there is no significant difference when running the analyzer on CPU for low QPs, the overhead when using higher QPs can be more critical.

To reduce the complexity of the analyzer, we also tested using a light-weight analyzer instead of the Faster R-CNN and JDE networks. For this purpose, we used the YOLOv3 [3] object detector without performing any additional training or optimization of the network. This analyzer also produces a list of objects, which is then used by the encoder. However, as the analyzer detects different objects in some cases, the encoder will create a different bitstream. The results for this additional test are summarized in Table 3.

Table 3. Additional results using the YOLOv3 analyzer

| Dataset | YOLOv3 | Original analyzer |
| --- | --- | --- |
| SFU-HW-objects-v1 | −22.6% | −23.9% |
| TVD (object tracking) | −24.8% | −22.9% |

As these results show, the proposed method does not depend on having a highly complex analyzer such as Faster R-CNN but can also efficiently work with lightweight object detectors such as YOLO when compressing video. Comparing the YOLO results with the original analyzer results for the video datasets in Table 3 also shows that no performance penalty is obtained when using different networks in the analyzer and in the evaluation.

# Conclusion and discussion

In this contribution we have presented Ericsson’s response to the CfP on Video Coding for Machines. This method is an encoder-only QP adaptation mechanism that shifts bits from background areas to the areas containing objects. The shown method produces a BD-rate gain of around −23% under the test conditions for the CfP.

Going forward, it is proposed to establish a new ad-hoc group to study non-normative encoder changes that benefit video compression for machine vision use cases.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

| [1] | C. Hollmann, J. Ström, P. Wennersten, L. Litwic, R. Sjöberg and M. Damghanian, "[VCM] Response to Call for Proposals from Ericsson," m60757, Online, October 2022. |
| --- | --- |
| [2] | ISO/IEC JTC 1/SC 29/WG 2, "Common Test Conditions and Evaluation Methodology for Video Coding for Machines," N00192, Online, April 2022. |
| [3] | J. Redmon and A. Farhadi, "YOLOv3: An Incremental Improvement," arXiv, 2018. |

Given that the task network can work on raw YUV files. Otherwise, some conversion to e.g. the PNG format is needed. The networks used in the CfP use the PNG format as input and the conversion is described in the CTC.