INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 2

MPEG TECHNICAL REQUIREMENTS

ISO/IEC JTC 1/SC 29/WG 2  m60757

Online – October 2022

| Title: | [VCM] Response to Call for Proposals from Ericsson |
| --- | --- |
| Source: | Christopher Hollmann, Jacob Ström, Per Wennersten, Lukasz Litwic, Rickard Sjöberg, Mitra Damghanian (Ericsson) |

# Abstract

In this contribution Ericsson’s response to the call for proposals for video coding for machines is described. The response, named Ericsson Machine Vision Codec (EMVC), is based on the ISO/IEC SC29 MPEG Versatile Video Coding (VVC) standard, and can be decoded using any compliant VVC decoder. The main technology is an encoder-only module that determines a QP offset on a Coding Tree Unit (CTU) basis depending on whether the CTU contains any objects or not. If the CTU contains objects, a further differentiation is made based on the size of the objects. Furthermore, the Random Access configuration is modified to use a higher quantization parameter on frames on a high temporal layer.

In terms of compression efficiency, EMVC achieves the same machine vision performance at lower bitrates compared to the anchor. For the SFU-HW-objects-v1 dataset the BD-rate gain is reported to be −23.92% (video object detection). For TVD, the BD-rate gain is reported at −22.92% (video object tracking), −31.20% (image object detection), and −37.72% (image instance segmentation). For OpenImages, the BD-rate gain is reported to be −8.65% (image object detection) and −10.96% (image instance segmentation). Lastly, for FLIR the BD-rate gain is reported to be −30.84% (image object detection) when only considering the lowest four rate points.

# Introduction

In July 2019 the MPEG Requirements subgroup started an investigation into the area of compressing video for machine vision purposes. This investigation was later continued under the auspices of the newly formed Working Group 2 (WG2). WG2 gathered uses cases and requirements for a potential standardization activity, leading to a call for evidence, issued in January 2021 [1]. Based on the outcome of the call for evidence, WG2 continued its work toward a formal standardization activity.

At the 7th WG2 meeting in April 2022, a call for proposals for video compression technologies for machine vision tasks was issued [2]. This call was updated at the following meeting without substantial changes to the evaluation methodology [3]. In this contribution we are describing our proposed solution that addresses the requirements produced by WG2 [4].

Our proposal to encode videos for machine vision purposes is called the Ericsson Machine Vision Codec (EMVC).

# Description of EMVC

The technology on which EMVC is building on is known from previous standardization work. It primarily consists of changes to the reference software encoder of the Versatile Video Coding (VVC) standard [5]. All changes are made to the VVC Test Model (VTM [6]) on the encoder side, leaving the decoder unchanged. Thus, bitstreams generated using EMVC can be decoded using any conforming VVC decoder such as VTM or VVdeC [7].

An overview of the different aspects of VVC can be found in [8]. A more detailed description of the algorithms that are part of VVC can be found in [9].

All changes are implemented on top of VTM-12.0, which is the same version as the anchor used for the evaluation of responses to the VCM CfP. The purpose is to show the results EMVC can achieve without adding any benefits from changing VTM versions. However, the modifications introduced by EMVC can be implemented on any VTM version that is VVC compliant.

As the necessary syntax elements are present in the High Efficiency Video Coding (HEVC) standard [10] as well, it is also possible to implement EMVC on top of HEVC, but this has not been tested. Even future codecs might serve as a basis for EMVC.

An overview of the general EMVC architecture is shown in Figure 1. The input data is fed into an analyzer and the encoder. The encoder takes the result of the analysis as an additional input and produces the bitstream. The bitstream is decoded using a VVC decoder and then the decoded video is analyzed using the networks defined in the CTC for VCM [11].

Video frames

Analyzer

Encoder

List of objects

Bitstream

Figure 1. Overview of EMVC

## The analyzer

The input video is analyzed frame-by-frame to find what objects are present in each frame. This is done with the help of a neural network, and the following neural networks are used to analyze the inputs:

- Faster R-CNN X101-FPN [12]: OpenImages (object detection), TVD (object detection and instance segmentation), FLIR (fine-tuned version), SFU-HW-objects-v1

- Mask R-CNN X101-FPN [12]: OpenImages (instance segmentation)

- JDE-1088x608 [13]: TVD (object tracking)

The analyzer produces a list of objects that can be found in each frame. This list is then used by the encoder as an additional input, on top of the encoder configuration and general information about the video-to-be-encoded.

However, EMVC is not depending on the listed networks but can work with nearly any other neural network (or even a non-neural network-based method) that produces an indication as to where objects in the current sequence can be found. The current implementation works on the bounding boxes of objects but can be expanded to work on other representations of objects such as segmentation masks. The usage of masks however increases the complexity of the algorithm as a more complicated overlap calculation between coding tree units (CTU) and the object masks needs to be applied. The overlap calculation used in EMVC is described in Section 2.2.2.2.

## The encoder

The base software for the encoder is VTM-12.0. The following subsections describe the changes made to the encoder.

### Input parameters

A new module has been implemented that handles reading and conversion of the list of objects. The encoder takes five new arguments:

- VCM (bool): indicates whether the VCM module shall be used

- VCMObjectFile (string): the path to the file containing the description of objects

- VCMFormat (int): a number indicating the format of the object file

- VCMThreshold (float): a threshold for using objects, objects with a score below the threshold are ignored

- VCMLargeObjects (int): a threshold indicating how many pixels an object needs to cover to be considered a large object

### QP offset calculation

The main change and heart of the algorithm is the QP offset calculation. EMVC uses adaptive QP to reduce the number of bits used to code background areas and other parts of the picture that are irrelevant for machine vision tasks. The QP is changed at the CTU level. While the VVC standard allows for more fine-grained QP changes at the coding unit (CU) level [9], EMVC utilizes changing QPs only at the CTU level, which is the coarsest level.

#### Certainty and size thresholds

The algorithm uses two different thresholds: one for the certainty score and one for large objects. A description of the thresholds can be found below. Note the that same values were used for the thresholds for all datasets, i.e., no dataset-specific thresholds were chosen.

Certainty threshold TC: Most networks provide a score between 0 and 1 that indicates how certain the network is that an object of the indicated class can be found at the described position. A low score indicates that the network is unsure about an object, whereas a score close to 1 indicates that the network is very sure about the object being there.

If the score of an object is less then TC, the object is treated as if it did not exist. The algorithm determining the QP offsets ignores objects with too low a score. The threshold is used to reduce the number of false positive, i.e., erroneous detection of objects. Falsely detected objects will increase the bitrate without providing any performance increase.

If no score is given, as for example by the JDE-1088x608 network, the score is assumed to be 1, i.e., no objects that are found by the network are ignored.

TC can be set using the VCMThreshold input parameter. In EMVC TC is set to 0.5.

Large object threshold TL: This threshold is used to classify objects as either small or large objects. The size of an object is determined by the size of the bounding box. The value of the threshold is the number of pixels that an object at least needs to cover to be considered a large object, i.e., if the bounding box of an object covers more pixels than TL, it is treated as a large object, otherwise as a small object.

A large object can often be detected even when the quality is lower, thus allowing the method to reduce the bitrate in areas where only large objects are present. If a sequence has a small frame size of less than TL, all objects will be considered to be small as there is no adjustment of the threshold based on the size of the frame.

TL can be set using the VCMLargeObjects input parameter. In EMVC TL is set to 150,000 (corresponding to an object of 500x300 pixels).

#### Categorization

Furthermore, the algorithm determines the overlap of the bounding boxes of objects and each CTU. This is done using the following formula, where xOL and xOR are the left and right side of the bounding box, xCL and xCR the left and right side of the CTU, yOT and yOB are the top and bottom of the object, and yCT and yCB are the top and bottom of the CTU:

overlap= minxOR,xCR-maxxOL,xCL*minyOB,yCB-maxyOT,yCTxCR-xCL*yCB-yCT

Using TC and TL as described in Section 2.2.2.1, EMVC applies the following algorithm to determine the QP offsets for each CTU:

- Assign all CTUs in the current picture to category A.

- Iterate over all objects in the current picture:

  - If the score is less than TC, continue with the next object.

  - Determine the size of the object and determine whether it is a large or a small object by comparing it against TL.

  - Iterate over all CTUs in the current picture:

    - If the object has no overlap with the current CTU, continue with the next CTU.

    - If the object is a large object and the CTU is assigned to category B, continue with the next CTU.

    - If the object is a large object and the CTU is assigned to category A, change the assignment to category B and continue with the next CTU.

    - Change the assignment of the current CTU to category C and continue with the next CTU.

- Assign the following QP offsets to each CTU:

  - Category A: +4

  - Category B: +2

  - Category C: 0

The QP offset is then signaled in the bitstream using the delta QP mechanism that is part of the VVC standard.

## Encoder configuration

The configuration for the image datasets was not changed, i.e., it is identical to the one used for the anchors. For the video datasets TVD-tracking and SFU-HW one aspect of the JVET CTC was adjusted. This aspect is the QP offset for frames on higher temporal layers. Table 1 shows the values for both the original Random Access configuration used for the anchors and the modified version used for EMVC. The group of pictures (GOP) size was not changed and is set to 32, as it is for the anchor.

Table 1. QP offset for frames in the anchor and EMVC

| Temporal layer | Picture Order Count | Original config (anchor) | Modified config (EMVC) |
| --- | --- | --- | --- |
| 0 | 32 | -1 | -1 |
| 1 | 16 | 0 | 0 |
| 2 | 8, 24 | 0 | 5 |
| 3 | 4, 12, 20, 28 | 3 | 8 |
| 4 | 2, 6, 10, 14, 18, 22, 24, 28 | 5 | 10 |
| 5 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 18, 19, 21, 23, 25, 27, 29, 31 | 6 | 11 |

To summarize, this means that the QPs used for frames 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30 and 31 in each GOP are increased by 5 compared to the anchor configuration. Note that frame 0 is either an intra picture which is treated separately or frame 32 of the previous GOP.

Besides this change and the newly added parameters, the configuration is identical to the one used for the anchors, i.e., no additional tools were enabled, disabled or changed in their way of operation.

## The decoder

As previously mentioned, the modified encoder produces VVC-compliant bitstreams. No changes were made to the decoder. All bitstreams can be decoded using any VVC-compliant decoder such as VTM or VVdeC [7].

# Fulfillment of requirements

The following table shows which requirements EMVC matches:

| Requirement | Fulfillment | Reasoning |
| --- | --- | --- |
| a) VCM shall support video coding for machine task consumption purposes. | ☒ | Decoded video can be used as input for a multitude of task networks. |
| b) VCM shall support feature coding. | ☒ | If features are represented in YCbCr or RGB format, EMVC can compress them. |
| c) VCM shall support a coding efficiency improvement for at least 30% BD-rate over the VVC standard on machine vision tasks. | ☒ | For several datasets and machine vision tasks the compression performance exceeds 30%. |
| d) VCM shall support a broad spectrum of encoding rates. | ☒ | When using the default configuration, EMVC uses QP to encode at different bitrates. |
| e) VCM shall support various degrees of delay configuration. | ☒ | JVET has defined different common test conditions for different delay configurations. |
| f) VCM shall be agnostic to network models. | ☒ | As the output of EMVC is decoded video, there is no restriction on the network model. |
| g) VCM shall be agnostic to machine task types. | ☒ | As the output of EMVC is decoded video, there is no restriction on the machine task type. |
| h) VCM shall provide description of the meaning or the recommended way of using the decoded data. | ☒ | As the output of EMVC is decoded video, the recommended way of using the data is trivial. |
| i) VCM should support the use and inclusion of information such as descriptors in its bitstream. | ☒ | Descriptors can for example be included using the Supplemental Enhancement Information (SEI) mechanism. |
| j) A single VCM bitstream shall support any number of instances of machine tasks. | ☒ | There is no limitation as the usage of the decoded data is outside the scope of EMVC. |
| k) VCM shall support at least the following colour formats; monochrome, RGB, and YUV (YCbCr). | ☒ | Monochrome, RGB and YCbCr are supported by EMVC. |
| l) VCM shall support at least the following input bit depths: 8-bit and 10-bit. | ☒ | 8-bit and 10-bit encoding and decoding are supported, along with higher bit depths. |
| m) VCM complexity shall allow for feasible implementation within the constraints of the available technology at the expected time of usage. | ☒ | The complexity of EMVC is assumed to be reasonable for implementation. |
| n) VCM shall support rectangular picture format up to 7680x4320 pixels (8K). | ☒ | 8k input data is supported. |
| o) VCM shall support fixed and variable rational frame rates for video inputs. | ☒ | Various frame rates are supported. |
| p) VCM shall support any input source from video or image. | ☒ | Images can either be treated as a sequence with a frame count of one or using a separate All Intra configuration. |
| q) VCM shall support privacy and security. | ☒ | Privacy measures need to be taken before encoding video using EMVC (e.g., blurring of faces). |

# Evaluation

EMVC was evaluated according to the procedures defined in the Common Test Conditions and Evaluation Methodology for Video Coding for Machines (CTC) [11].

## Test environment

There was no change to the evaluation networks. The same networks were also used as the analyzer as described in section 2.1.

As EMVC reduces the bitrate quite drastically, new QPs were chosen for each rate point. These new QPs were selected so that the bitrate falls into the allowed range according to Table 3 and Table 4 in the CTC [11].

The evaluation scripts provided in the CfP package were used, with some minor adjustments to the file handling.

## Test results

All tests were executed using the original resolution of the respective video or image. No scaling operations were performed. The overview of all results can be found in Table 2. Detailed results can be found in the Excel document that is part of this contribution. Note that the BD-rate results for FLIR exclude the highest two rate points as including these does not provide a monotonic rate-distortion curve, which would lead to faulty BD-rate calculations. This issue is also present in the anchor, i.e., not an EMVC-specific problem.

Table 2. Overview of results

| MV Task | Dataset | BD-rate | EncT | DecT |
| --- | --- | --- | --- | --- |
| Object Detection | OpenImages | −8.65% | 0.81 | 0.96 |
|  | FLIR | −30.84% | 1.01 | 0.99 |
|  | TVD | −31.20% | 0.82 | 1.05 |
|  | SFU-HW | −23.92% | 0.77 | 0.84 |
| Instance Segmentation | OpenImages | −10.96% | 0.89 | 1.00 |
|  | TVD | −37.72% | 0.82 | 1.05 |
| Object Tracking | TVD | −22.92% | 0.66 | 0.95 |

Based on these results, the average gain per machine vision task are:

- Object detection: −23.65%

- Instance segmentation: −24.34%

- Object tracking: −22.92%

The results can also be averaged based on the type of input data:

- Video datasets: −23.42%

- Image datasets: −23.87%

To summarize, EMVC gives a gain of around −24% compared to the anchors used for the CfP. While there are some differences between the various datasets, larger variations happen for image data whereas the results for the video datasets show a lower deviation from the average.

The reported run times are mostly lower than the corresponding anchor run times. This stems primarily from using higher QPs compared to the anchor. As the VTM encoder is a quite complex piece of software, the additional overhead from the VCM module does not measurably impact the complexity of the encoder.

The analyzer adds additional complexity. In the proposed version of EMVC fairly complex neural networks are chosen such as Faster R-CNN and Mask R-CNN. However, as EMVC is not bound to a specific analyzer, the additional complexity can be managed on a use case by use case basis.

As required in the CfP, Table 3 shows the file sizes of the compiled decoder and encoder and compares them to the size of VTM-12.0 binaries that have been compiled using the same configuration and the same system. As noted previously, there is no change to the decoder and thus the size is unchanged. The size increase for the encoder is minimal at less than half a percent.

Table 3. Comparison of encoder and decoder sizes between VTM-12 and EMVC

|  | VTM-12.0 | EMVC | Change |
| --- | --- | --- | --- |
| Encoder | 5125896 | 5144008 | +0.35% |
| Decoder | 2963568 | 2963568 | 0% |

## Supplemental results

Besides the results presented in Section 4.2, we also tested some datasets using a different analyzer. For this we used the YOLOv3 object detector [14]. This analyzer also produces a list of objects, which is then used by the EMVC encoder. However, as the analyzer detects different objects in some cases, the encoder will create a different bitstream.

The results using EMVC in combination with YOLOv3 are summarized in Table 4.

Table 4. Comparison between YOLO and original analyzer

| Dataset | YOLOv3 | Original analyzer |
| --- | --- | --- |
| SFU-HW-objects-v1 | −22.6% | −23.9% |
| TVD (object tracking) | −24.8% | −22.9% |

In terms of runtime, no measurable difference is detected due to the overwhelming complexity of the encoder compared to the analyzer.

As these results show, EMVC does not depend on having a highly complex analyzer but can also efficiently work with lightweight object detectors such as YOLO. Comparing the YOLO results with the original analyzer results for the video datasets in Table 4 also shows that no performance penalty is obtained when using different networks in the analyzer and in the evaluation.

# Discussion and conclusion

In this contribution we have presented our response to the Call for Proposals on VCM. The proposed Ericsson Machine Vision Codec (EMVC) is based on VTM-12 and compliant with the VVC standard.

The key advantage of this proposed solution is that it leverages the existing state-of-the-art VVC standard, for which it is expected that there will be hardware-support for decoders available in the near future. Since the used per-block QP adaptation functionality has been supported across many generations of video codecs, it is also a reasonable assumption that previous codecs, such as the widely supported HEVC, may as well be adapted to work with the presented solution. It is also a reasonable assumption that future video coding standards will continue providing support for QP adaptation and thus EMVC can easily be applied to those as well.

EMVC employs encoder-only modifications and showcases how VVC can be optimized for the compression of video for machine vision purposes. This optimization shows that it is possible to reduce the bitrate by around 24% for the same machine vision task performance. One important aspect is that the additional complexity of the encoder is highly depending on application requirements and left to implementors. EMVC does not require adding support for a specific, high complexity neural network analyzer, but has been demonstrated to work with small and efficient analyzers as well.

EMVC is an example realization of what a real-life VVC-based solution dedicated to machine vision tasks could be. Therefore, we propose that EMVC is used as an additional reference point on top of the anchors that all proposals are compared against. Furthermore, we expect that any starting point for standardization activity should significantly outperform EMVC which we assert is more representative of what VVC can achieve than the current VCM anchors.

Due to the short time to prepare this response, we think that EMVC does not exhaust performance gains that can be achieved with VVC. Therefore, we recommend further study to understand the full potential of using video codecs in machine vision applications. One such approach would be for MPEG to issue a technical report on the matter at hand.

# References

| [1] | ISO/IEC JTC 1/SC 29/WG 2, "Call for Evidence for Video Coding for Machines," N00042, Online, January 2021. |
| --- | --- |
| [2] | ISO/IEC JTC 1/SC 29/WG 2, "Call for Proposals for Video Coding for Machines," N00191, Online, April 2022. |
| [3] | ISO/IEC JTC 1/SC 29/WG 2, "Call for Proposals for Video Coding for Machines," N00220, Online, July 2022. |
| [4] | ISO/IEC JTC 1/SC 29/WG 2, "Use cases and requirements for Video Coding for Machines," N00190, Online, April 2022. |
| [5] | ISO/IEC, 23090-3 Versatile Video Coding, 2021. |
| [6] | ISO/IEC JTC 1/SC 29/WG 5, "VVC Reference Model (VTM)," https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git. |
| [7] | A. Wieckowski, G. Hege, C. Bartnik, C. Lehmann, C. Stoffers, B. Bross and D. Marpe, "Towards A Live Software Decoder Implementation For The Upcoming Versatile Video Coding (VVC) Codec," in Proc. IEEE International Conference on Image Processing (ICIP), Abu Dhabi, United Arab Emirates, 2020. |
| [8] | B. Bross, Y.-K. Wang, Y. Ye, S. Liu, J. Chen, G. J. Sullivan and J.-R. Ohm, "Overview of the Versatile Video Coding (VVC) Standard and its Applications," IEEE Transactions on Circuits and Systems for Video Technology, vol. 31, no. 10, pp. 3736-3764, October 2021. |
| [9] | J. Chen, Y. Ye and S. Kim, "Algorithm description for Versatile Video Coding and Test Model 12 (VTM 12)," JVET-U2002, Online, January 2021. |
| [10] | ISO/IEC, 23008-2 High Efficiency Video Coding, 2020. |
| [11] | ISO/IEC JTC 1/SC 29/WG 2, "Common Test Conditions and Evaluation Methodology," N00231, Online, July 2022. |
| [12] | Y. Wu, A. Kirillov, F. Massa, W.-Y. Lo and R. Girshick, "Detectron2," 2019. [Online]. Available: https://github.com/facebookresearch/detectron2. [Accessed 30 09 2022]. |
| [13] | Z. Wang, L. Zheng, Y. Liu and S. Wang, "Towards Real-Time Multi-Object Tracking," in The European Conference on Computer Vision (ECCV), Glasgow, UK, 2020. |
| [14] | J. Redmon and A. Farhadi, "YOLOv3: An Incremental Improvement," arXiv, 2018. |