| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0112-v5 |
| --- | --- |

| Title: | EE2-3.6: IBC-CIIP, IBC-GPM, and IBC-LIC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Kai Zhang, Li Zhang, Na Zhang (Bytedance) Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang (Kwai) | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com machangyue@kuaishou.com |
| Source: | Bytedance Inc., Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports test results for EE2 Test 3.6a, 3.6b, and 3.6e on combined IBC and intra prediction (IBC-CIIP) in JVET-AB0191 and JVET-AB0188, and EE2 Test 3.6d and 3.6f on combination of IBC-CIIP, IBC with geometry partitioning (IBC-GPM), and IBC with local illumination compensation (IBC-LIC). On top of ECM-7.0, simulation results of Test3.6a, Test 3.6b, Test3.6e, Test3.6d and Test 3.6f are reported as below:

Test 3.6a:

AI: Class F {-0.11%, -0.05%, -0.09%; 101%, 101%}, Class TGM {-0.08%, -0.05%, -0.05%; 101%, 101%};

RA: Class F{-0.09%, -0.28%, -0.22%, 101%, 99%}, Class TGM {0.03%, -0.06%	, 0.14%; 101%, 100%}.

Test 3.6b:

AI: Class F {-0.36%, -0.23%, -0.34%; 99%, 100%}, Class TGM {-0.09%, 0.00%, -0.03%; 101%, 98%};

RA: Class F {-0.21%, -0.30%, -0.25%; 100%, 100%}, Class TGM {-0.04%, -0.13%, -0.01%; 101%, 101%}.

In v5, Test 3.6a* which is the combination of Test 3.6a and IBC-CIIP AMVP is added:

AI: Class F {-0.26%, -0.15%, -0.22%; 97%, 99%}, Class TGM {-0.44%, -0.41%, -0.37%; 101%, 96%};

RA: Class F {-0.14%, -0.09%, -0.23%; 100%, 100%}, Class TGM {-0.17%, -0.17%, -0.06%; 101%, 99%}.

Test 3.6e:

AI: Class F {-0.38%, -0.29%, -0.34%; 96%, 97%}, Class TGM {-0.44%, -0.41%, -0.37%; 100%, 96%};

RA: Class F{-0.22%, -0.24%, -0.35%; 100%, 99%}, Class TGM {-0.17%, -0.17%, -0.06%; 100%, 99%}.

Test 3.6d:

AI: Class F {-2.50%, -2.37%, -2.46%; 101%, 99%}, Class TGM {-1.12%, -1.05%, -1.05%; 103%, 99%};

RA: Class F {-1.56%, -1.62%, -1.61%; 100%, 99%}, Class TGM {-0.65%, -0.59%, -0.54%; 101%, 100%}.

In v5, Test 3.6f* which is the combination of Test 3.6a, IBC-CIIP AMVP, IBC-GPM and IBC-LIC is added:

AI: Class F {-2.49%, -2.32%, -2.42%; 100%, 98%}, Class TGM {-1.38%, -1.31%, -1.34%; 103%, 96%};

RA: Class F {-1.57%, -1.65%, -1.78%; 101%, 99%}, Class TGM {-0.70%, -0.62%, -0.49%; 102%, 99%}.

Test 3.6f:

AI: Class F {-2.53%, -2.38%, -2.50%; 99%, 98%}, Class TGM {-1.38%, -1.31%, -1.34%; 103%, 96%};

RA: Class F{-1.63%, -1.64%, -1.73%; 101%, 99%}, Class TGM {-0.70%, -0.62%, -0.49%; 102%, 99%}.

In v5, test results of the combination of Test 3.6b and IBC-LIC are added:

AI: Class F {-2.19%, -1.92%, -2.18%; 101%, 99%}, Class TGM {-0.44%, -0.39%, -0.47%; 103%, 99%};

RA: Class F{-1.44%, -1.54%, -1.56%; 101%, 100%}, Class TGM {-0.26%, -0.34%, -0.21%; 101%, 100%}.

Introduction

Extensions of intra block copy consisting of IBC-CIIP, IBC-GPM, and IBC-LIC were proposed in JVET-AB0188 [1] and JVET-AB0191 [2], and established as Test 3.6a, Test 3.6b, Test3.6e, Test3.6d and Test 3.6f in JVET-AB EE2 [3].

Test 3.6a: IBC-CIIP in JVET-AB0191.

Test 3.6b: IBC-CIIP in JVET-AB0188.

Test 3.6e: Combination of 3.6a and 3.6b.

Test 3.6d: Test 3.6b + IBC-GPM + IBC-LIC.

Test 3.6f: Test3.6e + IBC-GPM + IBC-LIC.

EE-3.6a: IBC-CIIP in JVET-AB0191

Combined intra block copy and intra prediction (IBC-CIIP) is a coding tool for a CU which uses IBC with merge mode and intra prediction to obtain two prediction signals, and the two prediction signals are weighted summed to generate the final prediction. Specifically, if the intra prediction is planar or DC mode, the final prediction is obtained as follows:

P=wibc*Pibc+(1≪shift-wibc)*Pintra+(1≪(shift-1))≫shift

wherein Pibc and Pintra denote the IBC prediction signal and intra prediction signal, respectively. (wibc, shift) are set equal to (1, 2) if both the up and left CUs are intra coded, (2, 2) if one of the up and left CUs are intra coded, (3, 2) if both the up and left CUs are IBC coded. Otherwise (i.e., if the intra prediction is directional mode), the final prediction is obtained by adaptively switching the prediction samples of the intra mode and the IBC. For purpose of illustration, assuming the size of the current CU is w*h and the intra mode is horizontal or vertical, the left 3/4w*h part (horizontal mode) or top w*3/4h part (vertical mode) of the final prediction is set to intra prediction signal if both the top and left neighboring CUs are intra coded; and the left 1/2w*h part (horizontal mode) or the top w*1/2h part (vertical mode) of the final prediction is set to intra prediction signal if only one of the top and left CUs are intra coded; and the left 1/4w*h part (horizontal mode) or the top w*1/4h part (vertical mode) of the final prediction is set to intra prediction signal if both the up and left CUs are IBC or inter coded. In the above, besides the intra prediction portion, the other part of the final prediction is set to the IBC prediction samples.

EE-3.6b: IBC-CIIP in JVET-AB0188

Combined intra block copy and intra prediction (IBC-CIIP) is a coding tool for a CU which uses IBC and intra prediction to obtain two prediction signals, and the two prediction signals are weighted summed to generate the final prediction as follows:

P=wibc*Pibc+(1≪shift-wibc)*Pintra+(1≪(shift-1))≫shift

wherein Pibc and Pintra denote the IBC prediction signal and intra prediction signal. (wibc, shift) are set equal to (13, 4) and (1, 1) for IBC merge mode and IBC AMVP mode.

An intra prediction mode (IPM) candidate list is used to generate the intra prediction signal, and the IPM candidate list size is pre-defined as 2. An IPM index is signalled to indicate which IPM is used.

EE-3.6e: Combination of 3.6a and 3.6b

Test 3.6e is a direct combination of Test 3.6a and Test 3.6b. Additionally, one flag is introduced in SPS level to indicate whether the blending method from EE3.6a or EE3.6b is utilized for IBC merge mode. At encoder side, the flag is determined by the hash block hit percent which is already available at ECM encoder.

IBC with geometry partitioning (IBC-GPM)

Intra block copy with geometry partitioning mode (IBC-GPM) is a coding tool which divides a CU into two sub-partitions geometrically. The prediction signals of the two sub-partitions are generated using IBC and intra prediction. IBC-GPM can be applied to regular IBC merge mode or IBC TM merge mode. An intra prediction mode (IPM) candidate list is constructed using the same method as GPM with inter and intra prediction for intra prediction, and the IPM candidate list size is pre-defined as 3. There are 48 geometry partitioning modes in total, which are divided into two geometry partitioning mode sets as follows:

Table 1: Geometry partitioning modes in the first geometry partitioning mode set

| ibc_gpm_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 8 | 8 | 16 | 16 | 24 | 24 |
| distanceIdx | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |

Table 2: Geometry partitioning modes in the second geometry partitioning mode set

| ibc_gpm_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 5 |
| distanceIdx | 0 | 1 | 3 | 0 | 1 | 3 | 0 | 1 | 3 | 0 |
| ibc_gpm_partition_idx | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| angleIdx | 5 | 5 | 11 | 11 | 11 | 12 | 12 | 12 | 13 | 13 |
| distanceIdx | 1 | 3 | 0 | 1 | 3 | 0 | 1 | 3 | 0 | 1 |
| ibc_gpm_partition_idx | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| angleIdx | 13 | 14 | 14 | 14 | 18 | 18 | 19 | 19 | 20 | 20 |
| distanceIdx | 3 | 0 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |
| ibc_gpm_partition_idx | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| angleIdx | 21 | 21 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |
| distanceIdx | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |

When IBC-GPM is used, an IBC-GPM geometry partitioning mode set flag is signalled to indicate whether the first or the second geometry partitioning mode set is selected, followed by the geometry partitioning mode index. An IBC-GPM intra flag is signalled to indicate whether intra prediction is used for the first sub-partition. When intra prediction is used for a sub-partition, an intra prediction mode index is signalled. When IBC is used for a sub-partition, a merge index is signalled.

IBC with local illumination compensation (IBC-LIC)

Intra block copy with local illumination compensation (IBC-LIC) is a coding tool which compensates the local illumination variation within a picture between the CU coded with IBC and its prediction block with a linear equation. The parameters of the linear equation are derived same as LIC for inter prediction except that the reference template is generated using block vector in IBC-LIC. IBC-LIC can be applied to IBC AMVP mode and IBC merge mode. For IBC AMVP mode, an IBC-LIC flag is signalled to indicate the use of IBC-LIC. For IBC merge mode, the IBC-LIC flag is inferred from the merge candidate.

Simulation results

Test 3.6a, Test 3.6b, Test3.6e, Test3.6d, and Test 3.6f are implemented on top of ECM-7.0 [4]. Simulations are performed following the SCC common test conditions of EE2 [5]. The simulation results of Test3.6a, Test 3.6b, Test3.6e, Test3.6d and Test 3.6f compared to ECM-7.0 are shown in Table 3, Table 4, Table 5, Table 6, and Table 7, respectively. In addition, test results for combination of Test 3.6b and IBC-GPM are shown in Table 8, and test results for combination of Test 3.6b and IBC-LIC are shown in Table 9, and test results of Test 3.6a* are shown in Table 10, and test results of Test 3.6f* are shown in Table 11.

Table 3: Test results of Test 3.6a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.11% | -0.05% | -0.09% | 101% | 101% |
| Class TGM | -0.08% | -0.05% | -0.05% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.09% | -0.28% | -0.22% | 101% | 99% |
| Class TGM | 0.03% | -0.06% | 0.14% | 101% | 100% |

Table 4: Test results of Test 3.6b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.36% | -0.23% | -0.34% | 99% | 100% |
| Class TGM | -0.09% | 0.00% | -0.03% | 101% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.21% | -0.30% | -0.25% | 100% | 100% |
| Class TGM | -0.04% | -0.13% | -0.01% | 101% | 101% |

Table 5: Test results of Test 3.6e

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.38% | -0.29% | -0.34% | 96% | 97% |
| Class TGM | -0.44% | -0.41% | -0.37% | 100% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.22% | -0.24% | -0.35% | 100% | 99% |
| Class TGM | -0.17% | -0.17% | -0.06% | 100% | 99% |

Table 6: Test results of Test 3.6d

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -2.50% | -2.37% | -2.46% | 101% | 99% |
| Class TGM | -1.12% | -1.05% | -1.05% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.56% | -1.62% | -1.61% | 100% | 99% |
| Class TGM | -0.65% | -0.59% | -0.54% | 101% | 100% |

Table 7: Test results of Test 3.6f

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -2.53% | -2.38% | -2.50% | 99% | 98% |
| Class TGM | -1.38% | -1.31% | -1.34% | 103% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.63% | -1.64% | -1.73% | 101% | 99% |
| Class TGM | -0.70% | -0.62% | -0.49% | 102% | 99% |

Table 8: Test results of combination of Test 3.6b and IBC-GPM

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.61% | -0.47% | -0.52% | 101% | 100% |
| Class TGM | -0.80% | -0.66% | -0.66% | 102% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.34% | -0.60% | -0.60% | 100% | 99% |
| Class TGM | -0.33% | -0.30% | -0.24% | 101% | 100% |

Table 9: Test results of combination of Test 3.6b and IBC-LIC

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -2.19% | -1.92% | -2.18% | 101% | 99% |
| Class TGM | -0.44% | -0.39% | -0.47% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.44% | -1.54% | -1.56% | 101% | 100% |
| Class TGM | -0.26% | -0.34% | -0.21% | 101% | 100% |

Table 10: Test results of Test 3.6a*

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.26% | -0.15% | -0.22% | 97% | 99% |
| Class TGM | -0.44% | -0.41% | -0.37% | 101% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.14% | -0.09% | -0.23% | 100% | 100% |
| Class TGM | -0.17% | -0.17% | -0.06% | 101% | 99% |

Table 11: Test results of Test 3.6f*

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -2.49% | -2.32% | -2.42% | 100% | 98% |
| Class TGM | -1.38% | -1.31% | -1.34% | 103% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.57% | -1.65% | -1.78% | 101% | 99% |
| Class TGM | -0.70% | -0.62% | -0.49% | 102% | 99% |

Conclusions

In this contribution, test results of EE2 Test 3.6a, 3.6b, and 3.6e on IBC-CIIP, and Test 3.6d and 3.6f on combination of IBC-CIIP, IBC-GPM, and IBC-LIC are reported. It is recommended to adopt Test 3.6f into ECM.

Reference

- Y. Wang, K. Zhang, L. Zhang, and N. Zhang, “Non-EE2: Extensions of intra block copy,” Document of Joint Video Experts Team, JVET-AB0188, Mainz, October 2022.

- C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, and X. Wang, “Non-EE2: Combining intra block copy and intra mode,” Document of Joint Video Experts Team, JVET-AB0191, Mainz, October 2022.

- V. Seregin, J. Chen, G. Li, K. Naser, J. Strom, F. Wang, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-AB2024, Mainz, October 2022.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).