| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0160 |
| --- | --- |

| Title: | On the condition of OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li | Email: | xlxiangli@google.com |
| Source: | Google, Inc. |  |  |

_____________________________

# Abstract

In this contribution a method to adaptively disable OBMC is proposed based on prediction samples. 1+% and 2+% average luma coding gains are reported for TGM class in RA and LDB with similar or even less encoding time. When CU level OBMC flag is removed (method2), the encoding complexity is reportedly further reduced. The efficiency impact on camera content is reportedly negligible: average luma BD-rate differences of 0.00%, 0.00%, 0.02% by method1 and 0.00%, 0.00%, 0.04% by method2 are reported for RA, LDB, and LDP, respectively.

# Introduction

Although OBMC is normally efficient for camera content, it may introduce coding loss for some screen content cases. To avoid the potential inefficiency, a CU level on/off flag is signaled in ECM-7 for a non-merge inter CU when the LIC flag of the CU is off and the block size is no smaller than 32 luma samples. The flag on/off is decided based on RD cost at encoder. For inter merge mode, OBMC is always on.

However, the granularity of CU level control sometimes may be too coarse as OBMC is applied at boundary level. While for merge modes, there is actually no on/off control.

# Proposed methods

Essentially, OBMC is a kind of multi-hypothesis prediction, which reduces prediction error based on the assumption that the prediction error of different hypotheses are uncorrelated. Intuitively, if the hypotheses of a block are too different, the multi-hypothesis prediction may be even worse as one or more hypotheses may be wrong. Therefore, it is proposed to use prediction samples to decide whether to apply OBMC for a boundary, i.e., if the max difference of the prediction samples are above a threshold, the OBMC is not applied for that boundary.

# Experimental results

The proposed method was implemented on top of ECM-7. As the proposed method controls OBMC on/off at boundary level without signaling, two tests were conducted to evaluate the performance with and without the CU level OBMC flag.

## Test 1 (with CU level OBMC flag)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.14% | 0.04% | 99% | 99% |
| Class A2 | 0.01% | 0.10% | -0.11% | 99% | 99% |
| Class B | -0.02% | 0.04% | -0.14% | 100% | 100% |
| Class C | -0.02% | 0.02% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.07% | 99% | 100% |
| Class D | -0.04% | 0.04% | 0.07% | 100% | 100% |
| Class F | -0.15% | -0.17% | -0.09% | 100% | 100% |
| Class TGM | -1.17% | -0.82% | -0.73% | 99% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.01% | -0.04% | 99% | 100% |
| Class C | -0.06% | 0.19% | 0.28% | 100% | 100% |
| Class E | -0.02% | -0.15% | 0.11% | 100% | 99% |
| Overall | 0.00% | 0.02% | 0.11% | 99% | 100% |
| Class D | -0.05% | 0.35% | 0.31% | 100% | 101% |
| Class F | -0.25% | 0.01% | -0.36% | 99% | 101% |
| Class TGM | -2.08% | -1.81% | -1.76% | 97% | 101% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-7 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.17% | -0.10% | 99% | 99% |
| Class C | 0.02% | -0.13% | 0.08% | 101% | 101% |
| Class E | 0.07% | 0.32% | 0.21% | 100% | 102% |
| Overall | 0.02% | -0.04% | 0.04% | 100% | 100% |
| Class D | -0.05% | 0.26% | -0.04% | 101% | 101% |
| Class F | -0.39% | -0.51% | -0.84% | 100% | 100% |
| Class TGM | -1.70% | -1.35% | -1.40% | 99% | 103% |

## Test 2 (CU level OBMC flag is removed)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.27% | 0.03% | 99% | 100% |
| Class A2 | 0.03% | 0.07% | 0.02% | 99% | 99% |
| Class B | -0.02% | 0.07% | -0.07% | 100% | 100% |
| Class C | -0.02% | 0.00% | 0.02% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class D | -0.03% | -0.01% | 0.04% | 101% | 100% |
| Class F | -0.13% | -0.03% | -0.01% | 100% | 100% |
| Class TGM | -1.10% | -0.79% | -0.71% | 98% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.05% | -0.20% | 98% | 100% |
| Class C | -0.02% | 0.13% | 0.16% | 99% | 100% |
| Class E | -0.02% | -0.24% | -0.54% | 101% | 99% |
| Overall | 0.00% | -0.04% | -0.17% | 99% | 100% |
| Class D | -0.01% | 0.78% | 0.26% | 100% | 100% |
| Class F | -0.16% | -0.17% | -0.53% | 99% | 100% |
| Class TGM | -2.04% | -1.65% | -1.60% | 96% | 102% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-7 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.00% | -0.13% | 99% | 99% |
| Class C | 0.09% | -0.39% | 0.15% | 101% | 101% |
| Class E | 0.05% | 0.78% | -0.50% | 101% | 101% |
| Overall | 0.04% | 0.07% | -0.13% | 100% | 100% |
| Class D | -0.06% | 0.05% | -0.06% | 101% | 101% |
| Class F | -0.45% | -0.15% | -0.05% | 99% | 99% |
| Class TGM | -1.63% | -1.28% | -1.28% | 97% | 103% |

# Conclusions

The proposed methods show 1+% and 2+% average luma gain for TGM class with similar or even less encoding time. When CU level OBMC flag is removed, the encoding complexity is further reduced as no block level on/off RD checking is needed for OBMC. Considering the proposed method is quite simple, it is suggested to include the method in ECM-8.

# Patent rights declaration(s)

Google, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).