| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0075-v2 |
| --- | --- |

| Title: | EE2-1.14c combination test of EE2-1.14a and EE2-1.14b |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanzhuo Ma, Junyan Huo, Hongqing Du, Hongli Zhang, Zhenyao Zhang, Wenhan Qiao, Fuzheng Yang (Xidian University) Fan Wang, Lai Zhang, Yue Yu, Haoping Yu, Dong Wang (OPPO) | Tel: Email: | yzma@mail.xidian.edu.cn wangfan6@oppo.com |
| Source: | Xidian University, OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-1.14c. EE2-1.14c is a combination test of EE2-1.14a: Extended the adjacent search area of IntraTMP and EE2-1.14b: Enlarge the search range for small blocks.

On top of ECM-8.0, simulation results are reported as follows:

EE2-1.14c

AI {-0.18% Y, -0.17% U, -0.21% V, 99% EncT, 101% DecT }

RA {-0.07% Y, -0.11% U, 0.05% V, 100% EncT, 100% DecT }

Introduction

In EE2-1.14a [1], Extended the adjacent search area of IntraTMP is investigated.

In EE2-1.14b [2], Enlarging the search range for small blocks is investigated.

Simulation results

EE2-1.14c was implemented on top of the ECM-8.0 [3] software and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AC2024 [4]). The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | -0.02% | 98% | 100% |
| Class A2 | -0.28% | -0.34% | -0.36% | 98% | 101% |
| Class B | -0.21% | -0.21% | -0.20% | 100% | 102% |
| Class C | -0.08% | -0.02% | -0.17% | 100% | 101% |
| Class E | -0.29% | -0.29% | -0.31% | 100% | 102% |
| Overall | -0.18% | -0.17% | -0.21% | 99% | 101% |
| Class D | -0.26% | -0.66% | -0.37% | 100% | 101% |
| Class F | 0.01% | -0.10% | 0.24% | 100% | 100% |
| Class TGM |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.04% | 100% | 99% |
| Class A2 | -0.15% | -0.25% | 0.01% | 100% | 100% |
| Class B | -0.09% | -0.10% | 0.06% | 101% | 101% |
| Class C | -0.04% | -0.14% | 0.06% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.11% | 0.05% | 100% | 100% |
| Class D | -0.05% | 0.36% | -0.47% | 100% | 100% |
| Class F | -0.01% | -0.41% | -0.33% | 99% | 99% |
| Class TGM |  |  |  |  |  |

Conclusions

Comparing EE2-1.14a, EE2-1.14b and EE2-1.14c, it is observed that the gain from EE2-1.14a and EE2-1.14b is additive.

Reference

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, “EE2-1.14b Enlarge search range for small blocks in IntraTMP”, JVET-AD0074, April 2023

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AC2024, January 2023

Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).