| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0131-v1 |
| --- | --- |

| Title: | AhG7: Coding efficiency/runtime tradeoff metrics |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrice Onno Guillaume Laroche | Tel: Email: | patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

The purpose of this contribution is to introduce a metrics that can be used to assess the tradeoff between the coding performance and the runtime by computing the corresponding slope angle between the BDR measure and the encoder/decoder runtime. The main goal is to introduce this metrics in the Excel sheet of the ECM results provided in the ECM Common Test Conditions to rapidly obtain a measure representative of the performance of one given technical proposal. This performance metrics could be useful to compare proposals in particular in the scope of the EE2 or of the AhG 7. It would indeed enable to rank rapidly the different proposals in addition to the traditional encoding/decoding runtime and BDR measure. This metrics could be also useful in order to set a minimum value to further consider a technology in an Exploration Experiment.

# Introduction

When assessing different proposals in the Exploration Experiment it is often difficult to precisely compare them each other. In addition, in the scope of the AhG 7 where tools off test are performed, a simple metrics to classify them in a relevant order based on the runtime and the BDR performance could be useful.

This contribution is reusing and extending the metrics principle already introduced by HHI in JVET-B0044 [1] when different tools were assessed for evaluating the different technologies in order to select the most promising technologies to be included into the first VTM.

# Proposal

This contribution proposes to simply use the slope between the reported BDR and runtime. This slope is expressed in degrees.

When considering a technology bringing a BDR gain G= -1% for an encoding runtime R=110%, the metrics will simply compute the following angle value A based on the slope between the encoding runtime and the BDR gain (as represented in the CTC Excel file) corresponding to the following equation:

A=ArtanG*100 R-1*10

# Examples

The plot in Figure 1 below shows examples of the Table 1 listing eight 2-uplets of BDR gain and associated runtime.

As often mentioned during the development of video coding standard related to conventional tools, a -1% BDR gain for 110% encoding time is considered as a pretty good tradeoff in terms of coding efficiency versus the runtime. In the Table 1 below we can rapidly identify that the G5-G8 2-uplets are quite good tradeoffs (tradeoff angle ≥ 45 degrees) while the 2-uplets G1-G2 are quite bad tradeoff (tradeoff angle < 30 degrees).

Table 1: Example of BDR values and associated metric highlighted in blue

With the proposed metrics, the higher the value of the tradeoff angle, the better the tradeoff between coding efficiency and runtime.

Figure 1: Plot of the runtime / BDR values provided in Table 1. Point above the line in blue are of better tradeoffs.

## Extension to decoder and YUV measure

The above examples are provided for some encoder samples but the same calculation can be used for the decoder.

In addition, the above examples are provided for the Y measure but the BDR could be computed based on the weighted BDR of the Y, U and V components as it was performed sometimes in the past in the development of HEVC by setting a weight of 10 on the BDR for luma according to the following relation:

YUV BDR = 10 (Y BDR + U BDR + V BDR) / 12

# Extension to the other BDR/runtime quadrants

Similar approach can be computed for the other quadrants where the run time is less that 100% and when the BDR value is positive.  In a similar way, the best compromise of each quadrant is obtained when the tradeoff angle is maximized. Depending on the quadrants, the inverse BDR or the inverse runtime is used.

## Runtime less than 100% - negative BDR gain: Towards Higher Gain (THG)

This quadrant corresponds to the case where a decrease of the runtime is performed by simplifying the implementation while still providing some gain. This case is an ideal situation.

## Runtime less than 100% - positive BDR gain: Towards Lower Runtime (TLR)

This quadrant corresponds to the case where a decrease of the runtime is reached by for example simplifying the encoder or decoder implementation. This may however lead to some coding efficiency losses and the tradeoff angle metrics can also asses the compromise between the losses and the run time in such case.

## Runtime more than 100% - positive BDR gain: Bad Tradeoff (BT)

This quadrant corresponds to a pretty bad compromise, but this situation may happen if a technology should be replaced by a less performing one introducing some losses and requiring more runtime.

The attached Excel file includes the metrics implemented through two small VBA functions to distinct the 4 quadrants and compute the metrics accordingly. Type “Alt+F11” in the Excel file to access the VBA source code.

## Illustration

The following Figure 2 is a copy paste of the provided Excel file. It contains several 2-uplets of BDR and runtime according to the different quadrants and the tradeoff angle corresponding to the proposed metrics.

Points in the green area are targeting higher coding efficiency gain and the metrics value can range from 0 to 180 degrees.

In the yellow area, the metrics can range from 0 to 90 degrees. When introducing a minimum loss while maximizing the runtime decrease, the angle metric tends to increase towards the maximum value of 90 degrees.

In a similar way, in the red area, the metrics can again range from 0 to 90 degrees. When introducing a minimum loss while minimizing the runtime increase, the angle metric tends to increase towards the maximum value of 90 degrees.

Figure 2: Examples of BDR and runtime uplets with different tradeoffs.

# Conclusion

This contribution proposes to use a simple metrics based on the slope between the runtime and the BDR gain to rapidly assess any technical proposal without introducing any new concept.

We suggest to add this metrics (adapted for the 4 quadrants) in the Excel result file associated to the CTC conditions to get one single value representative of the performance of the proposed technologies based on the BDR/ runtime figures provided by any proponent or any tool off tests.

# References

- JVET-B0044 - Coding Efficiency / Complexity Analysis of JEM 1.0 coding tools for the Random Access Configuration. H. Schwarz, C. Rudat, M. Siekmann, B. Bross, D. Marpe, T. Wiegand (Fraunhofer HHI)

# Patent rights declaration(s)

Canon Research Centre France does not have any current or pending patent rights relating to the technology described in this contribution.