| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0133-v3 |
| --- | --- |

| Title: | AHG10: Lambda-QP Relationship Fix for Slice-level Multi-QP Optimization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junqi Liao Li Li Dong Liu Houqiang Li Feng Wu | Email: | liaojq@mail.ustc.edu.cn |
| Source: | University of Science and Technology of China |  |  |

_____________________________

- Abstract

This contribution proposes to fix the problems of slice-level multi-QP optimization in VTM software. The main modifications this contribution proposes to the slice-level multi-QP optimization of the original version in VTM are as follows:

- This contribution proposes to modify the QP-lambda relationship in slice-level multi-QP RDO.

- This contribution proposes to fix the slice-level lambda while traversing the QP within a predefined range.

- It is proposed in this contribution to disable CU-level mode reuse when multi-QP optimization is enabled.

The test results show that the fix brings significant performance improvement compared with the original implementation as follows:

AI: {-8.25%, -5.68%, -6.00%}; RA: {xx%, xx%, xx%}; LDB: {-5.28%, -3.96%, -3.34%}; LDP: {-4.65%, -3.21%, -2.74%}

Introduction

Slice-level multi-QP optimization is an algorithm that traverses different slice-level QPs, and then selects the best one by comparing the RD cost of different QPs. A fixed slice-level lambda should be used when calculating the RD-cost in the process of multi-QP RDO.

There are some problems in the current VTM implementation of slice-level multi-QP optimization. This contribution proposes to fix these problems.

Proposal

Fix the QP-lambda relationship bug in slice-level multi-QP RDO

The QP-lambda relationship used in the original multi-QP optimization implementation is proposed in JVT-AE010 [1], and it is inconsistent with the current QP-lambda relationship. Therefore, we modify this calculation process as follows to align to align the correct QP-lambda relationship used by other parts of the current VTM.

Table 1: proposed source code modification to correct the QP-lambda relationship in slice-level multi-QP RDO

#if FIX_MULTIQP dFrameLambda = m_vdRdPicLambda[0]; #else int SHIFT_QP = 12 + 6 * (pcSlice->getSPS()->getBitDepth(CHANNEL_TYPE_LUMA) – 8 -DISTORTION_PRECISION_ADJUSTMENT(pcSlice->getSPS()->getBitDepth(CHANNEL_TYPE_LUMA))); // set frame lambda if (m_pcCfg->getGOPSize() > 1) { dFrameLambda = 0.68 * pow (2, (m_viRdPicQp[0] - SHIFT_QP) / 3.0) * (pcSlice->isInterB()? 2 : 1); } else { dFrameLambda = 0.68 * pow (2, (m_viRdPicQp[0] - SHIFT_QP) / 3.0); } #endif // FIX_MULTIQP

m_vdRdPicLambda[0] is the corresponding lambda of the slice-level QP calculated by calling calculateLambda() in initEncSlice(), which is consistent with the current VTM implementation.

Fix the setting of slice-level lambda in multi-QP optimization

In the original slice-level multi-QP optimization implementation in VTM, the lambda is also changed when traversing different QPs in the multi-QP RDO process. As we are performing RDO under a slice-level lambda, the lambda should not be changed. Therefore, we propose to fix the slice-level lambda, and only traverse different QPs to obtain the best QP. The modifications are as follows.

Table 2: proposed source code modification to fix the setting of slice-level lambda in multi-QP optimization

for ( uint32_t uiQpIdx = 0; uiQpIdx < 2 * m_pcCfg->getDeltaQpRD() + 1; uiQpIdx++ ) { pcSlice ->setSliceQp ( m_viRdPicQp [uiQpIdx] ); #if FIX_MULTIQP setUpLambda(pcSlice, m_vdRdPicLambda[0], m_viRdPicQp [0]); #else setUpLambda(pcSlice, m_vdRdPicLambda[uiQpIdx], m_viRdPicQp [uiQpIdx]); #endif // FIX_MULTIQP // try compress compressSlice ( pcPic, true, m_pcCfg->getFastDeltaQp()); uint64_t uiPicDist = m_uiPicDist; // Distortion, as calculated by compressSlice. // NOTE: This distortion is the chroma-weighted SSE distortion for the slice. // Previously a standard SSE distortion was calculated (for the entire frame). // Which is correct? #if W0038_DB_OPT // TODO: Update loop filter, SAO and distortion calculation to work on one slice only. // uiPicDist = m_pcGOPEncoder->preLoopFilterPicAndCalcDist( pcPic ); #endif // compute RD cost and choose the best double dPicRdCost = double(uiPicDist) + dFrameLambda * double(m_uiPicTotalBits); if ( dPicRdCost < dPicRdCostBest ) { uiQpIdxBest = uiQpIdx; dPicRdCostBest = dPicRdCost; } } // set best values pcSlice ->setSliceQp ( m_viRdPicQp [uiQpIdxBest] ); #if FIX_MULTIQP setUpLambda(pcSlice, m_vdRdPicLambda[0], m_viRdPicQp [0]); #else setUpLambda(pcSlice, m_vdRdPicLambda[uiQpIdxBest], m_viRdPicQp [uiQpIdxBest]); #endif // FIX_MULTIQP

Disable CU-level mode reuse when slice-level multi-QP optimization is enabled

We also observe that there is a conflict between slice-level multi-QP optimization and the macro definition REUSE_CU_RESULTS.

When REUSE_CU_RESULTS is enabled, the cache saving previous encoding results will affect the mode selection of the current block. However, when the slice-level multi-QP optimization is also enabled, the encoding results under the previous QPs will be retained in the cache. Therefore, when both slice-level multi-QP optimization and REUSE_CU_RESULTS are enabled, the coding results under previous QPs saved in the cache may influence the mode selection of the current block, leading to a crash. We propose to simply disable REUSE_CU_RESULTS when the slice-level multi-QP is enabled.

Table 3: proposed source code modification to disable CU-level mode reuse when slice-level multi-QP optimization is enabled

#if FIX_MULTIQP #define REUSE_CU_RESULTS 0 #else #define REUSE_CU_RESULTS 1 #endif // FIX_MULTIQP

Experimental results

The VTM software modified as proposed in this contribution has been tested under the JVET common test conditions [2] in VTM-19.2. It can be seen the coding results of the modified implementation brings significant performance improvements of about 8.25% (All Intra), x.xx% (Random Access), 5.28% (Low delay B) and 4.65% (Low delay P) in Y-BDBR compared to the original multi-QP optimization.

The value of DeltaQpRD is set to 5 when slice-level multi-QP optimization is on in the experiments.

Table 4: Performance of fixed slice-level multi-QP optimization compared with original implementation (All Intra)

|  | Over VTM19.2 multiQP_anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.18% | 2.85% | -1.59% | 100% | 99% |
| Class A2 | -9.38% | -4.13% | -4.38% | 100% | 101% |
| Class B | -8.31% | -7.76% | -6.98% | 100% | 100% |
| Class C | -7.20% | -9.09% | -8.54% | 99% | 101% |
| Class E | -7.51% | -7.77% | -7.03% | 100% | 101% |
| Overall | -8.25% | -5.68% | -6.00% | 100% | 100% |
| Class D | -7.61% | -8.35% | -8.60% | 100% | 101% |
| Class F (optional) |  |  |  |  |  |

Table 5: Performance of fixed slice-level multi-QP optimization compared with original implementation (Random Access)

|  | Over VTM19.2 multiQP_anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -11.15% | -6.58% | -6.58% | 100% | 100% |
| Class C | -10.49% | -7.26% | -9.33% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -10.20% | -7.54% | -7.61% | 99% | 101% |
| Class F (optional) |  |  |  |  |  |

Table 6: Performance of fixed slice-level multi-QP optimization compared with original implementation (Low delay B)

|  | Over VTM19.2 multiQP_anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.22% | -6.97% | -6.04% | 100% | 100% |
| Class C | -3.48% | -5.86% | -7.32% | 100% | 100% |
| Class E | -2.78% | 3.61% | 6.49% | 100% | 100% |
| Overall | -5.28% | -3.96% | -3.34% | 100% | 100% |
| Class D | -4.81% | -3.40% | -4.08% | 100% | 101% |
| Class F (optional) |  |  |  |  |  |

Table 7: Performance of fixed slice-level multi-QP optimization compared with original implementation (Low delay P)

|  | Over VTM19.2 multiQP_anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.91% | -6.97% | -6.20% | 100% | 100% |
| Class C | -2.51% | -4.50% | -6.37% | 101% | 100% |
| Class E | -2.09% | 4.79% | 7.87% | 100% | 100% |
| Overall | -4.65% | -3.21% | -2.74% | 100% | 100% |
| Class D | -4.42% | -2.43% | -3.47% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |

References

[1]	Tourapis A M, Leontaris A, Suhring K, et al. 264/14496-10 AVC Reference Software Manual; Doc[J]. JVT-AE010, 2009.

[2] Boyce J, Suehring K, Li X, et al. JVET-J1010: JVET common test conditions and software reference configurations[J]. JVET, San Diego, CA, USA, Tech. Rep. JVET-J1010, 2018.

Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).