| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0168-v1 |
| --- | --- |

| Title: | EE1-1.4 Channel redistribution for luma and chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Per Wennersten, Jacob Ström, Du Liu | Tel: Email: | [phone contact number] {per.wennersten, jacob.strom, du.liu} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the EE1-1.4 test results. It investigates modifying the number of channels in the head of the NN loop filter to reduce the complexity. Three configurations for the luma network and two configurations for the chroma network are tested.

The configuration for the luma network preferred by the proponents (Test 1) has 192 input channels for rec, 24 for pred, 12 for bs, qp, and IPB, and 48 for the output channels of the fuse block. It is reported that this model has RA Y/U/V results of +0.01/-0.01/+0.11 % and AI Y/U/V results of +0.08/-0.2/+0.03 % compared to NNVC-4.0 filter set #1, while reducing the number kMACs/pixel by 11% and the number of parameters by 5%.

Used together with the preferred tested luma model, the preferred chroma model (Test 4) has 96 input channels for luma, 192 for rec, 24 for pred, 12 for qp and bs, and 48 for the output channels of the fuse block. The reported performance for RA Y/U/V is ?/?/? % and for AI +0.08/+0.09/+0.82 %, compared to NNVC-4.0 filter set #1. It is further reported that the total kMACs/pixel for the filter with both luma and chroma models is then 603 kMacs/pixel (a reduction by 11%) and the number of parameters 2953k (a reduction by 5%).

# Introduction

JVET-AC0089 (JVET-AB-EE1-1.5.3) [1] describes NN loop filtering using only two models; one model for luma (for both intra and inter) using an additional input called IPB, and one model for chroma (for both intra and inter) without any new inputs. This was adopted at the last meeting, and the current NNVC-4.0 filter set #1 [2], hence uses five inputs (rec, pred, bs, qp, IPB) for the luma network. Each input is associated with a 3x3 convolutional layer and a PReLU. The overall structure of the NN head is illustrated in Figure 1. All the inputs have an equal number of channels, i.e., d1=d2=d3=d4=96 channels. After concatenating the five sets of 96 channels, there is a fuse block consisting of a 1x1 convolutional layer and a PReLU. The 1x1 convolutional layer has an input of 96×5=480 channels and an output of 96 channels.

JVET-AC0126 [3] assumes that some inputs are more important for the end result than others. As an example, the reconstruction input (‘rec’) is the signal that should be refined, whereas the other inputs are signals that are supposed to help with the refining. It is therefore hypothesised that one can get better results by increasing the number of channels for the reconstruction input while reducing the number of channels for the other inputs. Another hypothesis is that the remaining inputs are also not equally important. It is assumed that the prediction input carries more information than BS and IPB, since the latter two can only take a finite number of values, and hence the number of channels to the prediction input is reduced less than the other inputs.

JVET-AC0126 therefore proposes to increase the number of channels for the reconstruction input rec and reduce the number of channels for pred, bs, qp, and IPB. It also proposes to reduce the number of output channels in the fuse block.

This proposal investigates different versions of the idea presented in JVET-AC0126, as well as extending the idea to the chroma model.

# Proposed method

## Structure of the model

The structure of the proposed luma model is depicted in Figures 1, 2, and 3. Figure 1 shows the five inputs with different number of channels. The five inputs are the reconstructed luma samples (rec), the prediction luma samples (pred), the block boundary strength (bs), QP, and the block type (IPB). In the 3x3 convolutional layer connected to the inputs, the numbers of output channels are called d1, d2 and d3. The number of the output channel of the fuse block is d4.

Figure 2 shows the k-th residual block. Figure 3 shows the last part of the network. These two are the same as in NNVC-4.0 filter set #1.

Figure 1. Head of luma network. The inputs are combined to form the input y to the next part of the network.

Figure 2. The k-th residual block (k=0..7). The output y of the head is fed into a first residual block with input z0=y, which also takes the inputs rec, pred, part, bs, qp, and IPB. The output z1 is then fed into another such residual block.

Figure 3. The output of the last residual block is fed into this last part of the network.

The chroma model also has five inputs, like the luma model. It lacks the IPB input, but has the luma reconstruction (luma) instead. Because luma is potentially a quite important input to the chroma network, we assign a relatively high number of channels for it. The head of the chroma network is shown in Figure 4.

Figure 4. Head of chroma network.

## Test configurations

This document tests three different configurations for the luma network, with different values for d1, d2, d3 and d4 from Figure 1. Table 1 summarizes the three configurations along with the resulting complexity and filter set #1 numbers. kMACs/pixel is reported using per frame numbers.

The complexity and the number of parameters were measured using the thop library [4].

Table 1. Summary of the three luma tests included in this document.

|  | d1 | d2 | d3 | d4 | kmacs/pixel | #parameters |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 192 | 24 | 12 | 48 | 480 | 1471k |
| Test 2 | 192 | 16 | 8 | 48 | 478 | 1470k |
| Test 3 | 128 | 16 | 8 | 64 | 481 | 1483k |
| Filter set #1 | 96 | 96 | 96 | 96 | 538 | 1549k |

Test 1 is identical to the model in AC0126 but tested in NNVC 4.0. All three tests use the chroma model from filter set #1 without any redistribution of channels.

In addition, two different configurations for the chroma network are tested on top of Test 1 (the luma model from AC0126), with different values for d1, d2, d3, d4 and d5 from Figure 4. Table 2 summarizes these two configurations, reporting numbers only for the chroma model.

Table 2. Summary of the two chroma tests included in this document.

|  | d1 | d2 | d3 | d4 | d5 | kmacs/pixel | #parameters |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test 4 | 96 | 192 | 24 | 12 | 48 | 123 | 1482k |
| Test 5 | 64 | 128 | 16 | 8 | 64 | 123 | 1492k |
| Filter set #1 | 96 | 96 | 96 | 96 | 96 | 135 | 1555k |

## Training

| Table 3. Network Information for NN-based video coding tool testing in training stage. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-PCIE-40GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 340 for luma, 400 for chroma |
|  | Batch size: | 32 |
|  | Loss function: | Weighted L1 and L2 |
|  | Training time: | 10 days (Luma tests) 8 days (Chroma tests) |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, JVET-W-EE1-1.6, qp {17, 22, 27, 32, 37, 42} |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate: | 1e-4 and 1e-5 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## Inference

| Table 4. Network Information for NN-based video coding tool testing in inference stage. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL [3] |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 2953k(=1471k+1482k) |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 5.9MB |
|  | Multiplay Accumulate (MAC) | 603(=480+123) |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Results

The following tables show the results of the three tests outlined in section 2 compared to NNVC 4.0 and NNVC 4.0 filter set #1. The chroma models in Test 4 and Test 5 are tested together with Test 1 (i.e., the luma model from AC0126).

All the models are converted to integer SADL models.

Results are currently incomplete due to a crash in Test 5 RA and a problem starting Test 4.

## Luma model results compared to filter set 1

These tables describe test performance compared to NNVC 4.0 with filter set #1 models.

Test 1 (encoder and decoder runtimes are not reliable):

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 filter set 1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0,09% | 0,02% | 0,15% | 115% | 103% |
| Class A2 | -0,02% | -0,03% | 0,01% | 113% | 108% |
| Class B | 0,01% | 0,09% | 0,17% | 114% | 105% |
| Class C | 0,10% | -0,12% | 0,07% | 117% | 113% |
| Class E |  |  |  |  |  |
| Overall | 0,01% | -0,01% | 0,11% | 115% | 107% |
| Class D | 0,19% | -0,16% | -0,03% | 119% | 119% |
| Class F | 0,16% | 0,07% | 0,04% | 120% | 100% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0 filter set 1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0,02% | -0,09% | -0,06% | 121% | 114% |
| Class A2 | 0,07% | 0,24% | 0,50% | 119% | 117% |
| Class B | 0,08% | -0,77% | -0,10% | 118% | 121% |
| Class C | 0,11% | -0,05% | -0,08% | 115% | 120% |
| Class E | 0,14% | 0,03% | 0,02% | 116% | 118% |
| Overall | 0,08% | -0,20% | 0,03% | 118% | 118% |
| Class D | 0,07% | 0,08% | 0,12% | 118% | 116% |
| Class F | 0,16% | 0,01% | 0,01% | 123% | 110% |

Test 2:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 filter set 1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0,03% | 0,14% | 0,02% | 110% | 96% |
| Class A2 | 0,04% | 0,01% | -0,11% | 109% | 98% |
| Class B | 0,12% | 0,07% | 0,12% | 115% | 107% |
| Class C | 0,22% | -0,11% | -0,07% | 118% | 119% |
| Class E |  |  |  |  |  |
| Overall | 0,10% | 0,03% | 0,01% | 113% | 106% |
| Class D | 0,31% | -0,25% | -0,01% | 117% | 108% |
| Class F | 0,24% | 0,06% | -0,03% | 117% | 103% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0 filter set 1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0,06% | 0,26% | 0,02% | 118% | 104% |
| Class A2 | 0,07% | -0,12% | -0,20% | 119% | 102% |
| Class B | 0,14% | -1,31% | -0,51% | 122% | 106% |
| Class C | 0,09% | -0,11% | -0,19% | 110% | 103% |
| Class E | 0,15% | 0,05% | 0,02% | 111% | 102% |
| Overall | 0,11% | -0,36% | -0,21% | 116% | 104% |
| Class D | 0,07% | -0,12% | -0,09% | 113% | 108% |
| Class F | 0,19% | 0,00% | 0,00% | 125% | 102% |

Test 3:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 filter set 1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0,04% | -0,10% | 0,01% | 109% | 97% |
| Class A2 | 0,07% | 0,06% | -0,04% | 110% | 100% |
| Class B | 0,09% | 0,02% | 0,11% | 115% | 100% |
| Class C | 0,30% | -0,09% | 0,08% | 118% | 115% |
| Class E |  |  |  |  |  |
| Overall | 0,13% | -0,03% | 0,05% | 114% | 103% |
| Class D | 0,43% | -0,19% | -0,12% | 118% | 107% |
| Class F | 0,20% | 0,03% | 0,02% | 117% | 104% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0 filter set 1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0,01% | 0,44% | 0,14% | 115% | 101% |
| Class A2 | 0,08% | -0,23% | -0,52% | 112% | 99% |
| Class B | 0,04% | 0,32% | 0,23% | 113% | 108% |
| Class C | 0,06% | -0,17% | -0,09% | 110% | 109% |
| Class E | 0,05% | 0,01% | -0,01% | 115% | 118% |
| Overall | 0,05% | 0,09% | -0,02% | 113% | 107% |
| Class D | 0,06% | -0,26% | -0,19% | 115% | 112% |
| Class F | 0,10% | 0,00% | 0,00% | 123% | 111% |

## Chroma model results compared to filter set #1

Test 4:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0,10% | -0,87% | 1,24% | 124% | 114% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,17% | -1,03% | 0,12% | 126% | 117% |
| Class F |  |  |  |  |  |

Test 5:

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |  | EncT |  | DecT |  |
| Class A1 | -0,13% | -1,60% | 0,32% |  | 102% |  | 90% |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0,01% | 0,38% | 0,37% |  | 105% |  | 98% |  |
| Class C | 0,12% | -0,01% | 1,56% |  | 103% |  | 97% |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D | 0,23% | 0,76% | 0,59% |  | 105% |  | 97% |  |
| Class F | 0,19% | 0,64% | 0,46% |  | 105% |  | 89% |  |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | EncT |  | DecT |
| Class A1 | 0,01% | -0,05% |  | 0,14% |  | 108% |  | 93% |
| Class A2 | 0,07% | 0,80% |  | 0,77% |  | 105% |  | 98% |
| Class B | 0,08% | -0,57% |  | 0,49% |  | 106% |  | 94% |
| Class C | 0,11% | 0,57% |  | 1,28% |  | 102% |  | 100% |
| Class E | 0,14% | 0,44% |  | 0,23% |  | 105% |  | 101% |
| Overall | 0,08% | 0,17% |  | 0,61% |  | 105% |  | 97% |
| Class D | 0,07% | 1,10% |  | 1,13% |  | 105% |  | 95% |
| Class F | 0,16% | 0,62% |  | 0,58% |  | 112% |  | 94% |

## Luma model results compared to NNVC 4.0

These tables describe test performance compared to NNVC 4.0 with no NN loop filter active.

Test 1:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8,81% | -16,46% | -20,55% | 212% | 35118% |
| Class A2 | -9,91% | -20,17% | -16,06% | 207% | 34564% |
| Class B | -9,10% | -22,17% | -22,80% | 207% | 33491% |
| Class C | -10,58% | -23,29% | -24,30% | 182% | 30410% |
| Class E |  |  |  |  |  |
| Overall | -9,60% | -20,92% | -21,40% | 201% | 33160% |
| Class D | -12,59% | -24,09% | -27,25% | 182% | 32845% |
| Class F | -5,34% | -14,26% | -12,47% | 280% | 13636% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6,53% | -16,49% | -18,10% | 215% | 26755% |
| Class A2 | -6,64% | -18,88% | -14,54% | 176% | 22610% |
| Class B | -6,62% | -18,38% | -21,37% | 173% | 22299% |
| Class C | -7,33% | -18,96% | -21,81% | 152% | 14274% |
| Class E | -9,89% | -19,80% | -22,16% | 174% | 22475% |
| Overall | -7,31% | -18,51% | -19,92% | 175% | 20893% |
| Class D | -7,24% | -18,76% | -22,59% | 151% | 14003% |
| Class F | -5,03% | -15,16% | -13,88% | 152% | 16282% |

Test 2:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8,76% | -16,35% | -20,66% | 203% | 32593% |
| Class A2 | -9,85% | -20,15% | -16,16% | 199% | 31427% |
| Class B | -8,99% | -22,18% | -22,84% | 208% | 34141% |
| Class C | -10,47% | -23,28% | -24,40% | 184% | 32010% |
| Class E |  |  |  |  |  |
| Overall | -9,51% | -20,90% | -21,48% | 199% | 32703% |
| Class D | -12,49% | -24,18% | -27,25% | 178% | 29942% |
| Class F | -5,27% | -14,26% | -12,52% | 275% | 13975% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6,49% | -16,27% | -18,08% | 209% | 24363% |
| Class A2 | -6,64% | -19,16% | -15,15% | 177% | 19802% |
| Class B | -6,56% | -18,82% | -21,66% | 180% | 19429% |
| Class C | -7,35% | -19,00% | -21,90% | 145% | 12204% |
| Class E | -9,88% | -19,78% | -22,16% | 166% | 19527% |
| Overall | -7,29% | -18,65% | -20,12% | 173% | 18268% |
| Class D | -7,24% | -18,93% | -22,76% | 145% | 13074% |
| Class F | -4,99% | -15,17% | -13,88% | 155% | 15002% |

Test 3:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -8,69% | -16,55% | -20,68% | 202% | 32946% |
| Class A2 | -9,82% | -20,12% | -16,11% | 201% | 31930% |
| Class B | -9,02% | -22,23% | -22,85% | 209% | 31966% |
| Class C | -10,40% | -23,27% | -24,27% | 184% | 31094% |
| Class E |  |  |  |  |  |
| Overall | -9,48% | -20,95% | -21,45% | 199% | 31916% |
| Class D | -12,38% | -24,13% | -27,34% | 180% | 29548% |
| Class F | -5,30% | -14,27% | -12,49% | 273% | 14088% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6,54% | -16,16% | -17,98% | 204% | 23612% |
| Class A2 | -6,63% | -19,24% | -15,43% | 167% | 19268% |
| Class B | -6,66% | -17,40% | -21,09% | 166% | 19935% |
| Class C | -7,37% | -19,07% | -21,83% | 145% | 12881% |
| Class E | -9,96% | -19,81% | -22,18% | 172% | 22585% |
| Overall | -7,34% | -18,27% | -19,98% | 168% | 18893% |
| Class D | -7,25% | -19,06% | -22,85% | 147% | 13494% |
| Class F | -5,08% | -15,18% | -13,89% | 153% | 16350% |

## Chroma model results compared to NNVC 4.0

Test 4:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -10,57% | -23,82% | -23,37% | 194% | 30697% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -12,60% | -24,73% | -27,03% | 192% | 32353% |
| Class F |  |  |  |  |  |

Test 5:

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | EncT |  | DecT |
| Class A1 | -8,85% |  | -17,65% |  | -20,42% |  | 189% |  | 30682% |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -9,09% |  | -21,95% |  | -22,61% |  | 191% |  | 31377% |
| Class C | -10,56% |  | -23,19% |  | -23,16% |  | 161% |  | 26259% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -12,55% |  | -23,44% |  | -26,74% |  | 161% |  | 26780% |
| Class F | -5,31% |  | -13,80% |  | -12,11% |  | 245% |  | 12088% |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | EncT |  | DecT |  |
| Class A1 | -6,54% | -16,50% |  | -17,97% |  | 192% |  | 21872% |  |
| Class A2 | -6,64% | -18,50% |  | -14,32% |  | 157% |  | 18919% |  |
| Class B | -6,62% | -18,20% |  | -20,94% |  | 156% |  | 17295% |  |
| Class C | -7,33% | -18,49% |  | -20,84% |  | 135% |  | 11837% |  |
| Class E | -9,89% | -19,47% |  | -21,95% |  | 156% |  | 19343% |  |
| Overall | -7,31% | -18,24% |  | -19,49% |  | 156% |  | 17097% |  |
| Class D | -7,24% | -17,98% |  | -21,82% |  | 135% |  | 11430% |  |
| Class F | -5,03% | -14,64% |  | -13,43% |  | 139% |  | 13908% |  |

# Conclusions

Test 1 has the best compression results of the three luma tests. Because of this, we recommended to use the Test 1 model for luma and Test 4 model for chroma. We therefore propose that a training crosscheck with these two models are included in the next EE.

# References

[1] EE1-1.5: Combined intra and inter models for luma and chroma, JVET-AC0089, D. Liu, J. Ström, M. Damghanian, P. Wennersten, K. Andersson, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 29th Meeting, by teleconference, 11–20 January 2023

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-4.0

[3] EE1-related: Reduced complexity through channel redistribution in NN head, JVET-AC0126, P. Wennersten, J. Ström, D. Liu, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 29th Meeting, by teleconference, 11–20 January 2023

[4] https://pypi.org/project/thop/

[5] AhG14 SADL v4 changes, JVET-AC0057, F. Galpin, T. Dumas, P. Bordes, E. François, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 29th Meeting, by teleconference, 11–20 January 2023

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).