| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0187-v1 |
| --- | --- |

| Title: | Non-EE2: On large NSPT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Jie Zhao Seung-Hwan Kim 10680 Treena St. St# 550 San Diego, CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jaehyun.lim@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the 29th JVET meeting, Non-Separable Primary Transform (NSPT) was incorporated into ECM-8.0, where the NSPT is applied to 4x4, 4x8/8x4, 4x16/16x4, 8x8, and 8x16/16x8 transform blocks instead of the combination of DCT-2 and LFNST. This contribution introduces additional large NSPT for Nx32/32xN (N = 4 and 8) blocks of which matrix dimensions are 36x128 (N = 4) and 48x256 (N = 8), respectively. The number of transform sets and kernels per set are not changed for the proposed large NSPT. The BD-rate changes and encoding/decoding times for AI configuration are -0.09%/-0.11%/-0.11% for Y/U/V and 102%/100%, respectively.

Proposed architecture

In ECM-8.0, NSPT instead of the separable DCT-2 plus LFNST combination is applied to 4x4, 4x8/8x4, 4x16/16x4, 8x8, and 8x16/16x8 transform blocks [2]. In this contribution, the additional large NSPT for Nx32/32xN (N = 4 and 8) blocks is tested on top of the existing NSPT whose kernels are unchanged. As in ECM-8.0, the combination of DCT-2 and LFNST for the Nx32/32xN blocks is replaced by the newly added large NSPT. The NSPT matrix dimensions for 4x32/32x4 and 8x32/32x8 blocks are 36x128 and 48x256, respectively, and coefficient positions other than first 36 and 48 coefficients in scan order are zeroed-out. An NSPT index is not signalled when non-zero coefficients are found in the zeroed-out positions, of which manner is identical to the existing NSPT index parsing. The numbers of transform sets and kernels per set for the large NSPT are configured as 35 and 3, respectively, which are also the same as the existing NSPTs including mapping from intra prediction mode to the transform sets.

Experimental results

The proposed architectures have been implemented based on ECM-8.0 reference software according to common test conditions defined in JVET-Y2017 [1].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.02% | -0.12% | 102% | 101% |
| Class A2 | -0.12% | -0.17% | -0.10% | 102% | 100% |
| Class B | -0.10% | -0.15% | -0.15% | 102% | 101% |
| Class C | -0.05% | -0.06% | -0.05% | 101% | 100% |
| Class E | -0.12% | -0.12% | -0.15% | 101% | 101% |
| Overall | -0.09% | -0.11% | -0.11% | 102% | 100% |
| Class D | -0.03% | -0.30% | -0.26% | 101% | 101% |
| Class F | 0.03% | -0.06% | 0.12% | 101% | 100% |

Conclusion

The large NSPT kernels for Nx32/32xN (N = 4 and 8) are proposed in this contribution, which are applied in the same manner as the existing NSPT in ECM-8.0. The benefit of these large NSPT kernels is observed by 0.09% coding gain with 102%/100% EncT/DecT. It is recommended to further investigate the large NSPT for the Nx32/32xN blocks in the next round of EE2.

References

[1] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

[2] P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm), M. Koo, J. Zhao, J. Lim, S. Kim (LGE), “EE2-4.2: Non-Separable Primary Transform for Intra Coding”, JVET-AC0130, Jan. 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).