| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0192-v2 |
| --- | --- |

| Title: | EE2-1.20f: Combination Test of Test 1.12 + Test 1.13b |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Taoran Lu, Peng Yin, Sean McCarthy (Dolby) Jeeva Raj Arumugam, Ashwin Natesan, Vaibhav Valvaiker, Abhijeet Sarate, Jay N. Shingala (Ittiam) Xinwei Li, Ruling Liao , Jie Chen (Alibaba) | Tel: Email: | {jeeva.raj, jay.shingala }@ittiam.com {fpu, tlu, pyin}@dolby.com {sid.lxw, ruling.lrl, jiechen.cj}@alibaba-inc.com |
| Source: | Dolby Laboratories, Inc., Ittiam System, Alibaba group |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2 IntraTMP combination Test 1.20f. This is the combination test of Test 1.12 (Intra TMP with sub-pel precision) and Test 1.13b (adaptive intraTMP fusion with up to two predicted blocks and MTMP search) The impact on coding efficiency and runtimes over ECM-8.0 is reportedly {for Y, U, V, EncT an DecT} as follows:

EE2-1.20f: Test 1.12 + Test 1.13b

AI {-0.35% Y, -0.35% U, -0.35% V, 103% EncT, 100% DecT }

RA {-x.xx% Y, -x.xx% U, -x.xx% V, xxx% EncT, xxx% DecT }

# Proposed method

This contribution reports the simulation results of EE2 Test 1.20f which studies the joint combination of Test 1.12 [2] and Test 1.13b [3]. Test 1.12 tests the sub-pel precision applied on IntraTMP. The sub-pel positions around the integer-pel position obtained by template matching are further enabled for IntraTMP. Test 1.13b tests adaptive intraTMP fusion with up to two predicted blocks. The search process is improved using updated template. MTMP search from Test1.10 is added.

The combination is made as follows: when sub-pel precision is used, if updated template is used, sub-pel precision based intraTMP search will be performed with updated template. Otherwise, default 1.12 will be invoked. When sub-pel precision is not used, default 1.13b will be invoked.

The proposed syntax change is as follows:

…

intra_tmp_flag

if(intra_tmp_flag) {

intra_tmp_sub_pel_precision_idx

if(intra_tmp_sub_pel_precision_idx != 0) {

intra_tmp_sub_pel_direction_idx

}

intra_tmp_template_update_flag

}

…

intra_tmp_flag indicates whether the intra prediction type for current block is IntraTMP or not.

intra_tmp_sub_pel_precision_idx specifies the precision index for current block. The range of intra_tmp_sub_pel_precision_idx is 0 to 3, used to indicate integer-pel precision, 1/2-pel precision, 1/4-pel precision and 3/4-pel precision, respectively.

intra_tmp_sub_pel_direction_idx specifies the sub-pel direction index for current block. The range of intra_tmp_sub_pel_phase_idx is 0 to 7.

intra_tmp_template_update_flag indicates whether template updating is applied or not for intra TMP.

# Simulation results

The proposed method is implemented on top of ECM8.0 and evaluated using the common test conditions.

Table 2 Test 1.20f coding performance compared with ECM8.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.02% | -0.05% | 103% | 100% |
| Class A2 | -0.40% | -0.37% | -0.48% | 102% | 102% |
| Class B | -0.35% | -0.41% | -0.35% | 103% | 100% |
| Class C | -0.15% | -0.18% | -0.21% | 103% | 99% |
| Class E | -0.87% | -0.86% | -0.72% | 103% | 101% |
| Overall | -0.35% | -0.35% | -0.35% | 103% | 100% |
| Class D | -0.09% | -0.22% | -0.07% | 103% | 98% |
| Class F | -0.10% | -0.14% | -0.22% | 102% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusion

This contribution reports the test results of EE2 IntraTMP combination: test 1.20f.

# References

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AC2024, April 2023.

- X. Li, R.-L. Liao, J. Chen, Y. Ye, “EE2-1.12: Intra TMP with sub-pel precision”, JVET-AD0125, April 2023.

- J. R. Arumugam, A. Natesan, V. Valvaiker, A. Sarate, J. N. Shingala, F. Pu, T. Lu, P. Yin, T. Shao, A. Arora, S. McCarthy, “EE2-1.13: Test on Fusion of Intra Template Matching”, JVET-AD0158, April 2023.

# Acknowledgement

We would like to thank OPPO for helping the crosschecks.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).