| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0362-v2 |
| --- | --- |

| Title: | BoG report on NNPF and SEI processing order SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Miska M. Hannuksela Sachin Deshpande (BoG coordinators) | Email: | yekui.wang@bytedance.com miska.hannuksela@nokia.com sdeshpande@sharplabs.com |
| Source: | BoG on NNPF and SEI processing order SEI messages |  |  |

_____________________________

# Abstract

This document contains the report of the BoG on neural-network post-processing filter (NNPF) and SEI processing order SEI messages. Following the summary provided in JVET-AD0208, the BoG met on

- Friday 21 April 2023 at 1400-1900, discussing proposal items in the proposal categories 1, 2, and 4.

- Saturday 22 April 2023 at 0900-1300 and 1400-200, discussing proposal items in the proposal categories 5 to 9, 11 to 14, and 16.

# Recommendations

The BoG recommended the following actions:

- Action on item 1)a.ii: Plan, in v4 (of VSEI and VVC), to update the SEI processing SEI message to include NNPFs, without clearly specifying the behavior of multiple NNPFs activated for the same picture in v3 except for the special case of spatial resolution upsampling followed by picture rate upsampling.

- Action on item 2)b.ii: To allow input pictures to an NNPF activated for any current picture to come from an earlier CLVS than the CLVS containing the current picture, and for the derivation of which pictures are used as in the input pictures, instead of using picture order count values, plain language phrases like "the previous picture in output order" and "the n-th previous picture in output order" are used.

- Action on item 2)b.i.1: Add an indication nnpfc_absent_input_pic_zero_flag in the NNPFC SEI message that indicates how pictures that would not originate from the bitstream are expected to be replaced in the input tensor. nnpfc_absent_input_pic_zero_flag equal to 1 indicates that the NNPF expects an input picture that is not present in the bitstream to be represented sample arrays with sample values equal to 0. nnpfc_absent_input_pic_flag equal to 1 indicates that the NNPF expects an input picture that is not present in the bitstream to be represented by the closest input picture in output order within the bitstream.

- Action on item 2)b.i.2: Add a constraint that NNPF shall not be activated so that it creates an interpolated picture before the first picture in the bitstream in output order.

- Action on item 2)d: At the end of a bitstream, repeated inference of NNPF for creating interpolated pictures: When an NNPF is active until the end of the bitstream, the NNPF is applied up to but excluding a set of input pictures that would cause creation of any interpolated picture after the last picture of the bitstream in output order.

- Action on 2)f with the change of "the CLVS" to "the bitstream": Fix an asserted bug in the derivation of input pictures for the case when there are multiple input pictures for an NNPF activated for currCodedPic and currCodedPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, taking into account that there can be multiple pictures preceding an input picture in output order.

- Action on item 2)g: Specify that any multi-picture-input NNPF can only be used when all input pictures have the same dimensions after applying super resolution post-filter applicable to the input pictures, if any, using text provided in JVET-AD0057 item 6.

- Action on item 4)a: Add the following constraints: It is a requirement of bitstream conformance that the value of nnpfc_pic_width_in_luma_samples % outSubWidthC shall be equal to 0. It is a requirement of bitstream conformance that the value of nnpfc_pic_height_in_luma_samples % outSubHeightC shall be equal to 0.

- Action on item 4)f: Specify that the width of the NNPF output picture shall be in the range of CroppedWidth to CroppedWidth * 16, inclusive (instead of CroppedWidth to CroppedWidth * 16 − 1, inclusive), and the width of the NNPF output picture shall be in the range of CroppedHeight to CroppedHeight * 16, inclusive (instead of CroppedHeight to CroppedHeight * 16 − 1, inclusive).

- Action on item 5)e: Add a metadata extension mechanism in the NNPFC SEI message, to allow new metadata syntax elements to be added in the NNPFC SEI message in future VSEI amendments or editions, and allow legacy decoders (with the support of the NNPFC SEI message as defined in this VSEI edition) to skip and ignore such additional metadata syntax elements.

- Action on item 6)a.i: Move nnpfc_base_flag next to nnpfc_id in the NNPFC SEI message syntax.

- Action on item 6)b: Make changes to avoid a repetition of an NNPFC SEI message affecting the persistence of an NNPF update.

- Action on item 6)c: Fix the incorrect inference of upsampled resolution, colour primaries, transfer characteristics, and matrix coefficients syntax elements when nnpfc_property_present_flag is equal to 0

- Action on item 6)e: Enable the activation of the base NNPF with a particular nnpfc_id value after an NNPF update for the same nnpfc_id is available.

- Action on item 6)g: In the VVC text, disallow the NNPFA SEI message from associated with a non-output picture and disallow any input pictures being a non-output picture.

- Action on item 6)h: Change the persistence scope of the NNPFC SEI message to "The CLVS containing the SEI message".

- Action on items 8)a and 8)a.i: Move nnpfc_inp_order_idc and nnpfc_out_order_idc before the bit depth syntax elements, and gate the bit depth syntax elements so that the luma bit depth is present only when the input or output tensor contains luma, and the chroma bit depth is present only when the input or output tensor contains chroma. And update the related constraints on the values of nnpfc_out_tensor_luma_bitdepth_minus8 and nnpfc_out_tensor_chroma_bitdepth_minus8 accordingly.

- Action on item 8)b: Add syntax conditions such that nnpfc_luma_padding_val is present only when the input tensor contains a luma sample array, and nnpfc_cb_padding_val and nnpfc_cr_padding_val are present only when the input tensor contains chroma sample arrays.

- Action on item 8)c: Signal nnpfc_auxiliary_inp_idc before nnpfc_inp_order_idc, as the semantics of nnpfc_inp_order_idc syntax element use nnpfc_auxiliary_inp_idc.

- Action on item 9)a: Add the following constraint: When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_inp_order_idc shall not be equal to 0.

- Action on item 9)c: Add a constraint that disallows nnpfc_inp_order_idc indicating that the input tensor has chroma component when ChromaFormatIdc is equal to 0.

- Action on item 9)d: Make the following change (additions indicated with underline): When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_out_order_idc shall not be equal to 0 or 3.

- Action on item 9)e: Add a constraint such that when the purpose indicates colorization, there shall be chroma output by the NNPF.

- Action on item 11)a: Change the definition of the function InpSampleVal to take the component index (0 for luma, 1 for Cb, 2 for Cr) as an additional input argument. For nnpfc_padding_type equal to 4, change the function InpSampleVal to return the fixed luma, Cb, or Cr padding value corresponding to the component index given as input argument. For each invocation of the function InpSampleVal, add the component index among the input arguments.

- Action on item 12)a: Remove the following constraints on nnpfa_target_id values, which were asserted to be enforced by other constraints and are therefore asserted to be redundant: Values of nnpfa_target_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFA SEI message with nnpfa_target_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

- Action on item 13)a: Allow inclusion of NNPFC and NNPFA SEI messages in suffix SEI NAL units.

- Action on item 13)b: Disallow the post-filter hint SEI message to be present in both a prefix SEI NAL unit and a suffix SEI NAL unit in the same picture unit.

- Action on item 14)a: Specify an array of filtering strength control variables for multiple input pictures (as StrengthControlVal[ idx ] values for the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive), and derive an array scaled strengthControlScaledVal[ i ] values for multiple input pictures and use them to derive the input tensors.

- Action on item 14)c: Make changes to Equation (83) such that when nnpfc_inp_order_idc is equal to 1 (i.e., only chroma is present in the input tensor), the variable strengthControlScaledVal is derived from the variable inpTensorBitDepthC (instead of from inpTensorBitDepthY).

- Action on item 14)d: Set StrengthControlVal equal to ( SliceQpY + QpBdOffset ) ÷ ( 63 + QpBdOffset), instead of SliceQpY ÷ 63, where SliceQpY is that of the first slice of currCodedPic.

- Action on item 16)a.ii: Clarify that the output from the PostProcessingFilter (i.e. outputTensor) does not include overlap regions.

# BoG notes based on the JVET-AD0258 summary

In this summary, the proposals on NNPF SEI messages and the SEI processing order SEI message are classified into the following 19 categories:

1) On cascading of post-processing filters and indication of the preferred processing order when cascaded

2) On input pictures to an NNPF

3) On NNPF output pictures

4) On NNPF output picture size and bit depth

5) On general signalling of NNPF parameters: network property, activation parameters, extensibility

6) On NNPF repetition, update, persistence, activation, and association

7) On buffering requirements for NNPFs with multiple input pictures

8) On signalling of luma and chroma components related syntax elements

9) On presence of luma and chroma components in input and output tensors

10) On VUI parameters in the NNPFC SEI message

11) On the function InpSampleVal( )

12) On NNPF ID and description

13) On inclusion of NNPF SEI messages in SEI NAL units

14) On NNPFC auxiliary input

15) On additional NNPF capabilities

16) On NNPFC overlap

17) On value ranges of ue(v)-coded syntax elements

18) Editorial changes

19) Region-based filtering

Below is the list of design questions / proposals, wherein "BC#nnn" refers to ballot comment #nnn from MPEG input document m62571.

- On cascading of post-processing filters and indication of the preferred processing order when cascaded

  - Clearly allow cascading of post-processing filters (NNPFs or non-NN post-processing filters) signalled by SEI messages and clearly specify preferred processing order when cascading, and do one of the following? (BC#028 option 1, JVET-AD0087 option 1)

    - Update the SEI processing SEI message to cover NNPFs, and include the SEI processing order SEI message into VVC v3. (JVET-AD0087 option 1.1)

    - Update the SEI processing SEI message to include NNPFs but still plan the SEI processing order SEI message for VVC v4. (JVET-AD0087 option 1.2)

    - Address the signalling of preferred processing order of NNPFs in VSEI v3, leaving the SEI processing SEI message for different types of non-neural-network post-processing filters (JVET-AD0087 option 1.3), and do one of the following:

      - Use the value of nnpfc_id to indicate the preferred processing order of NNPFs. (JVET-AD0087 option 1.3.1)

      - Use a new SEI message to indicate the preferred processing order of NNPFs, e.g., like the proposal in JVET-AC0299. (JVET-AD0087 option 1.3.2)

  - (Ignore this if yes to 1)a) Clearly disallow cascading of post-processing filters (NNPFs or non-NN post-processing filters) signalled by SEI messages, and do all of the following? (BC#028 option 2, JVET-AD0087 option 2)

    - Explicitly disallow the activation of more than one post-processing filter (NNPFs or non-NN post-processing filters) for any particular picture.

    - Remove the process for the only currently allowed special-case cascading of NNPFs doing spatial resolution upsampling followed by picture rate upsampling.

    - Cancel the development of the SEI processing SEI message.

BoG discussion for items 1)a and 1)b was chaired by Sachin Deshpande.

Question: Regarding the approaches in item 1)a.iii, if cascading of an NNPF and a non-NN postprocessing filter is allowed, how to indicate the processing order of the two filters? No good answer was identified during the discussion.

On indication of (preferred) processing order in general: When adding another post-processing filter to an existing bitstream with some post-processing filters, how to indicate the (preferred) processing order of all the post-processing filters in the updated bitstream? It was said that JVET-AC0299 resolves this issue, by having some gaps initially.

A) One way to progress is to plan to do, in v4 (of VSEI and VVC), 1)a.ii without clearly specifying the behavior of multiple NNPFs activated for the same picture in v3 except for the special case of spatial resolution upsampling followed by picture rate upsampling. Note that even not needed for this purpose, with this option it is still possible in v4 to specify a new NNPFA SEI message for some new way of activating NNPFs.

B) Another way to progress is to plan to do, in v4, 1)a.ii and explicitly disallow the activation of more than one post-processing filter (NNPFs or non-NN post-processing filters) for any particular picture in v3.

C) Yet another way is to plan to do, in v4, 1)a.ii and add, in VVC, a "should" and non-NOTE wording saying that multiple-activated NNPFs for a picture are alternatives in v3 (JVET-AD0059 section 2.1), and in v4 a new NNPFA SEI message could be specified for cascading activation.

BoG recommendation: Adopt option A.

Items 1)c and 1)d are for v4 (of VSEI and VVC). To be reviewed after other items for v3.

  - (Ignore this if yes to 1)b) Indicate an NNPF group where each NNPF is activated at the same time and applied in cascade by specifying either of the following: (JVET-AD0059 item 1)

    - NNPF group and NNPF group activation SEI messages, where the NNPF group SEI message optionally includes complexity information. (JVET-AD0059 item 1 option A)

    - An NNPFC SEI message with nnpfc_mode_idc equal to 2 defines an NNPF group. The NNPFA SEI message activates a single NNPF or an NNPF group, depending on whether the nnpfa_target_id refers to an NNPFC SEI message with nnpfc_mode_idc less than 2 or nnpfc_mode_idc equal to 2, respectively. There is no change to the NNPFA SEI message syntax. (JVET-AD0059 item 1 option B)

  - (Ignore this if yes to 1)b) Update the SEI processing SEI message to enable signalling of prefix information for other types of SEI messages than the ITU-T T.35 SEI message?

    - Yes, per JVET-AD0059 item 2, which has the following syntax:

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0, b = 0; b < payloadSize; i++, b += 4 ) { |  |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(15) |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_prefix_bytes[ i ] | b(8) |
| b++ |  |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| b += po_num_prefix_bytes[ i ] |  |
| } |  |
| po_sei_processing_order[ i ] | u(16) |
| } |  |
| } |  |

    - Yes, per JVET-AD0061, which has the following syntax:

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0, b = 0; b < payloadSize; i++, b += 4 ) { |  |
| po_sei_payload_type[ i ] | u(16) |
| po_num_prefix_byte[ i ] | b(8) |
| b ++; |  |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| b + = po_num_prefix_byte[ i ] |  |
| po_sei_processing_order[ i ] | u(16) |
| } |  |
| } |  |

    - Yes, per JVET-AD0079, which has the following syntax:

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0, b = 0; b < payloadSize; i++, b += 5 ) { |  |
| po_sei_payload_type[ i ] | u(16) |
| po_num_bytes_in_prefix_indication[ i ] | b(8) |
| for( j = 0; j < po_num_bytes_in_prefix_indication[ i ]; j++ ) |  |
| po_prefix_data_byte[ i ][ j ] | b(8) |
| b += po_num_bytes_in_prefix_indication[ i ] |  |
| } |  |
| po_sei_processing_order[ i ] | u(16) |
| } |  |
| } |  |

- On input pictures to an NNPF

  - Specify the following: When applying an NNPF to a current picture for which the NNPF is activated, the current picture is always an input picture to the NNPF. (BC#023)

BoG discussion for this item chaired by Miska M. Hannuksela.

This is already the case, in both the VSEI and VVC texts. No action.

  - Limit input pictures to an NNPF activated for any current picture to be within the same CLVS as the current picture?

    - Yes, and add both of the following: (JVET-AD0054 item 1)

      - An indication nnpfc_absent_input_pic_zero_flag in the NNPFC SEI message that indicates how pictures that would not originate from the current CLVS are expected to be replaced in the input tensor. (JVET-AD0054 item 2)

        - nnpfc_absent_input_pic_zero_flag equal to 1 indicates that the NNPF expects an input picture that is not present in the current CLVS to be represented sample arrays with sample values equal to 0.

        - nnpfc_absent_input_pic_flag equal to 0 indicates that the NNPF expects an input picture that is not present in the current CLVS to be represented by the closest input picture in output order within the current CLVS.

      - A constraint that NNPF shall not be activated so that it creates an interpolated picture before the CLVS start in output order. (JVET-AD0054 item 3)

    - No, and for the derivation of which pictures are used as in the input pictures, instead of using picture order count values, plain language phrases like "the previous picture in output order" and "the n-th previous picture in output order" are used. (JVET-AD0090 item 1)

  - When an NNPF is activated for a current picture and the NNPF is indicated to have multiple input pictures but some of the input pictures are not present, the number of input pictures is updated to be equal to the number of pictures that satisfy the condition for input pictures and are present in the CLVS (or in the bitstream, if an input picture from an earlier CLVS of the CLVS containing the current picture is allowed). (JVET-AD0090 item 3)

BoG discussion for items 2)b and 2)c was chaired by Sachin Deshpande.

BoG recommendation: Adopt item 2)b.ii, i.e., to allow input pictures to an NNPF activated for any current picture to come from an earlier CLVS than the CLVS containing the current picture, and for the derivation of which pictures are used as in the input pictures, instead of using picture order count values, plain language phrases like "the previous picture in output order" and "the n-th previous picture in output order" are used.

BoG recommendation: Adopt items 2)b.i.1 and 2)b.i.2, for both of which "the current CLVS" is repalced with "the bitstream", for alignment with the action on item 2)b.ii.

Another option, as alternative to items 2)b.i.1 and 2)b.i.2 with the above adjustment, is to require that all input pictures for a current picture to be present in the bitstream, which seems technically equivalent to item 2)c. Due to the action on to items 2)b.i.1 and 2)b.i.2 with the above adjustment no action on this option and no action on item 2)c.

  - At the end of a CLVS, repeated inference of NNPF for creating interpolated pictures: When an NNPF is active until the end of the CLVS, the NNPF is applied up to but excluding a set of input pictures that would cause creation of any interpolated picture after the last picture of the CLVS in output order. (JVET-AD0054 item 4)

BoG discussion for this item was chaired by Sachin Deshpande.

BoG recommendation: Adopt item 2)d with the change of "the CLVS" to "the bitstream".

  - Limit the special treatment of temporal interleaving frame packing arrangement to picture rate upsampling only rather than for any multi-picture-input NNPF?

    - Yes (JVET-AD0057 item 5)

    - No (JVET-AD0090 item 2)

BoG discussion for this item was chaired by Sachin Deshpande.

Revisit.

  - Fix an asserted bug in the derivation of input pictures for the case when there are multiple input pictures for an NNPF activated for currCodedPic and currCodedPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, taking into account that there can be multiple pictures preceding an input picture in output order and there can also be no such pictures. (JVET-AD0090 item 2)

BoG discussion for this item was chaired by Sachin Deshpande.

BoG recommendation: Adopt the first part of item 2)f, not including the change that addresses that there can also be no such pictures.

  - Specify that any multi-picture-input NNPF can only be used when all input pictures have the same dimensions after applying super resolution post-filter applicable to the input pictures, if any. (JVET-AD0057 item 6, JVET-AD0143 item 1)

    - Specify in the VVC spec. (JVET-AD0057 item 6)

    - Specify in the VSEI spec. (JVET-AD0143 item 1)

BoG discussion for this item was chaired by Ye-Kui Wang.

BoG recommendation: Adopt item 2)g, using text provided in JVET-AD0057 item 6.

  - Add the following constraint: (JVET-AD0141 item 3)

An input picture for an activated NNPF shall not be a discardable picture.

BoG discussion for this item was chaired by Ye-Kui Wang.

It was commented that if to specify something like this, it would be a "should" wording and "discardable picture" should be clearly defined.

It was commented that in some cases, e.g., a temporal scalable bitstream wherein higher sublayer pictures are discardable, but allowing activation of NNPFs at those pictures is important for many use cases.

No action on this item.

  - Enable NNPFs to use input pictures that are not consecutive in output order, by making the following changes. (JVET-AD0145)

    - When nnpfc_num_input_pics_minus1 is greater than 0, signal a new flag nnpfc_consecutive_input_pics_flag to specify whether the input pictures are consecutive or not.

    - If nnpfc_consecutive_input_pics_flag is equal to 0, a loop of nnpfc_input_pic_use_flag[ i ] is signalled to specify whether the i-th input picture is used or not for the filtering process.

  - Enable the detection of discarded or lost input pictures to NNPFs by adding NNPFC picture identifier syntax elements, using either the following syntax options: (JVET-AD0159)

    - Use the following syntax to add a syntax element for each input picture (JVET-AD0159 option A):

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_delta_pic_identifier_minus1[ i ] | ue(v) |
| … |  |

    - Use the following syntax to add a single picture identifier delta syntax element for the NNPF input picture that precedes all other input pictures in output order (JVET-AD0159 option B):

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( nnpfc_num_input_pics_minus1 > 0 ) |  |
| nnpfc_delta_pic_identifier_minus1 | ue(v) |
| … |  |

BoG discussion for items 2)i and 2)j was chaired by Ye-Kui Wang.

It was commented that this item and the next item have some overlaps, and they both are related to a previous-meeting decision on not to take into temporal ID for derivation of the input pictures to an NNPF, and the basis for that decision seemed applied to these two items. But it was said that item j is about loss detection, not about temporal ID based operations.

It was commented that POC should not be used for such purposes (regarding item j). And an eralier decision made at this meeting changed to not use POC for derivation of the input pictures.

No action on item 2)i.

No action on item 2)j.

- On NNPF output pictures

  - Is the following sentence in JVET-AC2032 regarding the output order of pictures generated by applying the NNPF(s) clear?

The order of the pictures in the stored output tensor is in output order, and the output order generated by applying the NNPF in output order is interpreted to be in output order (and not conflicting with the output order of the input pictures).

    - No, not clear (BC#024, JVET-AD0053, JVET-AD0088).

  - Do all of the following, in the VVC text (JVET-AD0053):

    - Remove the reportedly unclear constraint regarding the output order of pictures resulting from applying the NNPF in output order.

    - When picture rate upsampling is not in use, constrain the sequence of output pictures formed by concatenating the output pictures in the order that output tensors are generated by applying the NNPF as activated by the NNPFA SEI messages:

      - to contain the output pictures in output order, and

      - to contain at most one output picture representing the same input picture.

    - Constrain the interpolated pictures of a picture rate upsampling NNPF, when applied as activated by NNPFA SEI message(s), to appear in output order.

    - Allow the same input picture of a picture rate upsampling NNPF to be filtered more than once when the NNPF is activated for different sets of input pictures by NNPFA SEI message(s).

    - When the same input picture is filtered more than once, informatively suggest the filtered picture resulting from the first activation in output order to be used for subsequent processing.

  - Specify a general post-processing filtering process using NNPFs, wherein the input to the process is a bitstream BitstreamToFilter, and the output of the process is a list of NNPF output pictures. Then clearly specify the above constraint based on the general post-processing filtering process. (BC#024, JVET-AD0088)

  - Disallow NNPFs to output a picture multiple times for any particular output order or output time instance. (JVET-AD0091 item 8)

  - Add a NOTE to describe/clarify the notion of output picture and the corresponding input picture as follows: (JVET-AD0151 item 1)

    - An input picture that has a corresponding picture outputted from NNPF filtering process may be replaced by the output picture in the picture bitstream after the filtering process.

    - An input picture that has no corresponding picture outputted from NNPF filtering process is present in the picture stream after them filtering process.

  - Include nnpfc_input_pic_output_flag[ i ] in the NNPFC SEI message when nnpfc_num_input_pics_minus1 is greater than 0 regardless of the nnpfc_purpose value. (JVET-AD0056 item 5, JVET-AD0091 item 2, JVET-AD0151 item 2 option 1, BC#011)

    - In addition, add the following constraint:

      - When nnpfc_purpose & 0x08 is equal to 0 and nnpfc_num_input_pics_minus1 is greater than 0, nnpfc_input_pic_output_flag[ i ] shall be equal to 1 for at least one value of i in the range of 0 to nnpfc_num_input_pics_minus1, inclusive. (JVET-AD0091 item 2)

      - When nnpfc_purpose & 0x08 is equal to 0, the value of nnpfc_input_pic_output_flag[ i ] shall be equal to 1 for at least one i in the range of 0 to nnpfc_num_input_pics_minus1, inclusive. (JVET-AD0151 item 2 option 1)

    - In addition, add the following inference (JVET-AD0056 item 5)

When nnpfc_num_input_pics_minus1 is equal to 0, nnpfc_input_pic_output_flag[ 0 ] is inferred to be equal to 1.

  - (Ignore this if yes to 3)f) Clarify that when the purpose of the NNPF does not include picture rate upsampling, the value of nnpfc_input_pic_output_flag[ 0 ] is inferred to be equal to 1 and the value of nnpfc_input_pic_output_flag[ i ] for i in the range from 1 to nnpfc_num_input_pics_minus1, inclusive, is inferred to be equal to 0. (JVET-AD0151 item 2 option 2)

- On NNPF output picture size and bit depth

  - Add the following constraints: It is a requirement of bitstream conformance that the value of nnpfc_pic_width_in_luma_samples % outSubWidthC shall be equal to 0. It is a requirement of bitstream conformance that the value of nnpfc_pic_height_in_luma_samples % outSubHeightC shall be equal to 0. (JVET-AD0056 item 4)

BoG discussion for this item was chaired by Ye-Kui Wang.

BoG recommendation: Adopt item 4)a.

BoG discussions for the remaining items in this category were Miska M. Hannuksela.

Notes for item 4)b:

It was generally agreed by the BoG to purpose signalling of the scaling ratios for deriving the NNPF output picture width and height.

Question: Has rounding been properly taken care of in the derivations of the scaling ratios and the calculations of the NNPF output picture width and height based on the scaling ratios.

It was agreed by the BoG to use "minus1" regardless of fixed-length coding or ue(v) coding.

Fixed-length coding or ue(v) coding? It was agreed by the BoG to use ue(v) coding.

Delta or not (equivalent to not having the possibility of allowing downsampling or having the possibility of allowing downsampling)? Revisit after disposal of the proposal on adding the downsampling capability and/or the deinterlacing capability.

Revisit. Hendry was requested to work on a text that implements the above BoG decisions, taking into account the rounding issue mentioned above.

On item 4)c, no benefit of having both and switching between the flag has been agreed in the BoG. No action on this item.

No action on items 4)d and 4)e due to the decisions on 4)b and 4)c.

Notes for item 4)f:

BoG recommendation: Even with the signalling of the scaling ratios, the techncial intent of the constraints is still needed for fixing bugs. Unless the constraint is contained in the constraints to the values of the scaling ratios, they need to be expressed based on the variables of the NNPF output picture width and height based on the scaling ratios.

On item 4)g: deferred.

On item 4)h: Revisit.

  - For spatial resolution upsampling, signal the scaling ratios instead of the NNPF output picture width and height. (JVET-AD0067 item 5, JVET-AD0091 item 3, JVET-AD0143 item 2, BC#013)

- With the following syntax (JVET-AD0067 item 5 option 1, where the numerator is signalled as a delta compared to denominator):

| nnpfc_out_pic_width_scale_factor_denominator_minus1 | u(20) |
| --- | --- |
| nnpfc_out_pic_width_scale_factor_numerator_delta | u(16) |
| nnpfc_out_pic_height_scale_factor_denominator_minus1 | u(20) |
| nnpfc_out_pic_height_scale_factor_numerator_delta | u(16) |

- With the following syntax (JVET-AD0067 item 5 option 2, where the numerator is signalled directly, not as a delta compared to the denominator):

| nnpfc_out_pic_width_scale_factor_denominator_minus1 | u(20) |
| --- | --- |
| nnpfc_out_pic_width_scale_factor_numerator_minus1 | u(20) |
| nnpfc_out_pic_height_scale_factor_denominator_minus1 | u(20) |
| nnpfc_out_pic_height_scale_factor_numerator_minus1 | u(20) |

- With the following syntax (JVET-AD0091 item 3, basically the same as the ratio signalling in the phase indication SEI message):

| nnpfc_hor_ratio_num_minus1 | u(8) |
| --- | --- |
| nnpfc_hor_ratio_den_minus1 | u(8) |
| nnpfc_ver_ratio_num_minus1 | u(8) |
| nnpfc_ver_ratio_den_minus1 | u(8) |

- With the following syntax (JVET-AD0143 item 2, using ue(v) instead of u(N) and without "minus1"):

| nnpfc_pic_width_ratio_num | ue(v) |
| --- | --- |
| nnpfc_pic_width_ratio_denum | ue(v) |
| nnpfc_pic_height_ratio_num | ue(v) |
| nnpfc_pic_height_ratio_denum | ue(v) |

  - For spatial resolution upsampling, enable signalling either the width plus height or the scaling ratios, controlled by a new flag. (JVET-AD0143 item 2)

  - (Ignore this if yes to 4)b and no to 4)c) Code nnpfc_pic_width_in_luma_samples as a delta with an offset of CroppedWidth and nnpfc_pic_height_in_luma_samples as a delta with an offset of CroppedHeight. (JVET-AD0067 item 3)

  - (Ignore this if yes to 4)b and no to 4)c) Add a sensibility constraint to prevent the NNPF purpose indicating resolution upsampling while the NNPF output picture having the same exact resolution as the input picture. (JVET-AD0067 item 4, JVET-AD0091 item 4, BC#020)

  - (Ignore this if yes to 4)b and no to 4)c) Specify the following constraints: (JVET-AD0091 item 4, BC#020)

    - The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16, inclusive. (instead of CroppedWidth to CroppedWidth * 16 − 1, inclusive)

    - The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16, inclusive. (instead of CroppedHeight to CroppedHeight * 16 − 1, inclusive)

  - Add a constraint on NNPF input and output picture width and height values to avoid changing in opposite directions from input to output (e.g., width getting bigger while height getting smaller). (JVET-AD0093)

  - Add a constraint on NNPF input and output bit depth values to avoid changing in opposite directions from input to output (e.g., luma bit depth getting bigger while chroma bit depth getting smaller). (JVET-AD0233 item 1, BC#018)

- On general signalling of NNPF parameters: network property, activation parameters, extensibility

  - Specify an activation parameters mechanism that enables passing of certain parameters to an NNPF when the NNPF is activated, including (JVET-AD0089, BC#015)

    - nnpfc_input_pic_output_flag[ i ]

    - nnpfc_out_order_idc

    - nnpfc_overlap

    - nnpfc_padding_type

    - nnpfc_luma_padding_val

    - nnpfc_cb_padding_val

    - nnpfc_cr_padding_val

    - StrengthControlVal

BoG discussions for this item and other items in this category were chaired by Sachin Deshpande.

There were some interests expressed.

There were concerns on the amounts of text changes at this stage of this project. Regarding this, it was mentioned that there is a possibility of finalizing v3 of VVC and VSEI in July 2023 instead of in April 2023.

It was commented that, if we don't do this in v3, it is possible to specify a new NNPFA mechanism in v4 that enables passing such parameters and override what's in the NNPFC SEI message, and with that, repeating the NNR bitstream unnecessarily can also be avoided.

There was no clear conclusion on which of the syntax elements listed above should be kept in the NNPFC and which should be as proposed.

No action on this item.

  - Is nnpfc_component_last_flag is needed at all? If needed, should it be an activation parameter, or a parameter signalled in the NNPFC SEI message (as currently)? (JVET-AD0089, BC#015)

BoG notes:

No action on this item.

  - Enable an NNR bitstream (i.e., an ISO/IEC 15938-17 bitstream) carried in an NNPFC SEI message to be referenced by other NNPFC SEI messages, by making the following changes: (JVET-AD0149)

    - Allow a new value for nnpfc_mode_idc (i.e., value 2) to specify that the NNPFC SEI message contains only an ISO/IEC 15938-17 bitstream.

      - NNPFC SEI message with nnpfc_mode_idc equal 2 does not contain any property information (i.e., the value of nnpfc_property_present_flag is equal to 0).

      - NNPFC SEI message with nnpfc_mode_idc equal 2 cannot be activated by NNPFA SEI message.

      - NNPFC SEI message with nnpfc_mode_idc equal 2 may be referred to by other NNPFC SEI message with different nnpfc_id.

      - The value of nnpfc_id of an NNPFC SEI message with nnpfc_mode_idc equal 2 is unique. When an NNPFC SEI message seiB with nnpfc_mode_idc equal 2 follows an NNPFC SEI message seiA with nnpfc_mode_idc equal 2, in decoding order, and both seiA and seiB have the same nnpfc_id values, then seiB is a repetition of seiA.

    - When the value of nnpfc_mode_idc is equal to 1, signal a new flag nnpfc_filter_in_uri_flag to specify whether the described NNPF is present in location pointed by an URI address or in previous NNPFC SEI message with nnpfc_mode_idc equal to 2.

    - When NNPFC SEI message seiA with nnpfc_mode_idc equal to 2 is referenced by NNPFC SEI message seiB with nnpfc_mode_idc equal to 1, seiA shall precede seiB in decoding order within the coded bitstream.

    - When the value of nnpfc_mode_idc is equal to 2, the following applies for the value of nnpfc_purpose:

      - If the value of nnpfc_purpose is set equal to 0, the SEI message may be referenced by other NNPFC SEI messages with any value of nnpfc_purpose.

      - Otherwise (i.e., the value of nnpfc_purpose is not 0), when the SEI message seiA references another NNPFC SEI message seiB, the value of nnpfc_purpose of seiA and seiB shall be the same.

BoG notes:

It was commented that the proposal here was an alternative approach to activation parameters, for avoiding repeating the NNR bitstream unnecessarily.

It was commented that mixing the URI mode with the other mode should not be done.

There was concern on doing this at this stage of this project.

No action on this item at this meeting.

  - Enable addition of machine analysis related indications in the NNPFC SEI message in a future VSEI version by making either of the following changes? (JVET-AD0052)

    - In this VSEI version, add the target usage (nnpfc_target_usage) syntax element as a ue(v)-coded bit field, where bit 0 is defined to indicate user viewing. A machine analysis target usage can be added in a future VSEI version by defining another bit of nnpfc_target_usage. (JVET-AD0052 option 1)

    - In this VSEI version, add an nnpfc_purpose bit to indicate whether the intended usages include user viewing. A machine analysis bit for nnpfc_purpose can be added in a future VSEI version. (JVET-AD0052 option 2)

BoG notes:

It was commented that the NNPFC SEI message syntax is special that the mode 0 does not allow using the general SEI extension mechanism that is possible for all other SEI messages.

Assuming that we have an extension mechanism, no action on the proposal at the meeting.

Revisit to determine whether mode 0 should have some extensibility mechanism enabled, e.g., the general SEI extension mechanism.

  - Add a metadata extension mechanism in the NNPFC SEI message, which is asserted to: (JVET-AD0056 item 11)

    - allow new metadata syntax elements to be added in the NNPFC SEI message in future VSEI amendments or editions, which could for example be additional means to describe the neural network complexity, and

    - allow legacy decoders (with the support of the NNPFC SEI message as defined in this VSEI edition) to skip and ignore such additional metadata syntax elements.

BoG notes:

It was concluded that the general SEI extension mechanism is by default a metadata extension mechanism.

Do we want to have a metadata extension mechanism, wherein decoders per the current version of the standard would ignore the extension bits, when present?

It was commented that it is possible to use a new SEI message to carry additional metadata without specifying such a metadata extension mechanism.

Do we want to have a non-metadata extension mechanism, wherein decoders per the current version of the standard would ignore the entire SEI message when extension bits are present?

It was commented that such capability could be realized by defining new NNPF modes. It was agreed not to add a non-metadata extension mechanism at this stage of the project.

BoG recommendation: Adopt item 5)e.

  - To enable future extensions of the NNPFA SEI message, e.g., for indicating regions for which an NNPF is activated, add an NNPFA SEI message extension mechanism in VSEI v3, where the NNPFA SEI message syntax contains any number of nnpfa_extension_bit[ i ] until the end of the payload. nnpfa_extension_bit[ i ] is reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this VSEI version. Decoders conforming to this VSEI version encountering an NNPFA SEI message with any nnpfa_extension_bit[ i ] present shall ignore the SEI message. (JVET-AD0058)

BoG notes:

This is propsoing a non-metadata extension mechanism, wherein decoders per the current version of the standard would ignore the entire SEI message when extension bits are present. The proposed details would change the existing general SEI extension mechanism from a metadata extension mechanism to a non-metadata extension mechanism, though.

No action on this item.

- On NNPF repetition, update, persistence, activation, and association

  - Keep both nnpfc_property_present_flag and nnpfc_base_flag?

    - Yes, and move nnpfc_base_flag next to nnpfc_id in the NNPFC SEI message syntax. (JVET-AD0056 item 1, JVET-AD0067 item 6)

This change is asserted to simplify decoder operation to conclude if the NNPFC SEI message contains a base NNPF or an NNPF update. (JVET-AD0056 item 1)

    - No, remove nnpfc_base_flag and disable two types of NNPF update: the so-called full update of an NNPF (wherein nnpfc_property_present_flag is equal to 1 and the NNPF property parameters are present in the NNPFC SEI message but are different from those in the base NNPF) and the update of the maximum NNPF complexity parameters to be smaller. (JVET-AD0091 item 1)

BoG discussion for this item was chaired by Sean McCarthy.

It was commented that the two are not mutually exclusive; it is possible to keep only one of the two flags and move the kept flag to an earlier position.

Is the full update of an NNPF (wherein nnpfc_property_present_flag is equal to 1 and the NNPF property parameters are present in the NNPFC SEI message but are different from those in the base NNPF) import to support?

Is the update of the maximum NNPF complexity parameters to be smaller important to support?

Do we want to keep both of the two flags or just keep one of the two?

No action on item 6)a.ii.

BoG recommendation: Adopt item 6)a.i.

  - It is asserted that a repetition of NNPFC SEI message defining the base NNPF affects the persistence of an NNPF update, which is asserted not to be the design intent. Make changes to avoid a repetition of an NNPFC SEI message affecting the persistence of an NNPF update. (JVET-AD0056 item 2)

BoG discussion for this item was chaired by Sachin Deshpande.

BoG recommendation: Adopt item 6)b.

  - It is asserted that the specified inference of upsampled resolution, colour primaries, transfer characteristics, and matrix coefficients syntax elements is incorrect when nnpfc_property_present_flag is equal to 0. (JVET-AD0056 item 3)

If moving nnpfc_base_flag earlier in the syntax is adopted, make the following changes (where removal is indicated with italics and addition is indicated with underline):

When nnpfc_property_present_flag is equal to 0, the values of all syntax elements that may be present only when nnpfc_property_present_flag is equal to 1 and for which inference values for each of them is not specified are inferred to be equal to their corresponding syntax elements, respectively, in the NNPFC SEI message that contains the base NNPF for which this SEI message provides an update.

Otherwise, make the following changes:

When nnpfc_property_present_flag is equal to 0, the values of all syntax elements that may be present only when nnpfc_property_present_flag is equal to 1 and follow nnpfc_base_flag for which inference values for each of them is not specified are inferred to be equal to their corresponding syntax elements, respectively, in the NNPFC SEI message that contains the base NNPF for which this SEI message provides an update.

BoG discussion for this item was chaired by Sachin Deshpande.

BoG recommendation: Adopt item 6)c.

  - Specify that any particular NNPFA SEI message with a particular nnpfa_target_id value ends the persistence of the previous NNPFA SEI message with the same nnpfa_target_id value, regardless of the value of nnpfa_cancel_flag of the particular NNPFA SEI message. (JVET-AD0056 item 14)

BoG discussion for this item was chaired by Sachin Deshpande.

No action on this item.

  - Enable the activation of the base NNPF with a particular nnpfc_id value after an NNPF update for the same nnpfc_id is available. The activation of the base NNPF could be advantageous, when an update is derived from the first GOP of a segment but the picture content of the segment changes later considerably. Make the following changes: (JVET-AD0056 item 13)

    - Add nnpfa_base_flag syntax element in the NNPFA SEI message under the condition that nnpfa_cancel_flag is equal to 0.

    - nnpfa_base_flag equal to 1 specifies that the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id. nnpfa_base_flag equal to 0 specifies the target NNPF as currently specified in JVET-AC2032.

BoG discussion for this item was chaired by Sachin Deshpande.

Whenever there is an update to an NNFP, two copies of the NNPF need to be stored, both the base and the updated version. The question is then, now that we have also the base NNPF available, why not allow the use of it by activating it?

BoG recommendation: Adopt item 6)e.

  - Add the following constraint: (JVET-AD0141 item 1)

The NNPFC SEI message shall not be associated with a discardable picture.

  - Add the following constraint: (JVET-AD0141 item 2)

The NNPFA SEI message shall not be associated with a non-output picture.

BoG discussion for the above two items was chaired by Sachin Deshpande.

No action on item 6)f.

BoG recommendation: Adopt item 6)g, but using the phrase picture_output_flag or the corresponding variable (whichever is better) equal to 0 instead of the term "non-output picture". Additionally, also disallow input pictures being pictures having picture_output_flag equal to 0. Integrate these changes into the VVC text.

  - Change the persistence scope of the NNPFC SEI message to "The CLVS containing the SEI message". (BC#006)

BoG discussion for the above item was chaired by Sachin Deshpande.

BoG recommendation: Adopt item 6)h.

- On buffering requirements for NNPFs with multiple input pictures

  - When the number of input pictures is greater than 1, add following syntax elements for the NNPFC SEI message to indicate the buffering and latency needs: (JVET-AD0055)

    - nnpfc_latency_info_present_flag specifying the presence of nnpfc_max_num_reorder_pics_increase and nnpfc_max_latency_increase

    - nnpfc_max_num_reorder_pics_increase specifying the maximum additional number of picture storage buffers needed for storing input pictures for post-filtering on top of what would be needed for picture reordering from decoding order to output order

    - nnpfc_max_latency_increase specifying the maximum additional number of pictures that need to be decoded to form an input picture set for post-filtering on top of the number of input pictures

BoG discussion for the above item was chaired by Ye-Kui Wang.

There were some interests on this expressed.

Since different ways of activating an NNPF would result in different delays, if to do this, is it better to signal it in the NNPFC SEI message or the NNPFA SEI message?

It was commented that the latency depends on how the filtering process is implemented; in other words, it is implementation dependent.

It was clarified the proponent that the intent of the signalling is to enable minimizing the delay in the post processor.

It was agreed that the current proposal is not mature enough for inclusion into v3.

No action on this item at this meeting.

- On signalling of luma and chroma components related syntax elements

  - Move nnpfc_inp_order_idc and nnpfc_out_order_idc before the bit depth syntax elements, and gate the bit depth syntax elements so that the luma bit depth is present only when the input or output tensor contains luma, and the chroma bit depth is present only when the input or output tensor contains chroma. (JVET-AD0056 item 6, JVET-AD0067 item 1, JVET-AD0233 item 4, BC#010)

    - Additionally, make the following changes (additions indicated with underline): (JVET-AD0056 item 6)

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 is present and nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is present and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC.

  - Add syntax conditions such that nnpfc_luma_padding_val is present only when the input tensor contains a luma sample array, and nnpfc_cb_padding_val and nnpfc_cr_padding_val are present only when the input tensor contains chroma sample arrays. (JVET-AD0056 item 9, JVET-AD0233 item 4, BC#010)

  - Signal nnpfc_auxiliary_inp_idc before nnpfc_inp_order_idc, as the semantics of nnpfc_inp_order_idc syntax element use nnpfc_auxiliary_inp_idc. (JVET-AD0067 item 2)

  - When the NNPF input is in the 4:0:0 format, syntax element(s) related to the chroma input tensor are not signalled. (JVET-AD0233 item 2, BC#012)

BoG discussions for the items in this category were chaired by Hendry.

Item 8)a.i is a needed piece for item 8)a to be complete.

BoG recommendation: Adopt items 8)a and 8)a.i.

BoG recommendation: Adopt item 8)b.

BoG recommendation: Adopt item 8)c.

If the input the NNPF input is in the 4:0:0 format, does it make sense at all to have nnpfc_inp_order_idc indicating that the input tensor has chroma component?

BoG recommendation: Adopt item 9)c, which adds a constraint that disallows nnpfc_inp_order_idc indicating that the input tensor has chroma component when ChromaFormatIdc is equal to 0.

With the above constraint, there is no need to act on item 8)d.

- On presence of luma and chroma components in input and output tensors

  - Add the following constraint: (JVET-AD0056 item 7)

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_inp_order_idc shall not be equal to 0.

  - Add the following constraint: (JVET-AD0056 item 7)

When nnpfc_purpose & 0x20 is not equal to 0, nnpfc_inp_order_idc shall be equal to 0.

  - Require nnpfc_inp_order_idc to be equal to 0 when ChromaFormatIdc is equal to 0. (JVET-AD0233 item 2)

  - Make the following change (additions indicated with underline): (JVET-AD0056 item 7)

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_out_order_idc shall not be equal to 0 or 3.

  - Add the following constraint, such that when the purpose indicates colorization, there shall be chroma output by the NNPF: (JVET-AD0056 item 7, JVET-AD0233 item 3, BC#019)

When nnpfc_purpose & 0x20 is not equal to 0, nnpfc_out_order_idc shall not be equal to 0.

BoG discussions for the items in this category were chaired by Sachin Deshpande.

BoG recommendation: Adopt item 9)a.

For item 9)b, it was pointed out that the current design for colourization might not be good, as in practice there are many monochrome contents bitstreams have ChromaFormatIdc greater than 0, and many old movie or TV content are monochrome content but with some yellow-ish colour. Colourization should be allowed to be applied to those types of content. Revisit after text changes for this purpose is available.

For item 9)c, see notes above for items in category 8).

BoG recommendation: Adopt item 9)d.

BoG recommendation: Adopt item 9)e.

- On VUI parameters in the NNPFC SEI message

  - When colour primaries, transfer characteristics, and matrix coefficients are indicated for the output tensor, make the following changes: (JVET-AD0056 item 8)

    - Move nnpfc_separate_colour_description_present_flag, nnpfc_colour_primaries, nnpfc_transfer_characteristics, and nnpfc_matrix_coeffs below nnpfc_out_format_idc, nnpfc_out_tensor_luma_bitdepth_minus8, nnpfc_out_tensor_chroma_bitdepth_minus8, and nnpfc_out_order_idc.

    - Include the nnpfc_matrix_coeffs syntax element in the syntax under the constraint that nnpfc_out_format_idc is equal to 1 (indicating integer sample values in the output tensor).

    - Include constraints on nnpfc_matrix_coeffs that correspond to the constraints of vui_matrix_coeffs but depend on the chroma format, chroma bit depth, and luma bit depth of the output tensor.

  - Include a syntax element nnpfc_full_range_flag in the NNPFC message when nnpfc_separate_colour_description_present_flag is equal to 1. (JVET-AD0067 item 7, JVET-AD0233 item 6, BC#014)

  - When the purpose indicates colourization and the output is of the 4:2:0 chroma format, the location of chroma samples is signalled. (JVET-AD0233 item 6, BC#014)

- On the function InpSampleVal( )

  - It is asserted that for fixed padding, the InpSampleVal function incorrectly returns an array whereas a scalar value is expected. Therefore, make the following changes: (JVET-AD0056 item 10)

    - Change the definition of the function InpSampleVal to take the component index (0 for luma, 1 for Cb, 2 for Cr) as an additional input argument.

    - For nnpfc_padding_type equal to 4, change the function InpSampleVal to return the fixed luma, Cb, or Cr padding value corresponding to the component index given as input argument.

    - For each invocation of the function InpSampleVal, add the component index among the input arguments.

BoG discussions for the items in this category were chaired by Ye-Kui Wang.

BoG recommendation: Adopt item 11)a.

- On NNPF ID and description

  - Remove the following constraints on nnpfa_target_id values, which are asserted to be enforced by other constraints and are therefore asserted to be redundant: (JVET-AD0056 item 12)

Values of nnpfa_target_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFA SEI message with nnpfa_target_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

  - Add information to the NNPF SEI message to help identify between possible multiple NNPFs with the same nnpfc_purpose. (JVET-AD0066 proposal 1)

    - Option 1: Add a new syntax element containing human readable information about the NNPF, and the presence of this new syntax element is controlled by a new flag.

    - Option 2: Add a new flag, and signal the existing syntax element nnpfc_tag_uri when nnpfc_mode_idc is equal to 1 (as currently) and additionally when the new proposed flag is equal to 1.

  - Add a constraint on nnpfc_tag_uri when nnpfc_mode_idc is equal to 1: When a base NNPF is updated, the base and the update shall have the same nnpfc_tag_uri value. (JVET-AD0066 proposal 2)

BoG discussions for the items in this category were chaired by Ye-Kui Wang.

BoG recommendation: Adopt item 12)a.

On item 12)b:

Option 1 is about user interface. It is commented that user interface should not be in the scope.

For option 2, one usage mentioned was to specify some semantics of the tag URI. It was commented that overloading the current semantics of the tag URI is undesirable.

It was commented that without such a mechanism, if there are multiple NNPFs with the same purpose, and similar complexity, it is not possible to differentiate them; but on the other hand, different NNPFs have different NNPF IDs.

No action on item 12)b.

No action on item 12)c.

- On inclusion of NNPF SEI messages in SEI NAL units

  - Allow inclusion of NNPFC and NNPFA SEI messages in suffix SEI NAL units? (JVET-AD0057 item 1, JVET-AD0091 item 7)

  - Disallow post-filter hint, NNPFC, and NNPFA SEI messages present in both a prefix SEI NAL unit and a suffix SEI NAL unit in the same picture unit. (JVET-AD0057 item 2)

BoG discussions for the items in this category were chaired by Sachin Deshpande.

It was mentioned the ARIB disallows the use of suffix NAL units.

BoG recommendation: Adopt item 13)a.

On item 13)b: A potential use case for allowing such presence of NNPFC, and NNPFA SEI messages was identified during the discussion.

BoG recommendation: Action to disallow the post-filter hint SEI message to be present in both a prefix SEI NAL unit and a suffix SEI NAL unit in the same picture unit.

- On NNPFC auxiliary input

  - Specify an array of filtering strength control variables for multiple input pictures (as StrengthControlVal[ idx ] values for the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive), and derive an array scaled strengthControlScaledVal[ i ] values for multiple input pictures and use them to derive the input tensors. (JVET-AD0065 item 1)

  - Derive filtering strength control variable based on whether it is for chroma or luma and use that to derive the input tensors. (JVET-AD0065 item 2)

  - Make the following changes to Equation (83), such that when nnpfc_inp_order_idc is equal to 1 (i.e., only chroma is present in the input tensor), the variable strengthControlScaledVal is derived from the variable inpTensorBitDepthC (additions indicated with underline): (JVET-AD0078 item 3, BC#026)

if( nnpfc_inp_format_idc  = =  1 ) {
	if( nnpfc_inp_order_idc  = =  0  | |  nnpfc_inp_order_idc  = =  2  | |  nnpfc_inp_order_idc  = =  3 )
		strengthControlScaledVal = Floor ( StrengthControlVal * ( ( 1  <<  inpTensorBitDepthY ) − 1 ) )
	else if(nnpfc_inp_order_idc  = =  1 )
		strengthControlScaledVal = Floor ( StrengthControlVal * ( ( 1  <<  inpTensorBitDepthC ) − 1 ) )
} else
	strengthControlScaledVal = StrengthControlVal

  - Fix an asserted bug on value assignment of StrengthControlVal in the VVC spec, by setting StrengthControlVal equal to ( SliceQpY + 48 ) ÷ 111, instead of SliceQpY ÷ 63, where SliceQpY is that of the first slice of currCodedPic. (JVET-AD0233 item 5, BC#025)

BoG discussions for the items in this category were chaired by Miska M. Hannuksela.

BoG recommendation: Adopt item 14)a.

It turned out 14)b and 14)c are actually proposing the same.

BoG recommendation: Adopt item 14)c.

BoG recommendation on item 14)d: Set StrengthControlVal equal to ( SliceQpY + QpBdOffset ) ÷ ( 63 + QpBdOffset), instead of SliceQpY ÷ 63, where SliceQpY is that of the first slice of currCodedPic.

- On additional NNPF capabilities

  - Specify NNPF purposes with downsampling capabilities, including chroma downsampling, resolution downsampling, picture rate downsampling, and bit depth downsampling, using one of the following three options: (JVET-AD0093)

    - Specify new purposes with capabilities of chroma downsampling, resolution downsampling, picture rate downsampling, and bit depth downsampling. (JVET-AD0093 option 1)

    - Extend the current purposes of chroma upsampling, resolution upsampling, picture rate upsampling, and bit depth upsampling chroma resampling to resolution resampling, picture rate resampling, and bit depth resampling, respecively, with both upsampling capabilities and downsampling capabilities. (JVET-AD0093 option 2)

    - Based on either JVET-AD0093 option 1 or 2, only specify NNPF purposes with downsampling capabilities for a subset of the four, e.g., only for resolution resampling. (JVET-AD0093 option 3)

  - Specify NNPF purposes involving depth, using one of the following three options: (JVET-AD0094)

    - Add depth generation as an additional NNPF purpose. (JVET-AD0094 option 1)

    - Depth pictures may be used as input to an NNPF to assist other post processing tasks. (JVET-AD0094 option 2)

    - Combination of JVET-AD0094 options 1 and 2: Add depth generation as an additional NNPF purpose, and depth pictures may be used as input to an NNPF to assist post processing tasks. (JVET-AD0094 option 3)

  - Add a value to the nnpfc_purpose syntax element for a colour range conversion purpose (e.g., SDR to HDR conversion). When the colour range conversion purpose is indicated by nnpfc_purpose, the nnpfc_colour_primaries, nnpfc_transfer_characteristics and nnpfc_matrix_coeffs syntax elements are present in the SEI message. This replaces the nnpfc_separate_colour_description_present_flag. (JVET-AD0154 item 1)

  - Add a value to the nnpfc_purpose syntax element for a deinterlacing purpose. (JVET-AD0154 item 2, JVET-AD0240 item 1)

    - When the deinterlacing purpose is indicated by nnpfc_purpose, the nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples syntax elements are present in the SEI message, and the width and height of the output pictures are allowed to be smaller than the width and height of the input pictures. (JVET-AD0154 item 2)

  - Add new NNPF purposes, including background replacement/blurring, film gran synthesis, video stabilization, face/object redaction, visualization for computer vision task, and video retargeting. (JVET-AD0240 item 1)

  - Change the coding of nnpfc_purpose from u(16) to u(32), and add a syntax element to indicate how video format changes. (JVET-AD0240 item 2)

- On NNPFC overlap

  - It is asserted that the current StoreOutputTensor equations store the filtered picture from the outputTensor without considering the overlap regions. Make either of the following changes: (JVET-AD0104)

    - Add overlap removal in Equation 93. (JVET-AD0104 option 1, preferred by the proponent)

    - The post processing filter takes nnpfc_overlap as an additional argument and performs the overlap removal. (JVET-AD0104 option 2)

BoG discussions for the items in this category were chaired by Ye-Kui Wang.

It was clarified by the proponent that the intent was to fix a bug.

It was commented that it was not a bug, but the current text would require the NNPF to remove the overlap region before putting data into the output tensor.

A: Add, in a NOTE, an expression saying something like "The PostProcessingFilter removes the overlap regions and the output from PostProcessingFilter (i.e. outputTensor) does not include overlap regions".

B: Add, not in NOTE, the expression as in A.

C: B plus replacing " PostProcessingFilter( inputTensor)" with " PostProcessingFilter( inputTensor, nnpfc_overlap)".

BoG recommendation: Adopt item 16)a.ii without replacing "PostProcessingFilter( inputTensor)" with "PostProcessingFilter( inputTensor, nnpfc_overlap)".

- On value ranges of ue(v)-coded syntax elements

  - (Ignore this if to specify nnpfc_padding_type as an NNPF activation parameter as in JVET-AD0089, part of item 5)a.) Update the specification of the value range of nnpfc_padding_type to take into account that the values in the range 5 to 15, inclusive, are reserved, as follows (removals indicated with italics and additions indicated with underline): (JVET-AD0091 item 5, BC#021)

nnpfc_padding_type indicates the process of padding when referencing sample locations outside the boundaries of the cropped decoded output picture as described in Table 23. The value of nnpfc_padding_type shall be in the range of 0 to 15, inclusive 0 to 4, inclusive, in bitstreams conforming to this edition of this document. Values of 5 to 15, inclusive, for nnpfc_padding_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_padding_type in the range of 5 to 15, inclusive. Values of nnpfc_padding_type greater than 15 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

  - (Ignore this if to specify nnpfc_luma_padding_val, nnpfc_cb_padding_val, and nnpfc_cr_padding_val as NNPF activation parameters as in JVET-AD0089, part of item 5)a.) Add missing value ranges of ue(v)-coded syntax elements nnpfc_luma_padding_val, nnpfc_cb_padding_val, and nnpfc_cr_padding_val, as follows: (JVET-AD0091 item 5, BC#022)

    - The value of nnpfc_luma_padding_val (or the corresponding variable) shall be in the range of 0 to 2inpTensorBitDepthY − 1, inclusive.

    - The value of nnpfc_cb_padding_val (or the corresponding variable) shall be in the range of 0 to 2inpTensorBitDepthC − 1, inclusive.

    - The value of nnpfc_cr_padding_val (or the corresponding variable) shall be in the range of 0 to 2inpTensorBitDepthC − 1, inclusive.

- Editorial changes

  - Change all instances of "nnpfc_parameter_parameter_type_idc" to "nnpfc_parameter_type_idc". (JVET-AD0078 item 1)

  - Change "4:2:4 format" to "4:4:4 format". (JVET-AD0078 item 2)

  - Restructure the semantics of nnpfc_purpose including table 20 with the definition of the nnpfc_purpose. (JVET-AD0154 item 3)

  - For nnpfc_purpose, instead of singalling different meaningful bits in a single syntax element, split the syntax element into several separate flags. The splitting allows the specification to refer to a meaningful syntax element name instead of and operation of the syntax element and a mask value. (JVET-AD0370)

- Region-based filtering

  - Add both of the following as the high-level design decisions for VSEI v4 based on which detailed syntax and semantics can be presented in the upcoming JVET meetings (JVET-AD0058):

    - Either an NNPFA SEI message extension for indicating regions or an NNPF regional activation SEI message

    - In NNPFC SEI message, a new nnpfc_auxiliary_inp_idc value for providing indications about regions as input to the NNPF