| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0096-v1 |
| --- | --- |

| Title: | [AHG8] Study on using different VTM versions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Jacob Ström | Email: | christopher.hollmann@ericsson.com, jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution the results of a study on different VTM versions in the context of machine analysis of coded video content are reported. Eight different versions of VTM from VTM-13 to VTM-20 are compared to the VTM-12 anchor.

It is reported that even though the variation in PSNR between different VTM version is rather small, the variation in mAP/MOTA performance is in some cases substantial.

It is proposed to replace the current VTM-12 anchor with a more recent one such as VTM-20.

# Introduction

At the 29th JVET meeting AHG8 on the study of optimizations for encoders and receiving systems for machine analysis of coded video content was established. One mandate of this AHG was to “coordinate software development, and investigate the possibility of migrating the software basis to newest VTM version” [1].

In this contribution a study of newer VTM versions is reported. As VTM-12.0 is used as the anchor, tests have been done with all newer main versions of the VTM software, namely VTM-13.0, VTM-14.0, VTM-15.0, VTM-16.0, VTM-17.0, VTM-18.0, VTM-19.0 and VTM-20.0.

The tests were performed according to the common test conditions (CTC) defined at the 30th JVET meeting in Antalya, TR [2].

# Test results

Tests were run for all three configurations: random access, low delay and all intra. Both video datasets were tested. Detailed results can be found in the attached Excel documents. The anchor is in all cases VTM-12.0.

As there are a number of cases where the rate points are non-monotonic, the reported results are taken from the Pareto-mAP/Pareto-MOTA column of the reporting template. In cases where all rate points are monotonic, these results are identical to the ones using all six rate points. For information, results where the rate points are non-monotonic are underlined.

As additional information, the average BD-rate for Y-PSNR for all sequences is shown in the final row of the tables with the per-sequence information.

## Random access

The results for the SFU-HW dataset on sequence-level are listed in Table 1, and Table 2 shows the class-wise results. For TVD results can be found in Table 3. Notable changes compared to the respective previous version include, based on the difference of the configuration files for the different versions:

- VTM-13: deblocking

- VTM-16: deblocking, temporal filter

- VTM-17: deblocking, temporal QP offsets

- VTM-20: MaxMergeRdCandNumTotal

Table 1. Results for SFU-HW sequence-wise in random access configuration

| Sequence | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic | 2.62% | −0.66% | −0.66% | −2.91% | −20.36% | −20.36% | −20.36% | −23.43% |
| ParkScene | 14.28% | 14.51% | 14.51% | 17.66% | 3.72% | 3.72% | 3.72% | 21.79% |
| Cactus | 4.59% | 7.71% | 7.71% | −11.36% | 8.04% | 8.04% | 8.04% | −1.71% |
| Basketball-Drive | 6.63% | 4.86% | 4.86% | −2.33% | 3.05% | 3.05% | 3.05% | 0.31% |
| BQTerrace | −4.37% | −6.04% | −6.04% | −5.42% | −7.16% | −7.16% | −7.16% | −6.82% |
| Basketball-Drill | −0.76% | −6.68% | −6.68% | −3.78% | −8.81% | −8.81% | −8.81% | −2.21% |
| BQMall | −0.05% | 9.62% | 9.62% | 9.81% | −1.46% | −1.46% | −0.98% | 5.00% |
| PartyScene | 4.42% | −11.37% | −11.37% | 2.38% | −5.10% | −5.10% | −5.10% | −7.10% |
| RaceHorsesC | 14.47% | 7.49% | 7.49% | 10.11% | −1.23% | −1.23% | −1.23% | −5.07% |
| Basketball-Pass | −5.64% | 0.19% | 0.19% | 5.25% | −2.77% | −2.77% | −2.77% | −7.35% |
| BQSquare | −0.38% | −0.07% | −0.07% | −6.45% | −2.78% | −2.78% | −2.78% | −0.58% |
| Blowing-Bubbles | −8.46% | 3.23% | 3.23% | −6.07% | −1.62% | −1.62% | −1.62% | −4.40% |
| RaceHorsesD | −3.78% | 3.76% | 3.76% | 0.63% | −10.39% | −10.39% | −10.39% | −8.51% |
| Average | 1.81% | 2.04% | 2.04% | 0.58% | −3.61% | −3.61% | −3.57% | −3.08% |
| Y-PSNR | −0.31% | −0.50% | −0.50% | −0.37% | −0.88% | −0.88% | −0.88% | −0.94% |

Table 2. Results for SFU-HW class-wise in random access configuration

| Class | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class AB | 5.02% | 4.12% | 4.12% | −3.88% | 0.90% | 0.90% | 0.90% | −2.92% |
| Class C | 7.74% | 1.86% | 1.86% | 6.06% | −3.31% | −3.31% | −3.31% | −1.78% |
| Class D | −5.72% | 1.32% | 1.32% | −8.76% | −3.52% | −3.52% | −3.52% | −5.91% |
| Average | 2.35% | 2.43% | 2.43% | −2.19% | −1.98% | −1.98% | −1.98% | −3.54% |

Table 3. Results for TVD in random access configuration

| Sequence | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVD-01_01 | 3.23% | 4.54% | 4.54% | −6.13% | 3.55% | 3.55% | 3.50% | 4.44% |
| TVD-01_02 | −10.82% | −9.03% | −9.03% | 6.31% | 16.60% | 16.60% | 16.70% | −3.73% |
| TVD-01_03 | −7.12% | −3.53% | −3.53% | −21.47% | −10.43% | −10.43% | −10.43% | −8.20% |
| TVD-02_01 | 43.11% | 21.39% | 21.39% | 21.46% | −2.51% | −2.51% | −2.52% | 10.63% |
| TVD-03_01 | 5.53% | −4.31% | −4.31% | 5.00% | 1.11% | 1.11% | 1.10% | 0.65% |
| TVD-03_02 | −16.43% | −17.54% | −17.54% | −11.49% | −14.93% | −14.93% | −12.99% | −8.79% |
| TVD-03_03 | 7.66% | 3.39% | −6.36% | 0.62% | −8.31% | −8.31% | −6.97% | −5.80% |
| Average | 3.59% | −0.73% | −2.12% | −0.81% | −2.13% | −2.13% | −1.66% | −1.54% |
| Y-PSNR | −0.16% | −0.14% | −0.14% | −0.71% | −1.67% | −1.67% | −1.68% | −1.71% |

## Low delay

The results for the SFU-HW dataset on sequence-level are listed in Table 4, and Table 5 shows the class-wise results. For TVD results can be found in Table 6. Notable changes compared to the respective previous version include, based on the difference of the configuration files for the different versions:

- VTM-13: deblocking

- VTM-15: temporal filter

- VTM-16: deblocking, temporal filter

- VTM-17: deblocking, temporal QP offsets

- VTM-20: MaxMergeRdCandNumTotal

Table 4. Results for SFU-HW sequence-wise in low delay configuration

| Sequence | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic | 0.00% | 0.00% | −2.95% | −18.39% | −0.77% | −0.77% | −0.77% | −2.53% |
| ParkScene | −0.12% | −0.12% | −14.48% | −5.26% | −10.88% | −10.88% | −10.88% | −4.77% |
| Cactus | 0.00% | 0.00% | 2.01% | −7.86% | 8.82% | 8.82% | 8.82% | 5.87% |
| Basketball-Drive | 0.00% | 0.00% | 4.71% | 6.68% | 9.81% | 9.81% | 9.81% | 4.95% |
| BQTerrace | 0.00% | 0.00% | −8.69% | −0.06% | −3.14% | −3.14% | −3.14% | −7.12% |
| Basketball-Drill | 0.00% | 0.00% | −0.75% | 0.78% | −0.18% | −0.18% | −0.36% | 1.72% |
| BQMall | 0.12% | 0.12% | −6.51% | −1.91% | −6.31% | −6.31% | −5.97% | −3.13% |
| PartyScene | 0.00% | 0.00% | −7.96% | −4.39% | −6.54% | −6.54% | −6.54% | −7.71% |
| RaceHorsesC | 0.00% | 0.00% | 7.98% | 7.72% | 4.06% | 4.06% | 4.06% | −2.84% |
| Basketball-Pass | −0.13% | −0.13% | −14.38% | 0.36% | −2.18% | −2.18% | −2.18% | −7.91% |
| BQSquare | 0.00% | 0.00% | 6.61% | 4.29% | −13.43% | −13.43% | −13.43% | −8.87% |
| Blowing-Bubbles | 0.00% | 0.00% | −4.07% | 3.51% | 8.79% | 8.79% | 8.79% | −1.88% |
| RaceHorsesD | 0.00% | 0.00% | 0.80% | −0.16% | −4.24% | −4.24% | −4.24% | −5.67% |
| Average | −0.01% | −0.01% | −2.90% | −1.13% | −1.25% | −1.25% | −1.23% | −3.07% |
| Y-PSNR | 0.00% | 0.00% | −0.24% | −0.12% | −0.92% | −0.92% | −0.93% | −1.02% |

Table 5. Results for SFU-HW class-wise in low delay configuration

| Class | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class AB | −0.12% | −0.12% | −0.61% | −8.56% | 1.28% | 1.28% | 1.28% | −1.01% |
| Class C | −0.10% | −0.10% | −3.55% | 0.17% | −3.90% | −3.90% | −3.90% | −4.25% |
| Class D | −0.06% | −0.06% | −0.29% | 4.45% | 3.17% | 3.17% | 3.17% | −0.43% |
| Average | −0.09% | −0.09% | −1.48% | −1.31% | 0.18% | 0.18% | 0.18% | −1.90% |

Table 6. Results for TVD in low delay configuration

| Sequence | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVD-01_01 | 1.64% | 1.64% | 18.93% | 7.98% | −11.36% | −11.36% | −10.54% | −7.63% |
| TVD-01_02 | 0.51% | 0.51% | −10.20% | −19.01% | −21.73% | −21.73% | −21.73% | −15.55% |
| TVD-01_03 | −1.22% | −1.22% | 14.34% | 1.40% | −20.86% | −20.86% | −16.87% | −6.27% |
| TVD-02_01 | −10.45% | −10.45% | −13.64% | −10.98% | −35.46% | −35.46% | −35.46% | −23.50% |
| TVD-03_01 | 0.21% | 0.21% | 2.05% | −2.00% | −6.08% | −6.08% | −6.36% | −2.95% |
| TVD-03_02 | 1.52% | 1.52% | −41.30% | −4.36% | −1.91% | −1.91% | 5.86% | −9.41% |
| TVD-03_03 | −1.35% | −1.35% | 4.36% | 9.16% | −8.15% | −8.15% | −6.20% | −5.18% |
| Average | −1.30% | −1.30% | −3.64% | −2.54% | −15.08% | −15.08% | −13.04% | −10.07% |
| Y-PSNR | 0.00% | 0.00% | −0.26% | −0.93% | −4.58% | −4.58% | −4.57% | −4.65% |

## All intra

The results for the SFU-HW dataset on sequence-level are listed in Table 7, and Table 8 shows the class-wise results. For TVD results can be found in Table 9. Notable changes compared to the respective previous version include, based on the difference of the configuration files for the different versions:

- VTM-16: deblocking

Table 7. Results for SFU-HW sequence-wise in all intra configuration

| Sequence | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic | 0.00% | 0.00% | 0.00% | −5.92% | −5.92% | −5.92% | −5.92% | −7.69% |
| ParkScene | 0.00% | 0.00% | 0.00% | 0.15% | 0.15% | 0.15% | 0.15% | 9.24% |
| Cactus | 0.00% | 0.00% | 0.00% | 5.76% | 5.76% | 5.76% | 5.76% | 7.04% |
| BasketballDrive | 0.00% | 0.00% | 0.00% | 1.17% | 1.17% | 1.17% | 1.17% | 1.57% |
| BQTerrace | 0.00% | 0.00% | 0.00% | 2.39% | 2.39% | 2.39% | 2.39% | 3.79% |
| BasketballDrill | 0.00% | 0.00% | 0.00% | −0.95% | −0.95% | −0.95% | −0.95% | 2.60% |
| BQMall | 0.00% | 0.00% | 0.00% | 1.69% | 1.69% | 1.69% | 1.68% | 4.02% |
| PartyScene | 0.00% | 0.00% | 0.00% | 4.85% | 4.85% | 4.85% | 4.85% | 4.00% |
| RaceHorsesC | 0.00% | 0.00% | 0.00% | 5.38% | 5.38% | 5.38% | 5.38% | 7.42% |
| BasketballPass | 0.00% | 0.00% | 0.00% | −4.01% | −4.01% | −4.01% | −4.01% | −2.17% |
| BQSquare | 0.00% | 0.00% | 0.00% | −1.64% | −1.64% | −1.64% | −1.64% | −0.87% |
| BlowingBubbles | 0.00% | 0.00% | 0.00% | −1.43% | −1.43% | −1.43% | −1.43% | 0.01% |
| RaceHorsesD | 0.00% | 0.00% | 0.00% | 6.05% | 6.05% | 6.05% | 6.05% | 6.86% |
| Average | 0.00% | 0.00% | 0.00% | 1.04% | 1.04% | 1.04% | 1.04% | 2.76% |
| Y-PSNR | 0.00% | 0.00% | 0.00% | −0.22% | −0.22% | −0.22% | −0.22% | −0.21% |

Table 8. Results for SFU-HW class-wise in all intra configuration

| Class | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class AB | 0.00% | 0.00% | 0.00% | 1.02% | 1.02% | 1.02% | 1.02% | 3.26% |
| Class C | 0.00% | 0.00% | 0.00% | 1.93% | 1.93% | 1.93% | 1.93% | 3.47% |
| Class D | 0.00% | 0.00% | 0.00% | −1.15% | −1.15% | −1.15% | −1.15% | 0.45% |
| Average | 0.00% | 0.00% | 0.00% | 0.60% | 0.60% | 0.60% | 0.60% | 2.39% |

Table 9. Results for TVD in all intra configuration

| Sequence | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVD-01_01 | 0,03% | 0,03% | 0,03% | 1.39% | 1.39% | 1.39% | 1.23% | −2.84% |
| TVD-01_02 | 0,10% | 0,10% | 0,10% | −4.52% | −4.52% | −4.52% | −4.52% | −3.10% |
| TVD-01_03 | 0,11% | 0,11% | 0,11% | −6.20% | −6.20% | −6.20% | −6.21% | −11.33% |
| TVD-02_01 | −0,24% | −0,24% | −0,24% | 15.20% | 15.20% | 15.20% | 15.20% | 11.86% |
| TVD-03_01 | −0,30% | −0,30% | −0,30% | 0.97% | 0.97% | 0.97% | 0.97% | 0.30% |
| TVD-03_02 | −0,48% | −0,48% | −0,48% | −2.85% | −2.85% | −2.85% | −2.85% | −0.15% |
| TVD-03_03 | 0,25% | 0,25% | 0,25% | −4.80% | −4.80% | −4.80% | −4.80% | −8.19% |
| Average | −0,08% | −0,08% | −0,08% | −0.12% | −0.12% | −0.12% | −0.14% | −1.92% |
| Y-PSNR | 0.00% | 0.00% | 0.00% | −0.93% | −0.93% | −0.93% | −0.93% | −0.93% |

## Summary

In Table 10 the results for the different configurations are summarized. The results are taken from the “Average” rows in Table 1, Table 3, Table 4, Table 6, Table 7, and Table 9. The largest jump occurs from VTM-16 to VTM-17 in TVD / LD. Besides this one case, the results on the dataset basis look quite stable, and there are no excessively large differences when looking at the averages over the entire datasets.

Table 10. Summarized results for all configurations

| Dataset / Configuration | VTM13 | VTM14 | VTM15 | VTM16 | VTM17 | VTM18 | VTM19 | VTM20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SFU-HW / RA | 1.81% | 2.04% | 2.04% | 0.58% | −3.61% | −3.61% | −3.57% | −3.08% |
| SFU-HW / LD | −0.01% | −0.01% | −2.90% | −1.13% | −1.25% | −1.25% | −1.23% | −3.07% |
| SFU-HW / AI | 0.00% | 0.00% | 0.00% | 1.04% | 1.04% | 1.04% | 1.04% | 2.76% |
| TVD / RA | 3.59% | −0.73% | −2.12% | −0.81% | −2.13% | −2.13% | −1.66% | −1.54% |
| TVD / LD | −1.30% | −1.30% | −3.64% | −2.54% | −15.08% | −15.08% | −13.04% | −10.07% |
| TVD / AI | −0,08% | −0,08% | −0,08% | −0.12% | −0.12% | −0.12% | −0.14% | −1.92% |

# Discussion

In this contribution we have tested different VTM versions that are newer than the VTM version that is currently used for the anchor in the study on optimizations for encoders and receiving systems for machine analysis of coded video content.

There are several conclusions that can be drawn from this study:

- Having more data to determine average performance values is a benefit. While the performance on a per-sequence basis can vary by quite large numbers from one VTM version to the next, the averages over multiple sequences tend to be more consistent.

- Changes that have minor impact on PSNR results can have a large impact on the machine analysis performance. There are several examples where the Y-PSNR result barely changes, but the BD-rate for the task performance changes more drastically.

- Different VTM versions produce similar bitrate and machine task performance, i.e., there is no significant BD-rate change purely due to varying bitrates.

The reason for the BD-rate results varying quite strongly from one version to the next can likely be attributed to one of various reasons:

- The number of things being measured is small. When using PSNR as distortion metric, we get one measurement in each sample. Hence, there are a lot of measurements to average the noise over. However, in VCM we get one measurement per object, and there are a lot fewer objects in a video than pixels.

- The measurements are binary. In regular PSNR, we compare one grayscale value to another grayscale value. If the encoding is better, they will come closer, whereas if the encoding is worse, they will move farther apart. But there is no “catastrophic failure”. The opposite is true in VCM. Either the object is detected, and we get 100% success, or it is not detected, and we get 0% success.

Based on the outcome of this study we propose to update the anchors to VTM-20.0. Moving to a newer VTM version has various advantages such as:

- Non-normative tools have been improved. For example, the MCTF received an update in VTM-16 that gives an additional performance benefit. Another example is the RPR tool, which was improved significantly since version 12.

- New tools have been added to the encoder. The Block Importance Mapping (BIM) tool was added in VTM-16. Testing the performance of tools such as BIM for this application is much easier if they are already included in the software basis.

- Various bugs have been found and fixed in newer versions of the software.

# References

| [1] | J.-R. Ohm, "Meeting Report of the 29th JVET Meeting," JVET-AC1000, Teleconference, January 2023. |
| --- | --- |
| [2] | S. Liu and C. Hollmann, "Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content," JVET-AD2031, Antalya, TR, April 2023. |

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).