| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0164-V1 |
| --- | --- |

| Title: | EE1-1.2 Complexity-performance tradeoff of decomposition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Yun Li Marta Karczewicz | Email: | dmytror@qti.qualcomm.com yli30@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AC meeting and includes model architecture, training process and training test set. In test EE1-1.2 complexity-performance trade-off optimization of the convolution decomposition in unfied filter model have been studied.

This test introduces reduced ranking of the two-components decomposition in residue blocks of the backbone. With parameter C31 reduced to 48, complexity of the results model is reduced by 10% to 426kMAC/pixel (block-wise 128x128). To mitigate the impact of the rank reduction on decomposition symmetry, order of the decomposition (horizontal-vertical or vertical-horizontal) is alternated every block in the backbone. The method was tested with HOP Stage 1 training and AI results are reported. Training for latter stages of HOP is ongoing, results to be provided once available.

Comparing to EE1-1.0 HOP, proposed filter provides {0.0%, 0.3%, 0.11%} bd-rate penalty in AI configuration with 10% complexity reduction in kMAC/pixel terms. Comparing to NNVC5.0 with NN tools OFF, proposed filter provides: {-7.9%, -19.0%, -20.2%} bd-rate gain in AI configuration. Comparing to NNVC-5.0 (NN Intra and LOP enabled), proposed filter being tested with NN Intra enabled provides: {-XX%, -XX%, -XX%} bd-rate gain in AI configuration.

It is proposed to study the proposed method in the next round of EE.

# Introduction

A unified filter architecture for the High-performance Operation Point (HOP), shown in Figure 1, integrated multiple previous proposed contributions, e.g. multiscale feature extraction backbone with the two-component decomposition [3], and reducing input feature computations [4]. Parameters of HOP, its training process and assessment procedure were specified in JVET-AD2023 and status of its development between AD and AE meeting were reported in JVET-AE0042. Stage1 model and its performance was fully assessed (on AI results) at the time of submission of this contribution.

Figure 9. HOP unified architecture (joined luma and chroma)

# Proposed method

Proposed method tested in sub-test 1.2.1 introduces reduced ranking of the two-components decomposition in residue block of the backbone, modifying C31 parameter, of Unified Filter Architecture. To alleviate introduced decomposition asymmetry, with reduced rank being used in horizontal direction only, order of the decomposition (horizontal/vertical) is being switched every residue block in the backbone. Two types of the residue block in backbone introduced. Type 1 (T1), shown in Figure 2.a, residue bock implements 3x1 convolution with C31=48, followed by 1x3 convolution and Type 2 (T2), shown in Figure 2.b, residue bock first performs 1x3 convolution with C31=48, followed by the 3x1 convolution.

- b.

Figure 2 Proposed backbone residue blocks: a. - Type 1, b. - Type 2.

Figure 3 shows integrated NN filter model with blocks T1 and T2 being switching in processing order.

Figure 3 Proposed model with switched order decompositions (Type 1 and Type2) and reduced rank, C31=48.

The model was trained following HOP Stage 1 procedure, as only available at the of submission. Training for latter stages of HOP is ongoing, results to be provided once available.

Parameters of the training and inference testing for proposed method are shown in Table 1 and Table 2, respectively.

Table1. Network information for the inference

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.45M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 426 (336 frame-wise) |
|  | Total Conv. Layers |  | 154 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 2.59/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training (HOP aligned)

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | Framework: | PyTorch v1.8 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 39 |
|  | Batch size: | 64 |
|  | Training time: | ~240h |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Simulation results

Simulations are performed based on the common test conditions for neural network-based video coding technology [5]. The NNVC SW was used for inference test of the proposed filter model.

Comparative results of proposed method vs. EE1.0 HOP are shown in Table 3. NnIntra tool was disabled for both, anchor and test. EE1.0 HOP results have been produced with the anchor model for Stage 1 distributed as a part of NNVC SW [6].

Table 3 Proposed method vs EE1-1.0 (HOP)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Test Over EE1-1.0 (HOP) |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -0.05% | -0.13% | -0.76% | 98% | 89% | 0% |
| Class A2 | 0.02% | 0.56% | 0.52% | 96% | 92% | 0% |
| Class B | 0.10% | 0.33% | -0.02% | 99% | 96% | 0% |
| Class C | 0.04% | 0.30% | 0.03% | 100% | 92% | 0% |
| Class E | 0.02% | 0.10% | 0.90% | 97% | 93% | 0% |
| Overall | 0.04% | 0.25% | 0.11% | 98% | 93% | 0% |
| Class D | 0.06% | 0.92% | 0.76% | 98% | 97% | 0% |

Comparative performance of the proposed method (NnIntra tools OFF) vs NNVC5.0 (NN Tools OFF) are shown in Table 3.

Table 3 Proposed method vs NNVC 5.0 (NN tools OFF)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Test Over NNVC 5.0 (NN Tools OFF) |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -6.81% | -16.76% | -21.25% | 424% | 181364% | 0% |
| Class A2 | -7.19% | -20.06% | -19.09% | 270% | 154782% | 0% |
| Class B | -6.93% | -18.76% | -18.52% | 240% | 123820% | 0% |
| Class C | -8.21% | -18.47% | -21.14% | 177% | 72898% | 1% |
| Class E | -10.57% | -21.09% | -21.51% | 268% | 124355% | 0% |
| Overall | -7.85% | -18.97% | -20.15% | 256% | 121831% | 0% |
| Class D | -8.04% | -17.41% | -20.83% | 160% | 64423% | 1% |

Comparative results of proposed method vs. NNVC 5.0 (NN tools OFF) are shown in Table 3. NnIntra tool was disabled for both, anchor and test.

Table 4 Proposed method with NN Intra ON vs NNVC-5.0 (NN Intra and LOP ON)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Test Over NNVC 5.0 (NN Tools OFF) |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 |  |  |  |  |  | 0% |
| Class A2 |  |  |  |  |  | 0% |
| Class B | -2.78% | -14.17% | -13.87% | 152% | 2178% | 0% |
| Class C | -3.76% | -12.46% | -15.16% | 131% | 1966% | 0% |
| Class E | -4.52% | -17.02% | -17.64% | 181% | 2620% | 0% |
| Overall |  |  |  |  |  | 0% |
| Class D | -3.10% | -12.54% | -14.98% | 126% | 1738% | 0% |

# Conclusion

In sub-test 1.2.1, reduced ranking of the two-components decomposition in residual block of the backbone were tested. With parameter of the decomposition C31 reduced to 48, instead of 64, complexity of the results model is reduced by 10% to 426kMAC/pixel. To mitigate the impact of the rank reduction on decomposition asymmetry, order of the decomposition (horizontal/vertical) was switched every second residual block. The method was tested with HOP Stage 1 training and AI results reported.

Comparing to EE1-1.0 HOP, proposed filter provides {0.1%, 0.4%, 0.3%} bd-rate penalty in AI configuration with 10% complexity reduction in kMAC/pixel terms. Comparing to NNVC5.0 with NN tools OFF, proposed filter provides: {-7.8%, -18.9%, -20.0%} bd-rate gain in AI configuration. Comparing to NNVC-5.0 (NN tools ON), proposed filter being tested with NnIntra enabled provides: {-XX%, -XX%, -XX%} bd-rate gain in AI configuration.

It is proposed to study the proposed method in the next round of EE.

# References

- EE1-1.1: Complexity reduction on neural-network loop filter, JVET-AD0156, J.N. Shingala, A. Shyam, A. Suneja, S. P Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy (Dolby).

- AhG14 & AHG11: Report on AhG teleconference on high operation point (HOP) unified filter training, JVET-AE0042, E. Alshina, F. Galpin.

- EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6, JVET-AD0211, JVET-AD0211, Y. Li, S. Eadie, D. Rusanovskyy, M. Karczewicz.

- EE1.3: Reduced complexity CNN-based in-loop filtering, JVET-AD0205, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

- HOP Stage I model and simulation results, available online: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/hop_models_draft/models/HOP/stage1/QC/.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).