| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0165-V2 |
| --- | --- |

| Title: | EE1-4.4: Low complexity NN filter with design elements of Unified Filter Architecture and EE1-1.2 and EE1-1.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yun Li Dmytro Rusanovskyy Marta Karczewicz | Email: | yli30@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AC meeting and includes model architecture, training process and training test set. Unified data set creation, training, and performance assessment of the HOP architecture have been ongoing between JVET-AD and AE meetings, and Stage1 model and its performance was fully evaluated (on AI results) at the time of submission of this contribution. A Low-complexity Operation Point (LOP) in-loop filter, with complexity of 16.2 kMAC/pixel (frame-wise), was also adopted in JVET-AC meeting to the NNVC software to serve as an anchor for further HOP development.

In this contribution, Unified Filter Architecture of HOP was adjusted to meet complexity constraints of LOP. This was achieved by reducing complexity of the headblock, backbone and utilizing the CP decomposition in the backbone blocks. End-to-end training of this model was conducted following the process specified for HOP Stage1. Training for latter stages of HOP is ongoing, results to be provided once available.

Achieved model complexity in kMAC/pixel: 14.8 (frame-wise); 16.7 (block-wise of block 256x256); 18.7 (block-wise of block 128x128).

Comparing to NNVC-5.0 HOP Anchor (NN Intra and LOP filter enabled), the proposed filter (quantized model) provides: {0.08%, -5.41%, -5.83%} in AI test configuration. For the joint YCbCr metric, the BD-rate gain is -1.1%.

Comparing to NNVC-5.0 (NN tools OFF), the proposed filter (quantized model) provides: {-4.42%, -10.55%, -11.17%} in AI test configuration.

Comparing to NNVC-5.0 (NN tools OFF), the proposed filter (quantized model) with NN Intra enabled provides: {-7.73%, -13.64%, -14.28%} in AI test configuration.

It is proposed to study the proposed method in the next round of EE.

# Introduction

A unified filter architecture for the High-performance Operation Point (HOP) was defined at JVET-AC meeting. The HOP includes unified filter architecture, training process and training test set. Unified data set creation, training, and performance assessment of the HOP architecture for high performance operational point have been ongoing between JVET-AD and AE meetings. Its progress and status have been reported in JVET-AE0042[2], and Stage1 model and its performance was fully assessed (on AI results) at the time of submission of this contribution.

A unified filter architecture for the High-performance Operation Point (HOP), shown in Figure 1, integrated multiple previous proposed contributions, e.g., multiscale feature extraction backbone with the two-component decomposition [3], and reducing input feature computations [4]. Parameters of HOP, its training process and assessment procedure were specified in JVET-AD2023 and status of its development between AD and AE meeting were reported in JVET-AE0042.

Figure 1. HOP unified model (joined luma and chroma)

# Proposed method

In this contribution, we rely on the HOP filter architecture, the training dataset and end-to-end training process to derive a filter that meets the complexity constraints of LOP.

In order to reduce the complexity of the filter to below 20kMAC/pixel, the following changes have been introduced. The complexities of the head block were reduced by decreasing the number of the extracted feature maps. In the backbone the number of residual blocks was reduced to 11, and the complexity of the residual blocks was further reduced by employing CP decomposition along with the reduction of the features.. In the tail block following the backbone, the convolution was also decomposed. In addition, the activations layer of the head blocks was removed, and some layers of the CP decomposition were merged. The proposed architecture is shown in Fig. 2.

Training was conducted end-to-end, following the HOP specified procedure, without computing tensor decomposition beforehand.

Figure 2. The proposed filter architecture. In the blocks marked with “Separable”, depth-wise convolution is performed.

The information for the inference and the training stage is presented in Table 1 and 2, respectively.

Table1. Network information for the inference

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 56544 /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 18.7 (14.8 frame-wise) |
|  | Total Conv. Layers |  | 67 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 0.125/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | Framework: | PyTorch v1.8 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 130 |
|  | Batch size: | 64 |
|  | Training time: | ~240h |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Simulation results

Simulations are performed based on the common test conditions for neural network-based video coding technology [5]. The NNVC SW was used for the inference test of the proposed new model. The integer implementation has been tested and the results are reported below.

Comparative performance of the proposed method (quantized model) vs. HOP Anchor, (NNVC5.0+NN Intra and LOP filter enabled) are shown in Table 3.:

Table 3 Proposed method (int 16) vs NNVC-5.0 (NN Intra and LOP filter enabled)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC5 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | 0.16% | -5.88% | -7.45% | 95% | 126% | 0% |
| Class A2 | -0.07% | -5.45% | -4.61% | 97% | #DIV/0! | 0% |
| Class B | 0.10% | -4.98% | -5.02% | 97% | 124% | 0% |
| Class C | 0.11% | -5.00% | -5.30% | 98% | 122% | 0% |
| Class E | 0.09% | -6.18% | -7.49% | 95% | 117% | 0% |
| Overall | 0.08% | -5.41% | -5.83% | 96% | #DIV/0! | 0% |
| Class D | 0.38% | -4.92% | -5.07% | 97% | 110% | 0% |
| Class F | -0.06% | -2.75% | -3.77% | 97% | 94% | 0% |

As observed in Tables 3, the proposed filter architecture improved the coding performance against the HOP anchor by {0.08%, -5.41%, -5.83%} for luma and chroma (-1.1% of the BD-rate gain for the joined YCbCr metric), respectively, at the frame-wise complexity of 14.8 kMAC.

Comparative performance of the proposed method (quantized model) vs. NNVC-5.0 (NN tools off) is shown in Table 4.

Table 4 Proposed method (int 16) vs NNVC-5.0 (NN tools off)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC5 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -4.30% | -9.07% | -11.85% | 108% | 5524% | 0% |
| Class A2 | -4.00% | -10.63% | -9.67% | 101% | 4603% | 0% |
| Class B | -3.96% | -10.54% | -10.42% | 100% | 4567% | 0% |
| Class C | -4.32% | -11.69% | -12.10% | 99% | 3378% | 0% |
| Class E | -5.86% | -10.44% | -11.98% | 105% | 4965% | 0% |
| Overall | -4.42% | -10.55% | -11.17% | 102% | #DIV/0! | 0% |
| Class D | -4.55% | -10.19% | -11.35% | 100% | 3240% | 0% |
| Class F | -2.53% | -6.80% | -7.46% | 95% | 2959% | 0% |

Comparative performance of the proposed method (quantized model) with NN Intra enabled vs NNVC-5.0 (NN tools off) is shown in Table 5.

Table 5 Proposed method (int 16) with NN Intra enabled vs NNVC-5.0 (NN tools off)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC5 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -8.30% | -12.70% | -15.55% | 170% | 6158% | 4% |
| Class A2 | -6.69% | -13.11% | -12.25% | 171% | 5339% | 3% |
| Class B | -6.94% | -13.23% | -13.37% | 168% | 5386% | 3% |
| Class C | -7.23% | -14.34% | -14.88% | 156% | 4224% | 3% |
| Class E | -10.21% | -14.83% | -15.77% | 163% | 5588% | 5% |
| Overall | -7.73% | -13.64% | -14.28% | 165% | #DIV/0! | 4% |
| Class D | -7.23% | -12.81% | -14.19% | 151% | 4205% | 3% |
| Class F | -4.61% | -8.56% | -9.24% | 126% | 3495% | 2% |

As observed in Tablet 4, the proposed LOP improved the coding performance against the NNVC anchor by {-4.42%, -10.55%, -11.17%}, for luma and chroma, respectively, at the frame-wise complexity of 14.8 kMAC. In addition, the coding gain is {-7.73%, -13.64%, -14.28} with NN Intra enabled as shown in Table 5.

# Conclusion

In this contribution, Unified Filter Architecture was adjusted to meet complexity constraints of LOP. This was achieved by reducing complexity of the headblock, backbone and utilizing the CP  decomposition in the backbone blocks. End-to-end training of this model was conducted following the process specified for HOP. The proposed filter has a model complexity in kMAC/pixel: 14.8 (frame-wise), 16.7 (block-wise of block 256x256), and18.7 (block-wise of block 128x128).

Comparing to NNVC-5.0 HOP Anchor (NN Intra and LOP filter enabled), the proposed filter (quantized model) provides: AI: {0.08%, -5.41%, -5.83%}. Comparing to NNVC-5.0 (NN tools OFF), the proposed filter (quantized model) provides: AI: {-4.42%, -10.55%, -11.17%}, and AI: {-7.73%, -13.64%, -14.28} with NN Intra enabled for the test.

It is proposed to study the proposed method in the next round of EE.

# References

- EE1-1.1: Complexity reduction on neural-network loop filter, JVET-AD0156, J.N. Shingala, A. Shyam, A. Suneja, S. P Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy (Dolby).

- AhG14 & AHG11: Report on AhG teleconference on high operation point (HOP) unified filter training, JVET-AE0042, E. Alshina, F. Galpin.

- EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6, JVET-AD0211, JVET-AD0211, Y. Li, S. Eadie, D. Rusanovskyy, M. Karczewicz.

- EE1.3: Reduced complexity CNN-based in-loop filtering, JVET-AD0205, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).