| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0168-v2 |
| --- | --- |

| Title: | Non-EE2: Sample-based BDOF for Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Han Huang, Zhi Zhang, Cagan Selim Coban, Vadim Seregin, Marta Karczewicz | Email: | chunchic@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposed to extend the notion of sample-based bi-directional optical flow (BDOF) to refining bi-prediction samples of chroma components. The same process of sample-based BDOF in ECM is applied to each chroma sample in a coding unit, except that its per-sample motion refinement is inferred from its co-located luma components in the same coding unit. In other words, the per-sample motion refinement of a luma sample is shared across all color components. Then, each bi-prediction chroma sample is adjusted using an offset value computed by the same BDOF function of gradient vector and motion refinement. The experimental results on top of ECM-9.0 are reported, as follows:

- Class B: (Y) 0.01%, (U) -0.13%, (V) -0.03%, (Enc) 100.3%, (Dec) 100.2%

- Class C: (Y) -0.01%, (U) -0.15%, (V) -0.13%, (Enc) 99.6%, (Dec) 99.3%

- Class D: (Y) 0.00%, (U) -0.45%, (V) -0.35%, (Enc) 100.3%, (Dec) 99.9%

- Class F: (Y) -0.02%, (U) -0.11%, (V) -0.09%, (Enc) 100.3%, (Dec) 100.0%

- Class TGM: (Y) 0.01%, (U) -0.08%, (V) -0.08%, (Enc) 100.4%, (Dec) 99.5%

for Class D sequences, showing Y/U/V BD-rate of 0.02%/-0.42%/-0.35%, with 100% encoding time and 101% decoding time.

# Introduction

In ECM-9.0, two decoder-side inter prediction techniques, multi-pass DMVR and sample-based bi-directional optical flow (BDOF), are applied to further refine bi-prediction samples. At the last step of multi-pass DMVR, the motion field of the current CU is updated on a subblock-by-subblock basis using a delta motion (called motion refinement) derived through BDOF model. This updated motion field is used as the base to perform motion compensation for both luma and chroma components. Then, the sample-based BDOF refers to the same BDOF model to derive per-sample motion refinement for each luma prediction sample. The per-sample motion refinement is used as the input to compute a per-sample adjustment offset to update the corresponding luma bi-prediction sample. However, such sample-based approach is applied solely to luma component and to none of others. The chroma counterparts are constrained to reference only the subblock-based motion refinement (derived based solely on luma samples) for its own motion compensation and have no access to the sample-based one, potentially causing inconsistency in design. Accordingly, this contribution proposed to relax the constraint to make sample-based BDOF applicable to all color components.

# Proposal

The sample-based bi-directional optical flow is applied to the motion compensation process of chroma samples. Similar to luma samples, each chroma sample on both prediction blocks starts with computing its gradient vector using its immediately neighboring ones. The motion refinement of the sample-based BDOF for each luma sample has been derived, and thus can be shared with all chroma components. Then, since both the gradient vector and motion refinement of a chroma samples are ready for use, the same BDOF function of gradient vector and motion refinement can be used to compute an offset value, that is b(x,y), to adjust the corresponding chroma bi-prediction sample. The resulting chroma bi-prediction sample is represented as follows:

predBDOF(x,y) = ( I(0)(x,y) + I(1)(x,y) + b(x,y) + ooffset ) >> shift,

where I(0)(x,y) and I(1)(x,y) are the respective prediction samples located at sample position (x,y) in respective prediction blocks of L0 and L1, ooffset denotes rounding offset, and shift is shifting bits.

# Simulation results

The proposed method was implemented on top of the ECM-9.0 software [1] and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AD2024). The testing results are summarized below.

- Performance results of the proposed method relative to ECM-9.0 anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.13% | -0.03% | 100.3% | 100.2% |
| Class C | -0.01% | -0.15% | -0.13% | 99.6% | 99.3% |
| Class E | x | x | x | x | x |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.45% | -0.35% | 100.3% | 99.9% |
| Class F | -0.02% | -0.11% | -0.09% | 100.3% | 100.0% |
| Class TGM | 0.01% | -0.08% | -0.08% | 100.4% | 99.5% |

# References

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

# Patent rights declaration(s)

Qualcomm Incorporated does not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).