| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0006-v2 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Roman Chernyak Fabrice Le Léannec Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com chernyak@global.tencent.com Fabrice.Leleannec@InterDigital.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 31st and 32nd JVET meetings.

# Introduction

The mandates for the AHG are as follows:

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-10.0 software version and the reference configuration encodings according to the ECM common test conditions.

- Investigate encoder speedup and other encoder software optimization such as reduction of memory consumption.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following adopted aspects were integrated into ECM-10.0:

JVET-AE0091: High-Accuracy template matching (Test 3.3e) (MR 449)

JVET-AE0174: Add non-inter TM SPS flag for tool control (MR 452)

JVET-AE0139(Test 5.2c): Improved ALF fixed filters (MR 450)

JVET-AE0059(Test3.1b): Convolutional cross-component model for inter prediction (MR 453)

JVET-AE0169 (Test 2.2c/2.3b): Bi-predictive IBC (MR 448)

JVET-AE0100 (Test 2.1b): Block vector guided CCCM (MR 455)

JVET-AE0125 (Test 4.1) Shifting Quantization Center (MR 461)

JVET-AE0150: RPR filters and thresholds for scale factors 1.1x to 1.35x (MR 468)

JVET-AE0077(Test-2.9): Extended search areas for IntraTMP mode (MR 458)

JVET-AE0094(Test 2.6c): IBC with non-adjacent spatial candidates (MR 457)

JVET-AE0086(Test-4.2): Large NSPT (MR 466)

JVET-AE0091: Iterative BDOF Part (Test 3.5) (MR 460)

JVET-AE0084(Test-2.11a): Harmonization of IBC HMVP and IBC-LIC (MR 459)

JVET-AE0046(Test 3.2): Bi-predictive GPM (MR 465)

JVET-AE0159(Test 2.5c): Filtered Intra Block Copy (MR 462)

JVET-AE0169: IBC MBVD list construction (Test 2.4a) (MR 469)

JVET-AE0097: Remove offset update process in CCP merge (MR 463)

JVET-AE0043 (Test 2.7): Cross-component merge mode with temporal candidates (MR 467)

JVET-AE0102: LFNST/NSPT coefficients context (Test 4.3) (MR 478)

JVET-AE0151 (Test5.1b): CCSAO with history offsets and extended edge classifiers (MR 472)

JVET-AE0278: ALF, CCALF memory reduction (lossless change) (MR 480)

JVET-AE0078: IBC-LIC extension (Test 2.10a) (MR 482)

JVET-AE0136: Fix on TM-based reordering for affine MMVD (MR 456)

JVET-AE0162: fix MS-SSIM calculation for RPR/SR (MR 475)

JVET-AE0055: Add NumSignPredOverrideByQP to LD configurations (MR 454)

Bug fixes:

Fix: memory release for JVET_AB0155_SGPM (MR 452)

Fixes for DebugStream tool (MRs 488, 489, 490, 502)

Fix: buffer copy for AC0119 LM chroma fusion (MR 484)

Fix condition for checking buffer overflow (MR 494)

Fix: add initialization for BIF overhead parsing and copy (MR 504)

Fix: Add checks for scaled collocated picture, inter check for GPM (MR 508)

Fix: Valgrind error in spanGeoMMVDMotionInfo (MR 514)

Fix: use picture level reconstructed buffer for SBT sub-TUs in BIF (MR 512)

The following adopted aspects were integrated into VTM-11ecm anchor:

JVET-AE0057: MTT split modes early termination (MR 474)

JVET-AE0162: fix MS-SSIM calculation for RPR/SR (MR 476)

ECM-10.0 and VTM-11ecm10.0 were tagged on August 29, 2023.

## CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-AE2017 are summarized.

Next tables show ECM-10.0 performance over ECM-9.1 anchor.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.93% | -0.63% | -1.17% | 108.3% | 102.2% | 74.2% |
| Class A2 | -1.02% | -1.59% | -1.37% | 106.5% | 100.6% | 74.2% |
| Class B | -1.06% | -1.10% | -1.31% | 106.0% | 102.1% | 70.5% |
| Class C | -0.89% | -0.66% | -0.80% | 110.1% | 103.5% | 87.7% |
| Class E | -1.06% | -1.05% | -0.98% | 105.8% | 102.1% | 79.9% |
| Overall | -1.00% | -1.00% | -1.13% | 107.3% | 102.2% | 76.9% |
| Class D | -0.62% | -0.79% | -0.72% | 108.0% | 99.2% | 95.1% |
| Class F | -2.56% | -2.81% | -3.05% | 106.7% | 103.7% | 80.1% |
| Class TGM | -3.21% | -3.53% | -3.50% | 104.9% | 108.7% | 71.4% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-9.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -1.74% | -3.59% | -6.37% | 108.4% | 97.7% | 79.4% |
| Class A2 | -2.07% | -4.14% | -5.39% | 106.1% | 96.0% | 79.6% |
| Class B | -1.24% | -3.84% | -3.50% | 103.1% | 95.7% | 72.3% |
| Class C | -1.25% | -2.08% | -1.93% | 105.3% | 99.5% | 85.5% |
| Class E |  |  |  |  |  |  |
| Overall | -1.51% | -3.38% | -4.04% | 105.3% | 97.2% | 78.5% |
| Class D | -1.27% | -2.12% | -2.15% | 105.8% | 100.2% | 92.8% |
| Class F | -2.13% | -2.98% | -3.03% | 107.9% | 97.6% | 80.2% |
| Class TGM | -2.23% | -2.82% | -2.95% | 102.1% | 90.8% | 76.0% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-9.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.69% | -8.86% | -7.04% | 94.0% | 98.5% | 69.8% |
| Class C | -0.97% | -3.14% | -3.09% | 106.7% | 101.6% | 86.2% |
| Class E | -0.98% | -3.47% | -3.09% | 105.8% | 98.2% | 78.5% |
| Overall | -0.86% | -5.61% | -4.74% | 101.0% | 99.4% | 77.1% |
| Class D | -0.87% | -4.16% | -3.75% | 105.0% | 99.7% | 94.0% |
| Class F | -1.59% | -6.03% | -7.65% | 106.9% | 95.9% | 79.7% |
| Class TGM | -1.98% | -4.41% | -5.46% | 103.8% | 94.5% | 74.6% |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over ECM-9.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.42% | -8.57% | -7.36% | 93.1% | 95.5% | 69.6% |
| Class C | -0.84% | -3.48% | -3.68% | 105.2% | 94.6% | 86.3% |
| Class E | -0.59% | -2.68% | -2.66% | 107.0% | 98.0% | 78.5% |
| Overall | -0.60% | -5.40% | -4.96% | 100.4% | 95.8% | 77.1% |
| Class D | -1.01% | -2.98% | -4.80% | 105.1% | 95.4% | 94.2% |
| Class F | -2.00% | -6.37% | -6.84% | 109.5% | 96.1% | 79.5% |
| Class TGM | -2.71% | -5.41% | -6.28% | 105.5% | 98.0% | 73.5% |

The below tables show ECM-10.0 performance comparing to VTM-11.0ecm10.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -10.55% | -21.81% | -29.24% | 915.8% | 439.3% | #DIV/0! |
| Class A2 | -16.68% | -28.60% | -31.54% | 916.2% | 458.0% | #DIV/0! |
| Class B | -11.02% | -26.58% | -24.82% | 826.4% | 455.0% | #DIV/0! |
| Class C | -11.07% | -16.52% | -17.06% | 898.0% | 452.7% | #DIV/0! |
| Class E | -14.59% | -24.40% | -22.89% | 804.1% | 467.8% | #DIV/0! |
| Overall | -12.49% | -23.52% | -24.63% | 867.2% | 454.4% | #DIV/0! |
| Class D | -9.17% | -14.38% | -14.35% | 851.0% | 445.1% | #DIV/0! |
| Class F | -26.21% | -35.26% | -35.17% | 541.1% | 452.7% | #DIV/0! |
| Class TGM | -39.28% | -47.12% | -46.45% | 457.3% | 496.2% | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over VTM-11.0ecm10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -22.25% | -27.01% | -36.07% | 882.3% | 784.9% | #DIV/0! |
| Class A2 | -25.50% | -35.13% | -38.75% | 791.3% | 889.0% | #DIV/0! |
| Class B | -20.50% | -34.42% | -32.13% | 673.3% | 731.6% | #DIV/0! |
| Class C | -21.91% | -26.29% | -26.07% | 708.2% | 777.7% | #DIV/0! |
| Class E |  |  |  |  |  |  |
| Overall | -22.23% | -30.91% | -32.63% | 744.0% | 784.1% | #DIV/0! |
| Class D | -22.87% | -27.80% | -28.10% | 738.6% | 851.2% | #DIV/0! |
| Class F | -28.30% | -36.71% | -37.10% | 602.2% | 482.5% | #DIV/0! |
| Class TGM | -36.63% | -43.02% | -43.08% | 570.5% | 415.0% | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over VTM-11.0ecm10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -17.55% | -38.06% | -35.34% | 589.9% | 668.2% | #DIV/0! |
| Class C | -19.29% | -28.96% | -29.31% | 648.1% | 710.0% | #DIV/0! |
| Class E | -16.12% | -28.03% | -27.57% | 642.1% | 459.1% | #DIV/0! |
| Overall | -17.77% | -32.52% | -31.39% | 621.7% | 620.8% | #DIV/0! |
| Class D | -21.17% | -30.48% | -30.27% | 656.7% | 758.7% | #DIV/0! |
| Class F | -24.60% | -37.20% | -36.64% | 588.5% | 498.3% | #DIV/0! |
| Class TGM | -35.27% | -43.12% | -43.29% | 560.3% | 418.2% | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over VTM-11.0ecm10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -16.23% | -37.00% | -34.66% | 529.3% | 651.6% | #DIV/0! |
| Class C | -18.30% | -28.47% | -28.23% | 561.3% | 660.2% | #DIV/0! |
| Class E | -15.42% | -26.79% | -27.20% | 626.8% | 468.8% | #DIV/0! |
| Overall | -16.72% | -31.60% | -30.65% | 563.0% | 602.8% | #DIV/0! |
| Class D | -21.10% | -29.72% | -30.38% | 592.5% | 707.9% | #DIV/0! |
| Class F | -23.63% | -37.06% | -36.33% | 602.7% | 467.7% | #DIV/0! |
| Class TGM | -32.74% | -41.78% | -41.94% | 624.2% | 417.1% | #DIV/0! |

# ECM memory consumption

ECM memory consumption (VmPeak, GiB) is provided in ECM encoder log files and is summarized in the table below.

|  | AI | RA | LB |
| --- | --- | --- | --- |
| class A (A1& A2) | 9 | 20 |  |
| class B | 7 | 10 | 8 |
| class C | 5 | 6 | 5 |
| class D | 5 | 5 | 5 |
| class E | 6 |  | 6 |
| class F | 8 | 10 | 8 |
| class TGM | 8 | 12 | 11 |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already existed memory wherever possible, rather than systematically allocating new memory, and allocate only the required amount of memory if new memory is needed.

Memory consumption was further reduced after ECM-10.0 release, for the current master branch it is reduced by ~10% to ~60% depending on configuration and resolution. It is planned to tag it as ECM-10.1.

# Contributions

JVET-AF0090, “AHG6: Encoder memory profiling on ECM software“, 	Y. Yasugi, T. Ikai (Sharp), R. Chernyak, S. Liu (Tencent)

JVET-AF0156, “AHG6: ECM software optimizations”, F. Urban, T. Poirier, F. Le Léannec (InterDigital)

JVET-AF0177, “AHG6: Memory reduction for ECM encoder”, 	N. Hu, V. Seregin, M. Karczewicz (Qualcomm), Y. Yasugi, T. Ikai (Sharp)

JVET-AF0201, “AHG6: On ECM SW memory consumption”, R. Chernyak, M. Xu, S. Liu (Tencent), Y. Yasugi, T. Ikai (Sharp)

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

- Encourage people to continue working on ECM memory consumption reduction.

- Encourage people to continue working on speeding up ECM encoder to reduce the simulation time.