| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0007-v1 |
| --- | --- |

| Title: | AHG report: ECM tool assessment (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiang Li (chair) Lien-Fei Chen Zhipin Deng Jonathan Gan Edouard François Hong-Jheng Jhu Xinwei Li Hongtao Wang (vice chairs) | Tel: Email: | xlxiangli@google.com lienfeichen@global.tencent.com zhipin.deng@bytedance.com v-jonathan.gan@oppo.com Edouard.Francois@interdigital.com jhuhong-jheng@kwai.com sid.lxw@alibaba-inc.com hongtaow@qti.qualcomm.com |
| Source: | AhG7 on ECM tool assessment |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG7 on ECM tool assessment that has taken place between the 31st and 32nd JVET meetings.

# Mandates

The mandates given to the AHG are:

- Investigate methodology of tool assessment.

- Coordinate with AHG6 on resolving tool-off test related software issues (missing tool controls and software bugs).

- Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing.

- Prepare reporting of tool assessment results.

- Collect simulation results on non-CTC sequences, and report any issues identified with non-CTC sequences

- Develop methodology of more reliable runtime measurement

# Group off tests

## Test settings and crosschecking

The same four groups were used in this meeting cycle.

- Group 1: Inter template matching tools

- Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

- Group 3: Intra and IBC template matching (with search) related tools

- Group 4: Tools that require more processing on the neighboring reconstructed samples than VVC

Five group-off tests were performed and crosschecked on top of ECM-10 and later versions with bug fixes. The two anchors are ECM 10.0 and VTM-11ECM10.0. The cfg files used are also attached with this report.

The testers and crosscheckers are summarized in the table below. All the tests below have been crosschecked. Note that in all the crosschecks, the special setting for LD class B QP22 (with encoder configuration parameter MaxMTTHierarchyDepthByTidOverrideByQP) was not set so that the LDB class B results are slightly different from the results in tests. As the issue was spotted too late and LDB class B QP22 are the slowest points, reruns could not be finished before the JVET-AF meeting.

| Tests | Tester | Crosschecker |
| --- | --- | --- |
| Group 1 off | Charles Salmon-Legagneur (charles.salmon-legagneur@interdigital.com) | Jonathan Gan (v-jonathan.gan@oppo.com) |
| Group 2 off | Xinwei Li (sid.lxw@alibaba-inc.com) | Jonathan Gan (v-jonathan.gan@oppo.com) |
| Group 3 off | Zhipin Deng (zhipin.deng@bytedance.com) | Xiang Li (xlxiangli@google.com) |
| Group 4 off | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) | Xiang Li (xlxiangli@google.com) |
| Group 1-4 off | Hongtao Wang (hongtaow@qti.qualcomm.com) | Lien-Fei Chen (lienfei.chen@global.tencent.com), Xiang Li (xlxiangli@google.com) |

The software versions used in the tests are summarized below.

| Tests | Tester |  | Crosschecker |  |
| --- | --- | --- | --- | --- |
|  | Anchor | Test | Anchor | Test |
| Group 1 off | ECM-10 | ECM-10 | ECM-10 | ECM-10 |
| Group 2 off | ECM-10 (41ef8a15) | ECM-10 (41ef8a15) | ECM-10 | ECM-10 (41ef8a15) |
| Group 3 off | ECM-10 | ECM-10 | ECM-10 | ECM-10 (41ef8a15) |
| Group 4 off | ECM-10 | ECM-10 | ECM-10 | ECM-10 (41ef8a15) |
| Group 1-4 off | ECM-10 | ECM-10 (b0b4f1e9b) | ECM-10 | ECM-10 (41ef8a15) |

## Group 1 off

Group 1 includes inter template matching tools. The attached offgroup1.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  | Over VTM-11.0ecm10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 97.8% | 98.9% | 100.0% | -10.55% | -21.81% | -29.24% | 799.4% | 482.6% |
| Class A2 | 0.00% | 0.00% | 0.00% | 95.9% | 94.5% | 99.9% | -16.68% | -28.60% | -31.54% | 785.6% | 473.1% |
| Class B | 0.00% | 0.00% | 0.00% | 98.2% | 97.0% | 100.0% | -11.03% | -26.58% | -24.82% | 736.7% | 471.3% |
| Class C | -0.01% | -0.01% | -0.01% | 103.6% | 109.4% | 100.0% | -11.08% | -16.53% | -17.07% | 746.9% | 424.0% |
| Class E | -0.01% | -0.01% | -0.01% | 105.3% | 110.6% | 100.0% | -14.60% | -24.41% | -22.90% | 736.8% | 509.9% |
| Overall | -0.01% | -0.01% | -0.01% | 100.1% | 101.7% | 100.0% | -12.50% | -23.53% | -24.63% | 757.2% | 468.6% |
| Class D | -0.03% | -0.02% | -0.02% | 99.2% | 102.0% | 100.0% | -9.20% | -14.40% | -14.37% | 736.8% | 412.4% |
| Class F | -0.01% | -0.01% | -0.01% | 98.0% | 102.8% | 100.0% | -26.22% | -35.27% | -35.17% | 501.7% | 496.4% |
| Class TGM | 0.00% | 0.00% | 0.00% | 96.8% | 100.0% | 100.0% | -39.29% | -47.12% | -46.45% | 419.3% | 538.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  | Over VTM-11.0ecm10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 3.92% | 3.93% | 3.99% | 83.6% | 80.9% | 100.1% | -19.20% | -24.26% | -33.44% | 666.7% | 758.6% |
| Class A2 | 4.36% | 4.59% | 4.61% | 83.8% | 77.6% | 100.0% | -22.26% | -32.18% | -35.93% | 598.3% | 836.7% |
| Class B | 3.71% | 3.94% | 3.69% | 81.3% | 74.5% | 99.9% | -17.52% | -31.80% | -29.60% | 507.3% | 678.0% |
| Class C | 4.06% | 4.24% | 4.64% | 81.0% | 78.8% | 100.0% | -18.71% | -23.13% | -22.58% | 508.5% | 602.3% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 3.98% | 4.15% | 4.18% | 82.2% | 77.5% | 100.0% | -19.12% | -28.05% | -29.76% | 554.1% | 700.7% |
| Class D | 3.31% | 3.71% | 3.47% | 78.2% | 71.4% | 100.0% | -20.26% | -25.06% | -25.54% | 540.0% | 617.7% |
| Class F | 2.94% | 2.92% | 3.11% | 89.3% | 84.3% | 99.9% | -26.11% | -34.80% | -35.07% | 506.3% | 421.7% |
| Class TGM | 3.79% | 3.79% | 3.74% | 88.3% | 86.1% | 100.0% | -34.24% | -40.83% | -40.93% | 504.3% | 467.3% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  | Over VTM-11.0ecm10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 4.59% | 5.35% | 4.80% | 79.3% | 70.6% | 100.0% | -13.75% | -34.87% | -32.35% | 405.0% | 535.0% |
| Class C | 4.70% | 4.76% | 5.20% | 72.1% | 58.4% | 100.0% | -15.47% | -25.61% | -25.62% | 407.9% | 439.6% |
| Class E | 4.50% | 5.53% | 5.32% | 79.7% | 75.8% | 100.0% | -12.34% | -24.14% | -23.82% | 467.4% | 398.5% |
| Overall (Ref) | 4.60% | 5.20% | 5.06% | 76.9% | 67.5% | 100.0% | -13.97% | -29.10% | -27.98% | 420.8% | 465.5% |
| Class D | 4.13% | 4.64% | 4.25% | 72.8% | 60.7% | 100.0% | -17.89% | -27.18% | -27.32% | 468.6% | 513.5% |
| Class F | 4.24% | 3.47% | 4.11% | 82.1% | 72.8% | 100.0% | -21.33% | -34.99% | -34.06% | 470.0% | 412.4% |
| Class TGM | 5.78% | 5.60% | 5.50% | 91.3% | 85.5% | 100.6% | -31.51% | -39.83% | -40.05% | 469.8% | 374.6% |

## Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process. The attached offgroup2.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 (41ef8a15) |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 0.39% | 0.80% | 0.64% | 91.5% | 96.5% | 99.8% | -10.19% | -21.21% | -28.74% | 788.0% | 445.3% |
| Class A2 | 0.55% | 0.75% | 0.77% | 89.0% | 95.8% | 99.9% | -16.22% | -28.07% | -31.02% | 764.8% | 443.9% |
| Class B | 0.48% | 0.72% | 0.70% | 87.7% | 94.8% | 99.9% | -10.59% | -26.02% | -24.27% | 692.3% | 442.3% |
| Class C | 0.34% | 0.51% | 0.68% | 85.3% | 91.8% | 99.9% | -10.76% | -16.11% | -16.49% | 700.8% | 397.2% |
| Class E | 0.72% | 0.98% | 0.93% | 87.9% | 94.5% | 99.9% | -13.96% | -23.65% | -22.15% | 660.2% | 456.0% |
| Overall | 0.49% | 0.74% | 0.73% | 88.0% | 94.5% | 99.9% | -12.06% | -22.96% | -24.06% | 715.5% | 434.8% |
| Class D | 0.31% | 0.42% | 0.49% | 85.4% | 89.2% | 99.9% | -8.89% | -14.01% | -13.92% | 713.2% | 407.2% |
| Class F | 3.58% | 3.71% | 3.97% | 89.5% | 95.7% | 99.9% | -23.78% | -33.16% | -32.92% | 486.0% | 464.9% |
| Class TGM | 0.87% | 0.91% | 0.94% | 89.1% | 100.3% | 99.9% | -38.74% | -46.64% | -45.95% | 416.6% | 535.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-10 (41ef8a15) |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 2.37% | 0.74% | 1.65% | 91.2% | 98.4% | 99.9% | -20.39% | -26.36% | -34.98% | 776.8% | 861.1% |
| Class A2 | 1.68% | 0.94% | 1.39% | 90.1% | 99.7% | 100.2% | -24.23% | -34.48% | -37.87% | 669.6% | 996.2% |
| Class B | 1.65% | 1.11% | 1.29% | 89.6% | 96.9% | 99.7% | -19.18% | -33.64% | -31.27% | 607.6% | 832.9% |
| Class C | 1.25% | 0.97% | 0.53% | 87.7% | 95.2% | 99.9% | -20.95% | -25.63% | -25.72% | 627.8% | 872.8% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.70% | 0.96% | 1.18% | 89.5% | 97.3% | 99.9% | -20.90% | -30.21% | -31.85% | 656.4% | 879.9% |
| Class D | 0.68% | -0.03% | 0.11% | 86.3% | 95.2% | 99.9% | -22.32% | -27.76% | -28.00% | 666.9% | 961.1% |
| Class F | 3.73% | 3.60% | 3.45% | 88.9% | 95.8% | 100.0% | -25.71% | -34.60% | -35.11% | 566.3% | 524.4% |
| Class TGM | 1.92% | 1.93% | 2.02% | 90.0% | 100.3% | 100.0% | -35.38% | -41.89% | -41.89% | 549.7% | 508.2% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-10 (41ef8a15) |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.90% | 0.53% | 0.59% | 84.8% | 97.2% | 99.9% | -15.97% | -37.26% | -34.59% | 501.8% | 783.5% |
| Class C | 1.58% | 0.81% | 1.11% | 84.6% | 93.3% | 99.9% | -18.09% | -28.43% | -28.57% | 539.4% | 750.0% |
| Class E | 1.16% | 0.97% | 0.14% | 87.4% | 96.7% | 99.9% | -15.13% | -27.23% | -27.12% | 571.3% | 537.9% |
| Overall (Ref) | 1.61% | 0.73% | 0.65% | 85.4% | 95.8% | 99.9% | -16.46% | -31.81% | -30.72% | 531.0% | 702.9% |
| Class D | 0.77% | 0.41% | 0.60% | 84.0% | 93.9% | 99.9% | -20.52% | -30.29% | -30.09% | 580.6% | 933.4% |
| Class F | 3.38% | 1.85% | 1.52% | 88.6% | 96.3% | 99.8% | -22.07% | -35.99% | -35.72% | 544.4% | 557.4% |
| Class TGM | 2.01% | 2.04% | 2.34% | 90.5% | 102.1% | 100.2% | -33.94% | -42.10% | -42.06% | 513.2% | 496.1% |

## Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. The attached offgroup3.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 1.00% | 2.85% | 4.29% | 80.0% | 88.8% | 99.9% | -9.65% | -19.75% | -26.36% | 715.7% | 360.7% |
| Class A2 | 1.84% | 4.41% | 4.43% | 80.8% | 79.3% | 99.9% | -15.12% | -25.67% | -28.71% | 712.3% | 329.9% |
| Class B | 1.86% | 3.90% | 3.87% | 78.5% | 74.8% | 100.0% | -9.39% | -23.94% | -22.06% | 647.7% | 317.1% |
| Class C | 1.66% | 2.10% | 2.25% | 78.5% | 74.1% | 100.0% | -9.62% | -14.84% | -15.26% | 662.7% | 290.4% |
| Class E | 2.94% | 3.78% | 3.52% | 78.6% | 75.1% | 100.0% | -12.08% | -21.63% | -20.20% | 625.2% | 320.3% |
| Overall | 1.85% | 3.39% | 3.62% | 79.1% | 77.6% | 100.0% | -10.89% | -21.12% | -22.06% | 668.6% | 320.3% |
| Class D | 1.22% | 1.81% | 1.79% | 80.2% | 73.4% | 100.0% | -8.06% | -12.83% | -12.83% | 677.0% | 298.7% |
| Class F | 4.36% | 5.57% | 5.66% | 78.7% | 69.8% | 100.0% | -23.11% | -31.86% | -31.72% | 464.5% | 302.0% |
| Class TGM | 8.63% | 10.60% | 10.93% | 82.1% | 59.9% | 99.9% | -34.14% | -41.63% | -40.72% | 414.4% | 287.3% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-10 |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 0.96% | 2.00% | 5.80% | 95.1% | 99.0% | 100.2% | -21.50% | -25.59% | -32.98% | 813.1% | 702.3% |
| Class A2 | 1.10% | 3.01% | 3.27% | 95.7% | 100.1% | 99.6% | -24.68% | -33.27% | -36.81% | 739.3% | 828.6% |
| Class B | 1.00% | 3.61% | 3.47% | 94.5% | 98.4% | 99.9% | -19.70% | -32.07% | -29.80% | 645.0% | 684.2% |
| Class C | 0.80% | 1.16% | 1.19% | 94.4% | 99.0% | 100.0% | -21.28% | -25.40% | -25.17% | 663.9% | 707.8% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.96% | 2.51% | 3.29% | 94.8% | 99.1% | 99.9% | -21.48% | -29.24% | -30.60% | 699.6% | 721.1% |
| Class D | 0.68% | 1.56% | 1.85% | 95.2% | 98.4% | 100.0% | -22.34% | -26.62% | -26.73% | 711.7% | 728.1% |
| Class F | 2.80% | 3.63% | 3.81% | 92.3% | 99.5% | 100.2% | -26.43% | -34.60% | -34.82% | 589.0% | 412.3% |
| Class TGM | 5.50% | 7.34% | 7.70% | 93.9% | 98.0% | 100.0% | -33.15% | -38.85% | -38.74% | 585.9% | 381.1% |

## Group 4 off

Group 4 includes tools that require more processing on the neighboring reconstructed samples than VVC.

The attached offgroup4.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 1.60% | 5.59% | 7.45% | 86.9% | 94.4% | 99.9% | -9.12% | -17.75% | -24.20% | 752.4% | 416.8% |
| Class A2 | 2.48% | 6.17% | 7.18% | 83.7% | 96.3% | 99.9% | -14.70% | -24.44% | -26.94% | 702.7% | 429.4% |
| Class B | 1.01% | 6.50% | 5.30% | 87.5% | 96.9% | 100.0% | -10.14% | -22.27% | -21.13% | 691.2% | 442.6% |
| Class C | 0.89% | 2.91% | 2.83% | 84.5% | 95.8% | 100.0% | -10.29% | -14.24% | -14.84% | 689.1% | 416.4% |
| Class E | 1.16% | 6.61% | 3.92% | 87.6% | 98.0% | 100.0% | -13.60% | -19.81% | -20.08% | 666.8% | 463.0% |
| Overall | 1.35% | 5.51% | 5.19% | 86.1% | 96.3% | 100.0% | -11.34% | -19.68% | -21.04% | 698.3% | 433.4% |
| Class D | 0.87% | 2.53% | 2.34% | 85.6% | 96.9% | 100.0% | -8.38% | -12.27% | -12.38% | 696.3% | 433.3% |
| Class F | 1.21% | 4.10% | 4.28% | 92.3% | 97.1% | 100.0% | -25.33% | -32.83% | -32.59% | 477.0% | 458.9% |
| Class TGM | 2.27% | 3.84% | 3.76% | 91.4% | 96.2% | 100.0% | -37.97% | -45.22% | -44.56% | 415.0% | 500.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-10 |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 1.18% | 2.92% | 5.22% | 91.1% | 98.9% | 100.5% | -21.33% | -24.98% | -33.17% | 779.5% | 786.8% |
| Class A2 | 1.08% | 3.53% | 4.33% | 94.2% | 99.8% | 99.8% | -24.69% | -32.95% | -36.23% | 709.2% | 913.4% |
| Class B | 0.52% | 6.08% | 4.97% | 93.5% | 100.0% | 100.1% | -20.08% | -30.65% | -28.98% | 623.8% | 788.6% |
| Class C | 0.41% | 1.88% | 1.76% | 94.0% | 99.8% | 100.0% | -21.60% | -24.89% | -24.75% | 644.2% | 828.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.73% | 3.82% | 4.04% | 93.3% | 99.7% | 100.1% | -21.66% | -28.44% | -30.14% | 674.9% | 822.5% |
| Class D | 0.37% | 1.81% | 2.11% | 95.0% | 100.2% | 100.0% | -22.58% | -26.52% | -26.61% | 691.3% | 889.2% |
| Class F | 0.94% | 3.23% | 3.45% | 95.8% | 98.4% | 99.9% | -27.69% | -34.90% | -35.15% | 594.3% | 493.7% |
| Class TGM | 0.68% | 1.56% | 1.34% | 94.6% | 99.3% | 100.0% | -36.23% | -42.17% | -42.36% | 576.2% | 455.0% |

## Group 1-4 off

In this test, all the tools in the group 1-4 are switched off. The attached offgroup1-4.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 3.24% | 9.59% | 13.34% | 61.0% | 76.1% | 99.7% | -7.63% | -14.77% | -20.19% | 546.8% | 316.3% |
| Class A2 | 5.22% | 12.08% | 12.57% | 57.6% | 68.7% | 99.7% | -12.36% | -20.41% | -23.37% | 507.5% | 297.0% |
| Class B | 3.67% | 11.44% | 10.20% | 56.4% | 63.4% | 99.9% | -7.78% | -18.82% | -17.57% | 455.0% | 281.3% |
| Class C | 3.23% | 5.74% | 5.93% | 54.4% | 56.8% | 99.9% | -8.23% | -11.93% | -12.29% | 453.5% | 250.0% |
| Class E | 5.56% | 11.56% | 8.76% | 57.6% | 64.5% | 99.9% | -9.84% | -16.07% | -16.33% | 461.4% | 294.5% |
| Overall | 4.07% | 9.99% | 9.93% | 57.1% | 64.8% | 99.8% | -8.96% | -16.42% | -17.59% | 478.5% | 284.1% |
| Class D | 2.76% | 4.96% | 4.94% | 55.1% | 54.7% | 99.9% | -6.66% | -10.17% | -10.14% | 465.3% | 242.1% |
| Class F | 10.53% | 15.32% | 15.80% | 64.0% | 61.5% | 99.9% | -18.81% | -26.29% | -25.77% | 367.0% | 277.9% |
| Class TGM | 14.28% | 18.07% | 18.81% | 64.7% | 56.0% | 99.9% | -30.83% | -37.82% | -36.67% | 315.8% | 267.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-10 |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 | 9.01% | 10.53% | 18.25% | 65.8% | 80.5% | 99.8% | -15.22% | -19.48% | -25.41% | 564.9% | 579.9% |
| Class A2 | 8.71% | 12.98% | 14.22% | 65.7% | 77.3% | 99.6% | -18.99% | -26.94% | -30.19% | 499.4% | 641.7% |
| Class B | 7.23% | 14.79% | 13.93% | 63.7% | 73.7% | 99.8% | -14.70% | -24.88% | -22.86% | 434.1% | 532.6% |
| Class C | 6.73% | 8.54% | 8.96% | 60.7% | 65.9% | 99.9% | -16.60% | -19.91% | -19.32% | 433.5% | 499.6% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 7.75% | 11.91% | 13.53% | 63.7% | 73.5% | 99.8% | -16.17% | -22.89% | -23.89% | 470.4% | 552.8% |
| Class D | 5.26% | 7.62% | 8.28% | 61.7% | 64.5% | 99.9% | -18.72% | -22.17% | -21.99% | 462.6% | 530.5% |
| Class F | 11.45% | 14.38% | 14.99% | 69.5% | 76.9% | 99.8% | -20.36% | -28.13% | -28.20% | 443.8% | 344.8% |
| Class TGM | 13.48% | 16.18% | 16.41% | 72.2% | 77.4% | 99.9% | -28.09% | -33.82% | -33.78% | 453.6% | 318.9% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-10 |  |  |  |  |  | Over VTM-11ecm10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 7.07% | 9.46% | 9.24% | 56.8% | 64.8% | 100.0% | -11.69% | -32.34% | -29.54% | 337.7% | 424.7% |
| Class C | 6.74% | 7.49% | 8.02% | 54.4% | 53.9% | 99.9% | -13.83% | -23.70% | -23.63% | 346.6% | 369.5% |
| Class E | 6.48% | 14.60% | 11.45% | 66.3% | 75.6% | 99.9% | -10.66% | -17.94% | -19.36% | 435.9% | 334.0% |
| Overall (Ref) | 6.81% | 10.09% | 9.39% | 58.2% | 63.4% | 99.9% | -12.14% | -25.86% | -25.03% | 363.1% | 381.8% |
| Class D | 5.19% | 6.89% | 7.08% | 57.3% | 54.4% | 99.9% | -17.01% | -25.62% | -25.22% | 390.5% | 429.3% |
| Class F | 10.40% | 11.81% | 11.19% | 66.3% | 71.0% | 99.9% | -16.84% | -29.98% | -29.78% | 412.1% | 322.5% |
| Class TGM | 13.46% | 14.95% | 15.60% | 72.1% | 72.4% | 99.4% | -26.57% | -34.63% | -34.56% | 425.6% | 276.2% |

## Summary

The tool-off results on top of the recent ECM versions are summarized below. Note that SCC results are not included.

# Issues

## Resolved issues

- Software issues #51, #54 with tool off tests was resolved

## Open issues

- #53, decoding mismatch was observed when AMVR is off

- The variation of runtime over VTM11ECM9 is relatively large, for which the version of compiler matters

- Different software versions were used in AHG7 tests, which leads to slightly different results between testing and crosschecking.

- It was commented that it is not consistent to set LDB class B QP22 setting for ECM-10 anchor without also setting it for VTM11-ECM10 anchor. JVET-AF0165 is related to this issue.

# Input contributions

| JVET-AF0165 | m64994 | 2023-10-06 18:23:08 | 2023-10-06 19:45:18 | 2023-10-06 19:45:18 | Ahg7/Ahg12: On VTM-11ecm anchor low-delay test conditions | F. Le Léannec, S. Puri, E. François, K. Naser (InterDigital) |
| --- | --- | --- | --- | --- | --- | --- |

# Recommendations

- Continue and improve tool assessment

- Resolve identified software issues related to the tool assessment

- Review all the input contributions