| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0012-v2 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li R. Chernyak K. Naser Y. Yu | Tel: Email: | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@google.com chernyak@global.tencent.com karam.naser@interdigital.com yue.yu@oppo.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 31st meeting in Geneva, CH, 11–19 July 2023, and the 32nd meeting, Hannover, DE, 13–20 October 2023.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM10 algorithm description JVET-AE2025.

- Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Support AHG6 in generating anchors according to the test conditions in JVET-AE2017.

- Analyse the results of exploration experiments described in JVET-AE2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The Common Test Conditions were updated (JVET-AE2017). The peak memory usage has been added to the reporting template. The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AE2024). The combined improvements of the ECM-10.0 over VTM-11.0ecm10.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -10.55% | -21.81% | -29.24% | 915.8% | 439.3% |
| Class A2 | -16.68% | -28.60% | -31.54% | 916.2% | 458.0% |
| Class B | -11.02% | -26.58% | -24.82% | 826.4% | 455.0% |
| Class C | -11.07% | -16.52% | -17.06% | 898.0% | 452.7% |
| Class E | -14.59% | -24.40% | -22.89% | 804.1% | 467.8% |
| Overall | -12.49% | -23.52% | -24.63% | 867.2% | 454.4% |
| Class D | -9.17% | -14.38% | -14.35% | 851.0% | 445.1% |
| Class F | -26.21% | -35.26% | -35.17% | 541.1% | 452.7% |
| Class TGM | -39.28% | -47.12% | -46.45% | 457.3% | 496.2% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -22.25% | -27.01% | -36.07% | 882.3% | 784.9% |
| Class A2 | -25.50% | -35.13% | -38.75% | 791.3% | 889.0% |
| Class B | -20.50% | -34.42% | -32.13% | 673.3% | 731.6% |
| Class C | -21.91% | -26.29% | -26.07% | 708.2% | 777.7% |
| Class E |  |  |  |  |  |
| Overall | -22.23% | -30.91% | -32.63% | 744.0% | 784.1% |
| Class D | -22.87% | -27.80% | -28.10% | 738.6% | 851.2% |
| Class F | -28.30% | -36.71% | -37.10% | 602.2% | 482.5% |
| Class TGM | -36.63% | -43.02% | -43.08% | 570.5% | 415.0% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -17.55% | -38.06% | -35.34% | 589.9% | 668.2% |
| Class C | -19.29% | -28.96% | -29.31% | 648.1% | 710.0% |
| Class E | -16.12% | -28.03% | -27.57% | 642.1% | 459.1% |
| Overall | -17.77% | -32.52% | -31.39% | 621.7% | 620.8% |
| Class D | -21.17% | -30.48% | -30.27% | 656.7% | 758.7% |
| Class F | -24.60% | -37.20% | -36.64% | 588.5% | 498.3% |
| Class TGM | -35.27% | -43.12% | -43.29% | 560.3% | 418.2% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-9.0’s {-21.03%, -28.42%, -29.93%} to ECM-10.0’s {-22.23%, -30.91%, -32.63%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-9.0’s {-35.17%, -41.66%, -41.35%} to ECM-10.0’s {-36.63%, -43.02%, -43.08%}.

# Contributions

In addition to 24 EE2 contributions, 48 (comparing to 52 last meeting) EE2-related and AHG12-related contributions were received. There are also 7 contributions to AHG6 and AHG10 which address ECM encoder performance. The EE2-related and AHG12-related contributions can be subdivided as follows:

## Intra (7)

JVET-AF0059, "AHG12: Fix to interpolation filter for intra prediction", K. Andersson (Ericsson)

JVET-AF0083, "Non-EE2: Enhancements on CCP merge for chroma intra coding", H. Huang, Y. Yu, H. Yu, D. Wang (OPPO), Z. Deng, K. Zhang, L. Zhang (Bytedance)

JVET-AF0106, "EE2-related: Non-adjacent spatial candidates for DIMD merge", J. Huo, J. Fan, Z. Zhang, Y. Ma, F. Yang (Xidian Univ.), M. Li (OPPO)

JVET-AF0114, "AHG12: Local-Boosting on Cross-Component Merge Mode", H. Qin, K. Ding, Z. Xu (TCL)

JVET-AF0176, "Non-EE2: Decoder Derived Cross-Component Prediction", Y.-J. Chang, P.-H. Lin, V. Seregin, J.-L. Lin, M. Karczewicz (Qualcomm)

JVET-AF0199, "Non-EE2: Bilateral Filtering for Intra Prediction", W. Yin, K. Zhang, Y. Wang, L. Zhang (Bytedance)

JVET-AF0246, "AHG12: Harmonization of intra prediction mode derivation from neighboring blocks", D. Kim, K. Kim, J.-H. Son, J. -S. Kwak (WILUS)

## IntraTMP and IntraBC (15)

JVET-AF0053, "EE2-related: Test 2.5a and TIMD fusion improvement", P. Andrivon, M. Blestel (Ofinno), K. Naser, F. Le Léannec, P. Bordes, F. Galpin, A. Robert (InterDigital)

JVET-AF0072, "AHG12: FIBC flag inherits IntraTMP-FLM flag", M.-H. Jia, Y. Gao, Y. -X. Bai, S.-W. Xie, P. Wu, C. Huang (ZTE)

JVET-AF0084, "Non-EE2: Update on IBC-LIC Model Merge mode", L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AF0115, "Non-EE2: IBC with a further upward-extended reference area for screen content", Y. Kidani, H. Kato, K. Kawamura (KDDI)

JVET-AF0116, "Non-EE2: slope adjustment for IBC LIC", C. Ma, X. Xiu, W. Chen, H.-H.Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)

JVET-AF0117, "Non-EE2: extensions of IBC GPM", C. Ma, X. Xiu, W. Chen, H.-H.Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)

JVET-AF0129, "Non-EE2: Auto relocated block vector prediction", N. Zhang, K. Zhang, L. Zhang (Bytedance)

JVET-AF0132, "Non-EE2: Optimization of TIMD blending mode", C. Zhou, Z. Lv (vivo)

JVET-AF0137, "AHG12: IntraTMP with Merge Candidates", K. Naser, F. Le Léannec, A. Robert, F. Galpin (InterDigital)

JVET-AF0161, "AHG12: Harmonized IBC/ITMP search area with adaptive sampling", D. Ruiz Coll, B. Chen, (Ofinno), K. Naser, P. Bordes, F. Le Léannec, F. Galpin, (InterDigital)

JVET-AF0162, "AHG12: Fixes to template matching", J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba)

JVET-AF0166, "Non-EE2: Intra TMP fusion probing", J.-L. Lin, P.-H. Lin, Y.-J. Chang, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AF0184, "Non-EE2: Search range optimization for Intra TMP", G. Verba, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AF0226, "AHG12: SGPM with IntraTMP and IBC", K. Naser, F. Le Léannec, P. Bordes, Y. Chen (InterDigital)

JVET-AF0248, "Non-EE2: TIMD fusion optimization",	P. Andrivon, M. Blestel (Ofinno), C. Zhou, Zhuoyi Lv (vivo)

## Inter (12)

JVET-AF0113, "Non-EE2: Modifications of affine merge candidates", K. Kim, D. Kim, J.-H. Son, J.-S. Kwak (WILUS)

JVET-AF0118, "Non-EE2: Regression-based GPM blending", P. Bordes, F. Galpin, K. Naser, F. Urban, K. Reuze, F. Le Leannec, E. Francois (InterDigital)

JVET-AF0121, "AHG12: Adjusting out-of-boundary prediction samples", P. Astola, J. Lainema (Nokia)

JVET-AF0134, "AHG12: AMVP with SbTMVP mode", R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)

JVET-AF0135, "AHG12: Additional SbTMVP candidates", R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)

JVET-AF0160, "Non-EE2: On DMVR Extensions", M. Salehifar, Y. He, K. Zhang, L. Zhang (Bytedance)

JVET-AF0168, "Non-EE2: Geometric partitioning mode with affine prediction", K. Zhang, Z. Deng, L. Zhang (Bytedance)

JVET-AF0170, "Non-EE2: CIIP with subblock-based motion compensation", L. Zhao, K. Zhang, L. Zhang (Bytedance)

JVET-AF0171, "Non-EE2: Sample distance-based weighting for CIIP", J. Zhao, S. Kim (LGE)

JVET-AF0191, "Non-EE2: Enhancements on local illumination compensation", X. Xiu, C. Ma, N. Yan, H.-J. Jhu, C.-. Kuo, W. Chen, X. Wang (Kwai)

JVET-AF0194, "Non-EE2: On LIC flag in merge mode", Y. Zhang, V. Seregin, H. Wang, Z. Zhang, C.-C. Chen, H. Huang, M. Karczewicz (Qualcomm)

JVET-AF0200, "Non-EE2: Extension of local illumination compensation", Y. Wang, K. Zhang, Y. He, H. Liu, L. Zhang (Bytedance)

## In Loop Filters (4)

JVET-AF0169, "Non-EE2: Adaptive clipping with signaled lower and upper bounds", K. Cui, Z. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AF0178, "Non-EE2: Applying fixed filters to output of the Gaussian filter", N. Hu, M. Karczewicz, V. Seregin (Qualcomm)

JVET-AF0179, "Non-EE2: Fixed filter for Chroma ALF", N. Hu, M. Karczewicz, H. Wang, V. Seregin (Qualcomm)

JVET-AF0198, "Non-EE2: Coefficient Precision Adjustment for ALF", W. Yin, K. Zhang, L. Zhang (Bytedance)

## Transform (2)

JVET-AF0082, "Non-EE2: On LFNST/NSPT for inter coding", F. Wang, J. Gan, Y. Yue, H. Yu, D. Wang (OPPO)

JVET-AF0104, "Non-EE2: Direct Decoder-side Intra Mode Derivation for IntraTMP blocks", J. Huo, N. Qiu, Y. Ma, F. Yang (Xidian Univ.), Ming Li (OPPO)

## Coefficients and Entropy Coding (4)

JVET-AF0151, "EE2 related: CABAC parameters retraining", F. Galpin, F. Lo Bianco, C. Salmon-Legagneur, K. Naser (InterDigital)

JVET-AF0164, "AHG12: CABAC inter/intra model switch", F. Lo Bianco, F. Galpin, C. Salmon-Legagneur (InterDigital)

JVET-AF0185, "Non-EE2: Transform Coefficient Coding", P. Nikitin, M. Karczewicz, M. Coban, V. Seregin (Qualcomm)

JVET-AF0202, "EE2 related: CABAC context initialization retraining and slice type based window offsets", V. Seregin, M. Karczewicz (Qualcomm)

## Other (4)

JVET-AF0058, "AHG12: GOP-based RPR encoder control for ECM", K. Andersson, J. Ström, R. Yu, W. Ahmad, P. Wennersten (Ericsson)

JVET-AF0101, "AHG12: ECM software cleanup for decoder side intra predictions acceleration", Z. Fan, Y. Yasugi, T. Ikai (Sharp)

JVET-AF0165, "Ahg12: On VTM-11ecm anchor low-delay test conditions", F. Le Léannec, S. Puri, E. François, K. Naser (InterDigital)

JVET-AF0187, "Compression of Gaming Contents", Z. Lin, K. Cai, Y. Zhao, E. Alshina (Huawei)

# Recommendations

The AHG recommends to:

- To review all the related contributions.