| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AE0043-v1 |
| --- | --- |

| Title: | AHG11: Status of the joint EE1-0 (LOP.2) unified filter training |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Informational |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy, Y.Li (Qualcomm) J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya(Ittiam) T. Shao, P. Yin (Dolby) | Email: | dmytror@qti.qualcomm.com yli30@qti.qualcomm.com jay.shingala@ittiam.com ajayshyam@ittiam.com ajat.suneja@ittiam.com siddarth.badya@ittiam.com Tong.Shao@dolby.com pyin@dolby.com |
| Source: | EE1-Dolby, Ittiam, Qualcomm |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the Low-performance Operation Point (LOP) was proposed in JVET-AE0281 to meet LOP complexity constraints (LOP.2), following HOP based training process. This contribution reports on the progress of LOP.2 joint training and presents full set of LOP2.0 results. Information presented in this document was partly presented during the AhG11/14 Telco on 08/09/2023.

# Introduction

In JVET-AE0281 a Unified Filter Architecture for the Low-performance Operation Point (LOP) was proposed and agreed for a joint training targeting LOP.2.

The filter architecture for LOP.2 was a joint design from EE1-4.1 (JVET-AE0067), EE1-4.4 (JVET-AE0165), and JVET-AE0238. The training process are aligned to the HOP filter training. The unified LOP model uses the simplified head block, simplified backbone and the split luma-chroma architecture which filter luma and chroma in separate branches using the simplified backbone and simplified tail blocks, as shown in Figure 1.

The following joint training procedure was defined for LOP.2 in JVET-AE2023.

For the stages I and II the following sub-tests are planned:

Subtest EE1-0.1: Test LOP filter architecture defined in JVET-AE0281.

Subtest EE1-0.2: Optimize the headblock design of JVET-AE0281 toward improving hardware friendliness (e.g. multiple of 16).

Subtest EE1-0.3: Based on of JVET-AE0281, different combinations of model parameters in luma and chroma branches toward better complexity-performance trade-off:

  - Number of hidden layers: NY, NC

  - Number of channels: CY, CUV, CY21, CUV21

  - SIMD friendly sizes: number of channels preferably multiple of 16

After Stage II training, a single model architecture to be selected for unified LOP test using HOP based filter usage and tested in NNVC-6.0 using SADL:

Sub-test EE1-0.4: Filter usage:

For the final trained LOP test mode, study filter usage aspects such as combine interfaces for usage of HOP and LOP filters, adaptive block sizes, on/off control at low block level.

Figure 1. LOP2.0 filter architecture of JVET-AE0281

The document below describes the progress of the LOP2.0 filter training.

# Reuse of HOP training process

## Overview

Figure 2 Training steps for HOP model creation

For training of the LOP.2, a modified HOP training, shown in Figure 2, was utilized. The following stages were implemented:

- Stage I:

  - Reuse HOP stage 1 dataset, md5: 1862a271133e3fabdf78e19d0d2c608f.

  - Train LOP.2 model(s) on the Stage 1 dataset: resulting model is lop2.x_st1.

- Stage II:

  - Encode BVI and TVD set data with nnvc6.0 and model of the stage 1, lop2.x_st1.

  - Produce the Stage 2 dataset from.

  - Train LOP.2 model (s) on the Stage 2 dataset: resulting model is lop2.x_st2.

- Stage III:

  - Encode BVI and TVD set data with nnvc6.0 and model of the stage 2, lop2.x_st2.

  - Produce the Stage 3 dataset from.

  - Train LOP.2 model on the Stage 3 dataset: resulting model is lop2.0_st3 (final model).

# Stage 1 training

Joined training of the Stage 1 was conducted by Ittiam/Dolby and Qualcomm independently with multiple LOP2.x filter architectures, following sub-tests of EE1-0.1, 0.2 and 0.3. The results of training, such as validation loss, CTC inference results and model have been exchanged and cross-checked.

The following architectures have been tested in the Stage 1:

- EE1-0.1: Default architecture

Default architecture have been implemented by Ittiam/Dolby and Qualcomm independently. Identified incomplete SADL support for luma/chroma split functionality was addressed differently, resulting in variance of the split implementation.

Ittiam/Dolby implemented split explicitly using slicing of the input tensor into luma and chroma tensors, as shown in Figure 3.a. ONNX to SADL conversion script had issues in ‘slice’ layer parsing due to incorrect assumptions. In addition to this, ‘slice’ layer implementation inside SADL library had newly introduced bugs since SADL_v5.1.  Both these issues have been fixed and merged into SADL dev_v7 branch (Commit id #6cce76e37cda77504d9d7fd978e0c0840b2db5ef).

Qualcomm implemented split through two separate convolutions, output of which produces luma and chroma associated out channels, as shown in Figure 3.b. No changes to the ONNX/SADL conversion or quantization is required.

Both designs result in near identical normative implementation, and are within the scope of EE1 testing.

- b.

Figure 3. Tested LOP2.0 filter split implementation

Additionally, the following training configurations have been tested:

Schedulers:

Scheduler 1:  "lr_scheduler" : { "milestones" : [110,  115, 120] } , "mse_epoch" : 120

Scheduler 2:  "lr_scheduler" : { "milestones" : [120,  123, 126] }, "mse_epoch" : 126

Weights on training losses (Y,U,V):

Weights 1: “component_loss_weightings” : [6,2]

Weights 2: “component_loss_weightings” : [12,2]

Summary of the inference CTC results of independently trained Stage 1 model for sub-test EE1-0.1, are shown in Table 1 and Table 2. Complete simulation test results are provided with this document.

Table 1: Inference results of default LOP2.0 architecture (source: Qualcomm (Scheduler 2 and Weights 1) and Ittiam/Dolby (Schedule 1 and Weights 1)

|  | lop2.0_st1_float (QC) |  |  | lop2.0_st1_float (Ittiam/Dolby) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -3.82% | -9.35% | -10.73% | -3.68% | -9.00% | -10.12% |
| Class A2 | -4.13% | -11.69% | -9.39% | -4.08% | -11.48% | -9.56% |
| Class B | -3.92% | -11.21% | -9.85% | -3.96% | -10.70% | -10.32% |
| Class C | -4.34% | -12.04% | -12.07% | -4.29% | -11.49% | -12.42% |
| Class E | -5.83% | -11.00% | -11.60% | -5.86% | -11.00% | -12.43% |
| Overall | -4.35% | -11.13% | -10.71% | -4.32% | -10.77% | -10.98% |
| Class D | -4.60% | -10.96% | -11.69% | -4.64% | -10.64% | -12.77% |

Table 2: Inference results of default LOP2.0 architecture and Scheduler 2 (source: Ittiam/Dolby, Qualcomm, Schedule 2).

|  | Lop2.0_st1_float, weights1 |  |  | lop2.0_st1_float, weights2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -3.82% | -9.35% | -10.73% | -3.99% | -9.83% | -11.36% |
| Class A2 | -4.13% | -11.69% | -9.39% | -4.11% | -11.88% | -9.67% |
| Class B | -3.92% | -11.21% | -9.85% | -4.01% | -11.44% | -10.61% |
| Class C | -4.34% | -12.04% | -12.07% | -4.38% | -12.32% | -13.08% |
| Class E | -5.83% | -11.00% | -11.60% | -5.98% | -11.67% | -13.49% |
| Overall | -4.35% | -11.13% | -10.71% | -4.43% | -11.48% | -11.61% |
| Class D | -4.60% | -10.96% | -11.69% | -4.70% | -11.12% | -13.39% |

Based on the observed inference results, st1_lop2.0_float model, resulted from the default architecture of LOP2.0, as defined in JVET-AE0281, trained with Scheduler 2 and Weights 2 (12:1:1) was selected for producing dataset for Stage 2. Since Stage 2 dataset is temporal, and performance of quantization on the model is not fully assessed at this stage, SADL model with float32 implemented was selected for Stage2 dataset production.

Resulting training configuration is provided below:

"stage1" : {
        "training" : {
            "mse_epoch" : 126,
            "max_epochs" : 130,
            "component_loss_weightings" : [12, 2],
            "lr_scheduler" : {"milestones" : [120, 123, 126]
            }
        }
    }
Training progress and simulation results for the LOP2.0 Stage 1 have been communicated on the AhG11/14 Telco, held on August 09th, 2023. Presentation was prepared by EE1-0 proponents and provided with this contribution.

Resulting Stage 1 model, training scripts, configurations files and model have been shared on the EE1 SW repository:
https://vcgit.hhi.fraunhofer.de/jvet-ae-ee11/VVCSoftware_VTM/-/tree/EE1-0

# Stage 2 training:

Stage 1 model selected to produce Stage 2 data set was shared through EE1-0 SW repository. The originally, was produced and tested with SADL0003, latter converted to SADL004. Both versions of the model file have been tested and provided over the SW repository. NNVC6.0rc1 SW was selected to be used for Stage 2 dataset production.

Preliminary agreed scheduler for Stage 2 training is shown below:

"stage2" : {
        "training" : {
            "mse_epoch" : 58,
            "max_epochs" : 60,
            "component_loss_weightings" : [12, 2],
            "lr_scheduler" : {"milestones" : [55, 57, 58]
            }
        }
    }

# Communication:

Training progress and simulation results for the LOP2.0 Stage 1 have been communicated on the AhG11/14 Telco, held on August 09th, 2023. Presentation was prepared by EE1-0 proponents and provided with this contribution.

# Software availability and complexity:

SW branch for joint LOP2.0 training

https://vcgit.hhi.fraunhofer.de/jvet-ae-ee11/VVCSoftware_VTM/-/tree/EE1-0

Training scripts and configuration files can be found in folder:

training/training_scripts/NN_Filtering_LOP

Computational resources and storage requirements are identical to those of HOP training. DIV2K dataset of HOP is used for LOP training in stage 1.