| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0050-v1 |
| --- | --- |

| Title: | AHG9: On NNPF groups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input document presents a study of the neural-network post-filter (NNPF) group design in JVET-AE2032 "Technologies under consideration for future extensions of VSEI (draft 1)" (VSEI TuC). The following changes are proposed relative to JVET-AE2032:

- In the JVET-AE meeting, nnpfa_no_prev_clvs_flag and nnpfa_no_foll_clvs_flag were adopted to the neural-network post-filter activation (NNPFA) SEI message to control multi-input NNPFs in bitstream splicing. This contribution proposes to introduce similar syntax elements also to the NNPF group activation (NNPFGA) SEI message.

- It is asserted that bitstreams where nnpfa_target_id is equal to an nnpfgc_id value would be regarded as non-conforming bitstreams according to VSEI v3. Consequently, and also because the NNPFGA SEI message is asserted to provide more flexibility than the NNPFA SEI message, it is proposed to remove the possibility to reference NNPF group identifiers from the NNPFA SEI message.

Except for the changes proposed above, other aspects of the NNPF group design in the VSEI TuC are considered mature in this contribution and are proposed to be included in the VSEI output document of the JVET-AF meeting, which may either be a working draft or a new version of the VSEI TuC document.

# Aligning the NNPFGA SEI message with the NNPFA SEI message

## Problem statement

In the JVET-AE meeting, nnpfa_no_prev_clvs_flag and nnpfa_no_foll_clvs_flag were adopted to the NNPFA SEI message to control multi-input NNPFs in bitstream splicing. This contribution proposes to introduce similar syntax elements also to the NNPFGA SEI message.

## Proposed text relative to JVET-AE2032

| nn_post_filter_group_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfga_target_id | ue(v) |
| nnpfga_cancel_flag | u(1) |
| if( !nnpfga_cancel_flag ) { |  |
| nnpfga_persistence_flag | u(1) |
| nnpfga_no_prev_clvs_flag | u(1) |
| if( nnpfga_persistence_flag ) |  |
| nnpfga_no_foll_clvs_flag | u(1) |
| nnpfga_num_filters_minus2 | ue(v) |
| for( i = 0; i <= nnpfga_num_filters_minus2 + 1; i++ ) { |  |
| nnpfga_target_base_flag[ i ] | u(1) |
| nnpfga_input_all_pics_flag[ i ] | u(1) |
| if( !nnpfga_input_all_pics_flag[ i ] ) { |  |
| nnpfga_num_input_pics_minus1[ i ] | ue(v) |
| for( j = 0; j <= nnpfga_num_input_pics_minus1[ i ]; j++ ) |  |
| nnpfga_input_pic_skip_count[ i ][ j ] | ue(v) |
| } |  |
| nnpfga_num_output_entries[ i ] | ue(v) |
| for( j = 0; j < nnpfga_num_output_entries[ i ]; j++ ) |  |
| nnpfga_output_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |

nnpfga_no_prev_clvs_flag equal to 1 specifies that the input pictures for the NNPF group do not originate from a previous CLVS. nnpfga_no_prev_clvs_flag equal to 0 specifies that the input pictures for the NNPF group may or may not originate from a previous CLVS.

NOTE 2 – The value of nnpfga_no_prev_clvs_flag can be changed from 0 to 1, when the current CLVS is spliced from another bitstream next to the previous CLVS and this NNPFGA SEI message would cause one or more input pictures to be selected from one or more previous CLVSs and therefore is likely to impact the output of the target NNPF group negatively.

nnpfga_no_foll_clvs_flag equal to 1 specifies that when this NNPFGA SEI message persists for the last PU of a CLVS in output order, the NNPFGA SEI message is treated like it persisted for the last PU, in output order, of the current layer within the bitstream. When this NNPFGA SEI message does not persist for the last PU, in output order, of a CLVS in output order or nnpfga_no_foll_clvs_flag is equal to 0, the value of nnpfga_no_foll_clvs_flag causes no specific impact.

NOTE 3 – The value of nnpfga_no_foll_clvs_flag can be  changed from 0 to 1 for an NNPF group including a picture-rate-upsampling NNPF, when the following CLVS is spliced from a different bitstream next to the current CLVS. Consequently, the NNPF process of the picture-rate-upsampling NNPF interpolates pictures up to the end of the current CLVS using input pictures originating from the current CLVS only.

# NNPFA SEI message or NNPFGA SEI message for NNPF group activation?

The VSEI TuC includes two options for activating an NNPF group:

- Using the NNPFA SEI message but allowing to indicate an nnpfa_target_id value that is equal to an nnpfgc_id value rather than an nnpfc_id value.

- Using an NNPF group activation (NNPFGA) SEI message.

As an outcome of the JVET-AE meeting, it was left for further study if the extensions for the NNPFA SEI message that activate an NNPF group or the NNPF group activation SEI message is the best design choice, or if both would be justified.

It is asserted in this document that option 1 violates the following constraint in VSEI v3:

An NNPFA SEI message with a particular value of nnpfa_target_id shall not be present in a current PU unless one or both of the following conditions are true:

–	Within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id present in a PU preceding the current PU in decoding order.

–	There is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id in the current PU.

Consequently, bitstreams where nnpfa_target_id is equal to an nnpfgc_id value would be regarded as non-conforming bitstreams according to VSEI v3. It is therefore proposed that option 1 is removed from the VSEI output document of the JVET-AF meeting.

Additionally, option 2 gives an encoder more adaptation flexibility:

- Option 2 enables an encoder to select between a base NNPF and an updated NNPF separately for each member of an NNPF group, whereas option 1 only allows selection between the base and updated NNPFs collectively for all members of an NNPF group.

- Option 2 enables an encoder to control which ones of the available candidate input pictures resulting from the previous NNPF in a cascade are used as input pictures for the next NNPF in the cascade, whereas in option 1 the input pictures would need to be inferred.

Option 1 in the VSEI TuC lacks the text how to set the interface variables (CroppedWidth, CroppedHeight, CroppedYPic, CroppedCbPic, CroppedCrPic, BithDepthY, BitDepthC, ChromaFromatIdc, and StrengthControlVal) for members of an NNPF group.

# On NNPFGC SEI message

## Background: NNPF group design in the VSEI TuC

The VSEI TuC includes the neural-network post-filter group characteristics (NNPFGC) SEI message, which has the following aspects:

- NNPF group ID, nnpfgc_id, which uses the same value space as nnpfc_id

- The group type, nnpfgc_grouping_type

- The IDs of the members of the group, which can be nnpfc_id values or nnpfgc_id values

- The complexity of the NNPF group

Additionally, JVET-AF0051 proposes to add signalling of the quality improvement or the gain resulting by applying the NNPF group.

The following group types are specified:

- Cascade:
nnpfgc_grouping_type equal to 0 indicates that this SEI message specifies a group of cascaded neural-network post-filters.

- Alternatives: 
nnpfgc_grouping_type equal to 1 indicates that the NNPFs or NNPF groups identified by the nnpfgc_member_id[ i ] are alternatives to each other out of which the post-processor should select only one to be applied.

- Alternating: 
nnpfgc_grouping_type equal to 2 indicates that this SEI message specifies a group of NNPFs that are intended to be used jointly and are activated in an alternating manner so that at most one of these NNPFs is activated for any picture.

- Parallel: 
nnpfgc_grouping_type equal to 3 indicates that the NNPFs or NNPF groups identified by the nnpfgc_member_id[ i ] are intended to be used in parallel.

- Optional: 
nnpfgc_grouping_type equal to 4 indicates that the NNPFs or NNPF groups identified by the nnpfgc_member_id[ i ] are optional, i.e., may or may not be applied by the post-processor.

## Motivation why the NNPFGC SEI message is needed in addition to the SEI processing order SEI message

This input document concludes the SEI processing order SEI message lacks the following functionalities or capabilities of the NNPF group design in the VSEI TuC:

- Indicating different types of grouping

The SEI processing order SEI message indicates a processing order, i.e., essentially a cascade of operations implied by SEI messages. It can also indicate if there is no preferred order between two or more SEI messages. Furthermore, po_sei_importance_flag[ i ] equal to 1 indicates that if the decoding system cannot interpret or does not support the i-th SEI message, it should ignore the entire SEI processing order SEI message.

It is concluded in this document that grouping types 1 to 3 (alternatives, alternating, and parallel) are not supported by the SEI processing order SEI message.

- Indicating optional processing steps

When a second NNPF follows a first NNPF in the order indicated by an SEI processing order SEI message, it cannot be concluded if the second NNPF is to be applied only when the first NNPF is applied first or if the second NNPF could be applied even if the first NNPF is not applied or activated. In one example, it cannot be concluded from the SEI processing order and NNPFC SEI messages if the bitstream allows the use of picture rate upsampling NNPF without resolution upsampling. This may be important at session setup or negotiation when an upsampled picture rate is desirable but the computational and/or memory capability of the receiver would not allow both resolution upsampling and picture rate upsampling.

- Concluding the complexity of an NNPF group

This may be important at session setup or negotiation for determining whether a bitstream with NNPFs can be processed or which one of the multiple alternative bitstreams with NNPFs is suitable for the computational and/or memory capability of the receiver.

It is remarked that the complexity depends on the type of grouping. For an NNPF cascade, the complexity is essentially a sum of the complexity of each NNPF in the cascade, whereas in the case of a group of alternating NNPFs, the complexity is a maximum of any individual filter complexity in the NNPF group. Alternating NNPFs are intended to be used jointly and are activated in an alternating manner so that at most one of these NNPFs is activated for any picture.

A more detailed discussion has been included in JVET-AD0059.

As a conclusion, it is proposed to keep the NNPFGC SEI message in the VSEI output document of the JVET-AF meeting.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).