| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0052-v1 |
| --- | --- |

| Title: | AHG9: Proposed updates on the encoder optimization information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes the following updates to the encoder optimization information SEI message:

- Editorial updates to the phrasing of the cancellation and persistence.

- A syntax element eoi_object_based_idc to indicate the type of object-based optimization, including blurring, quantization adjustment, and overwriting sample values of areas outside the detected objects.

- Replacement of the 16-bit optimization_type by individual flags followed by reserved zero bits.

- Replacement of the optimization_ prefix in the syntax element names with eoi_ to obtain shorter syntax element names.

- Sensibility constraints that when the persistence is for the current picture only, the temporal optimizations (eoi_temporal_subsampling_flag and eoi_temporal_quality_flag) are required to be off.

- For temporal subsampling, addition of eoi_num_int_pics, which is indicative of the count of pictures that the encoding systems excluded between each pair of coded pictures in output order.

- For temporal quality optimization, a clarification of the semantics and addition of a related NOTE.

# Introduction

In the JVET-AE meeting, the encoder optimization information SEI message proposed in JVET-AE0064 was adopted into the technologies under consideration for future extensions of VSEI (draft 1), available as JVET-AE2032. The suggestions in the JVET-AE meeting for further improvements included the following:

- It was clarified that after cancellation another SEI message could follow. In this context, the interrelation between persistence and cancellation should be more clarified.

- It was commented that the high-level information “with object based optimization” is not useful. It should be more specifically say what was done, e.g. QP adjustment, blurring, omission of certain parts, etc. It is likely that more information needs to be conveyed for this optimization type.

- In terms of syntax, it was commented that defining optimization type as u(16) and then re-defining as flags is inappropriate and could be either right away defined as flags, or by numbers.

The following changes are proposed, where the proposed items 1 to 3 respectively correspond to the suggestions a to c above.

- Editorial updates are proposed to the phrasing of the cancellation and persistence with the aim of making the semantics unambiguous.

- A syntax element eoi_object_based_idc is proposed to indicate the type of object-based optimization, including blurring, quantization adjustment, and overwriting sample values of areas outside the detected objects.

- The 16-bit optimization_type is proposed to be editorially replaced by individual flags followed by reserved zero bits.

- Replacing the optimization_ prefix in the syntax element names with eoi_ is proposed to obtain shorter syntax element names.

- It is proposed to add sensibility constraints that when the persistence is for the current picture only, the temporal optimizations (eoi_temporal_subsampling_flag and eoi_temporal_quality_flag) are required to be off.

- For temporal subsampling, it is proposed to add eoi_num_int_pics, which is indicative of the count of pictures that the encoding systems excluded between each pair of coded pictures in output order. This enables a decoding system to perform picture rate upsampling back to the original picture rate.

- A clarification on the semantics of the temporal quality optimization and a related informative NOTE are proposed.

# Proposed specification text changes

Note: The proposed editorial replacement of the optimization_ prefix in the syntax element names with eoi_ is not illustrated as a change by strikethroughs optimization_ and highlights eoi_ below.

  - Encoder optimization information SEI message

    - Encoder optimization information SEI message syntax

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ){ |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_flag | u(1) |
| eoi_for_machine_analysis_flag | u(1) |
| optimization_type | u(16) |
| eoi_object_based_flag | u(1) |
| eoi_temporal_subsampling_flag | u(1) |
| eoi_spatial_subsampling_flag | u(1) |
| eoi_temporal_quality_flag | u(1) |
| eoi_spatial_quality_flag | u(1) |
| eoi_reserved_zero_11bits | u(11) |
| if( eoi_object_based_flag ) |  |
| eoi_object_based_idc | ue(v) |
| if( eoi_temporal_subsampling_flag ) |  |
| eoi_num_int_pics | ue(v) |
| } |  |
| } |  |

    - Encoder optimization information SEI message semantics

The encoder optimization information SEI message is used to indicate if the video has been optimized for human viewing or machine analysis and which types of optimization have been applied in pre-processing or encoding.

eoi_cancel_flag equal to 1 specifies that the persistence of the previous applied optimization encoder optimization information SEI message included in any previous PU in output order is cancelled. eoi_cancel_flag equal to 0 indicates that eoi_persistence_flag and eoi_method information on optimization that has been applied in pre-processing or encoding follows.

eoi_persistence_flag specifies the persistence of the optimization information provided in this SEI message the current layer. eoi_persistence_flag equal to 0 specifies that the optimization information applies for the current picture only. eoi_persistence_flag equal to 1 specifies that the optimization information applies for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with an encoder_optimization_info encoder optimization information SEI message is output that follows the current picture in output order.

eoi_for_human_viewing_flag equal to 1 specifies that purposes for the applied optimization include human viewing. eoi_for_human_viewing_flag equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

eoi_for_machine_analysis_flag equal to 1 specifies that purposes for the applied optimization include machine analsysis. eoi_for_machine_analysis_flag equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

eoi_object_based_flag equal to 1 indicates that object-based optimization has been applied in a manner that the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes. eoi_object_based_flag equal to 0 indicates that no object-based optimization has been applied.

eoi_temporal_subsampling_flag equal to 1 indicates that temporal subsampling optimization has been applied. eoi_temporal_subsampling_flag equal to 0 indicates that no temporal subsampling optimization has been applied. When eoi_persistence_flag is equal to 0, eoi_temporal_subsampling_flag shall be equal to 0.

eoi_spatial_subsampling_flag equal to 1 indicates that spatial subsampling optimization has been applied. eoi_spatial_subsampling_flag equal to 0 indicates that no spatial subsampling optimization has been applied.

eoi_temporal_quality_flag equal to 1 indicates that temporal quality optimization has been applied in a manner that quality fluctuates temporally. eoi_temporal_quality_flag equal to 0 indicates that no temporal quality optimization has been applied. When eoi_persistence_flag is equal to 0, eoi_temporal_quality_flag shall be equal to 0.

NOTE – For example, when certain highest temporal sublayers have been encoded with such coarse quantization that human viewers perceive the quality fluctuation annoying, but machine task performance is not compromised, eoi_for_human_viewing_flag, eoi_for_machine_analaysis_flag, and eoi_temporal_quality_flag can be set equal to 0, 1, and 1, respectively.

eoi_spatial_quality_flag equal to 1 indicates that spatial quality optimization has been applied in a manner that the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information (e.g., to reduce the amount of noise and remove speckles at the picture-level). eoi_spatial_quality_flag equal to 0 indicates that no spatial quality optimization has been applied.

eoi_reserved_zero_11bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Non-zero values of eoi_reserved_zero_11bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_reserved_zero_11bits is greater than 0, decoders conforming to this version of this Specification shall ignore all syntax elements following the syntax elements of the optimization information SEI message specified in this version of this Specification.

optimization_type indicates the types of optimization method as specified in Table 2.

Table x – Definition of optimization_type

| Value | Interpretation |
| --- | --- |
| optimization_type = = 0 | May be used as determined by the application |
| optimization_type > 0 && ( optimization_type & 0x01 ) = = 0 | No object-based optimization |
| ( optimization_type & 0x01 ) != 0 | With object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| optimization_type > 0 && ( optimization_type & 0x02 ) = = 0 | No temporal subsampling optimization |
| ( optimization_type & 0x02 ) != 0 | With temporal subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x04 ) = = 0 | No spatial subsampling optimization |
| ( optimization_type & 0x04 ) != 0 | With spatial subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x08 ) = = 0 | No temporal quality optimization |
| ( optimization_type & 0x08 ) != 0 | With temporal quality optimization |
| optimization_type > 0 && ( optimization_type & 0x10 ) = = 0 | No spatial quality optimization |
| ( optimization_type & 0x10 ) != 0 | With spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |

eoi_object_based_idc, when present, indicates the type of object-based optimization as specified in Table x2, where ( eoi_object_based_idc & bitMask ) not equal to 0 indicates that the object-based optimization type associated with the bitMask value in Table x2 has been applied. When eoi_object_based_idc is greater than 0 and ( eoi_object_based_idc & bitMask ) is equal to 0, the object-based optimization type associated with the bitMask value has not been applied. When eoi_object_based_idc is equal to 0, an application-defined type of object-based optimization has been applied. The value of eoi_object_based_idc shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 65 535, inclusive, for eoi_object_based_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_object_based_idc is in the range of 8 to 65 535, inclusive, decoders conforming to this version of this Specification shall ignore eoi_object_based_idc.

Table x2 – Definition of eoi_object_based_idc

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Areas outside the detected objects have been blurred prior to encoding. |
| 0x02 | Areas outside the detected objects have been encoded with coarser transform-domain quantization than the quantization used for the detected objects. |
| 0x04 | Areas outside the detected objects have been overwritten. For example, an encoding system can overwrite areas outside the detected objects with a constant sample value. |

eoi_num_int_pics greater than 0 indicates the count of pictures that the encoding systems excluded between each pair of coded pictures in output order within the persistence of this SEI message. eoi_num_int_pics equal to 0 indicates that the count of pictures that the encoding systems excluded between each pair of coded pictures in output order within the persistence of this SEI message is unknown or varying. The value of eoi_num_int_pics shall be in the range of 0 to 63, inclusive.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).