| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0059-v1 |
| --- | --- |

| Title: | AHG12: Fix to interpolation filter for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

It was noted that ECM-10.0 can give stripe like visual artifacts and it was discovered that the reason for that was in interpolation in intra prediction due to non-unity gain of one of the 64-phase interpolation filters. It is therefore suggested to correct the filter with non-unity gain using a mirror version of corresponding mirror phase filter.

Objective performance (BDR) versus ECM-10.0 is reported as:

AI: Y: -0,01%, U 0,01%, V -0,05%

It is asserted that fixing the gain of the interpolation filter with non-unity gain also can improve subjective quality.

# Introduction

In was observed that one of the 64-phase intra interpolation filters, phase 41/64, do not have unity gain and do not match with the corresponding mirror phase filter. The filter coefficients sum up to 246 when they should sum up to 256:

{   3, -21, 106, 180, -25,   3 },

Using an interpolation filter with non-unity gain can result in visual artifacts which also was the reason for this finding.

# Proposal

Use the mirror version of corresponding filter with mirror phase 23/64:

{   3, -25, 189, 106, -20,   3 },

as filter for phase 41/64:

{   3, -20, 106, 189, -25, 3}.

# Results

Encoding and decoding times are not accurate.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0,01% | -0,01% | 0,00% | 94,8% | 97,8% | 100,0% |
| Class A2 | -0,01% | 0,04% | -0,07% | 92,6% | 94,6% | 100,1% |
| Class B | -0,02% | -0,01% | -0,07% | 96,5% | 98,5% | 100,0% |
| Class C | -0,02% | 0,02% | 0,08% | 92,2% | 95,1% | 100,0% |
| Class E | -0,02% | 0,02% | -0,21% | 95,6% | 95,8% | 100,0% |
| Overall | -0,01% | 0,01% | -0,05% | 94,5% | 96,5% | 100,0% |
| Class D | 0,02% | 0,06% | 0,08% | 100,4% | 101,0% | 100,0% |
| Class F | -0,05% | 0,06% | 0,06% | 92,5% | 91,6% | 100,0% |

## Visual example

Figure 1: Cactus all-intra QP 37 at POC 41. Left: ECM-10.0, Right: Proposed.

# Conclusion

It has been found that one of the interpolation filters used for intra prediction have non-unity gain which can cause visual artifacts.

It is suggested to fix this issue to instead use a mirror version of corresponding mirror phase interpolation filter that has unity gain in the next version of ECM.

# References

[1] M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, Geneva Switzerland, July 2023

[2] ECM-10.0, available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-10.0

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).